3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220929

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2665 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - At step 22A of the test case, Prach configuration index and RaResponse window in the system information are updated to 157 and sl40 respectively in case of TDD - As per 38.331 : ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. As per the above RRC specification snippet , RaResponse window has to be limited to sl20. - At Step 22A prach-ConfigurationIndex is set as 157 , for which Prach is expected to be received in the subframe = 4 ( every SFN ). If Prach is received in Subframe =4 and slot = 1 on even SFN , no issue is observed as there is no interference from SSB in the next SFN ( sf = 0 slot = 0 ) But If RACH is received in subframe = 4 & Slot = 1 of Odd SFN , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) . To overcome this issue , it is proposed to make the prach-ConfigurationIndex as 91 for which Prach is expected to be received in the subframe = 8/9 ( every odd SFN ) and there is also no SSB interference during Rar transmission |  |  |  |  |  |  |  |  |
| Summary of change: |  | - At Step 22A, ra-ResponseWindow is set as sl20 - At Step 22A, prachConfigurationIndex is modified as 91 in case of TDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for NR testcase 8.1.1.4.1	3

2.1	Correction to the function fl_TC_8_1_1_4_1_TestBody	3

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required for NR testcase 8.1.1.4.1

  - Correction to the function fl_TC_8_1_1_4_1_TestBody

| Function name | Function fl_TC_8_1_1_4_1_TestBody |
| --- | --- |
| Reason for change | - At step 22A of the test case, Prach configuration index and RaResponse window in the system information are updated to 157 and sl40 respectively in case of TDD - As per 38.331 : ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. As per the above RRC specification reference, RaResponse window has to be limited to sl20. - At Step 22A prach-ConfigurationIndex is set as 157 , for which Prach is expected to be received in the subframe = 4 ( every SFN ). If Prach is received in Subframe =4 and slot = 1 ( on even SFN ), no issue is observed as there is no interference from SSB in the next SFN ( sf = 0, slot = 0 ) But If RACH is received in subframe = 4 & Slot = 1 of Odd SFN , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) . To overcome this issue , it is proposed to make the prach-ConfigurationIndex as 91 for which Prach is expected to be received in the subframe = 8/9 ( every odd SFN ) and there is also no SSB interference during Rar transmission |
| Summary of change | - At Step 22A, ra-ResponseWindow is set as sl20 - At Step 22A, prachConfigurationIndex is modified as 91 in case of TDD |
| TTCN module | RRC_Resume_NR5GC.ttcn |
| MCC160 Comment | Accepted as per agreed prose CR R5-224201 – see proposed implementation below |

Before Change:

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { //@sic R5-197053, R5-198912, R5s191051 sic@ var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo; var integer v_DRBId; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (value) CellGroupConfig v_CellGroupConfig; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; // @sic R5-204436 sic@ var PDSCH_Config v_PDSCH_Config; // @sic R5-204436 sic@ var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; // @sic R5-204436 sic@ var integer v_PrachConfigurationIndex; // @sic R5-213465 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters; // @sic R5-213465 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; // not used (needed when other information of the RRC message needs to be access) // @sic R5-213465 sic@ var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; // to store SYSTEM_IND if it takes over the RRC message // @sic R5-213465 sic@ var template (present) TimingInfo_Type v_TimingInfo; // @sic R5-213465 sic@ timer t_WaitTime := 60.0; //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value? f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); // @sic R5-200256, R5-201212, R5-201646, R5-204436 sic@ //@siclog "Step 4" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 5" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5s201461 sic@ v_PDUSessionInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); // @sic R5-200256 sic@ if (isvalue(v_PDUSessionInfo)) { // @sic R5s200517 sic@ f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 5"); // @sic R5-200256 sic@ } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session"); } //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 7" siclog@ //The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. f_NR_CellInfo_SetSIB1_UseFullResumeID(nr_Cell1, true_); //@siclog "Step 8" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //@siclog "Step 9" siclog@ //Wait for 2.1* modification period second for the UE to receive new system information. v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1); v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod); //@siclog "Step 10" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 11" siclog@ //Check: Does the UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? SRB.receive(car_NR_SRB0_RrcPdu_IND1(nr_Cell1, cr_38508_RRCResumeRequest1(tsc_NR_I_RNTI_Value1))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 12" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); // @sic R5-201646, R5-204436 sic@ //@siclog "Step 13" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 14" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRC_InactiveDef(nr_Cell1, -, false); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2 ,tsc_NR_38508_NextHopChainingCount); //@siclog "Step 15" siclog@ //The SS adjusts cell levels according to row T1 of table 8.1.1.4.1.3.2-1 for FR1 and table 8.1.1.4.1.3.2-2 for FR2. f_NR_SetCellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 16" siclog@ //Check: Does the UE transmit an RRCResumeRequest message with resumeCause set to rna-Update on NR Cell 2 within 60s? t_WaitTime.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))) { t_WaitTime.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 8.1.1.4.1 Step 16"); } [] t_WaitTime.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.1.1.4.1 Step 16"); } } //@siclog "Step 17" siclog@ //The SS transmits an RRCResume message //@sic R5-201646, R5-204436 sic@ v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit; v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config); f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig); //@siclog "Step 18" siclog@ //The UE transmit an RRCResumeComplete message to complete the resume procedure f_NR_RRCResumeComplete_Def(nr_Cell2); //@siclog "Step 19" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? f_NR5GC_CheckDataPath(nr_Cell2, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 19"); // @sic R5-200256, R5s200517 sic@ //@siclog "Step 20" siclog@ //@sic R5-213465 sic@ //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. f_NR_RRC_InactiveDef(nr_Cell2); //@siclog "Steps 21 - 22" siclog@ //@sic R5-214592 sic@ //Void //@siclog "Step 22A" siclog@ //@sic R5-213465, R5-214592 sic@ //The SS changes the prach-ConfigurationIndex in the system information. if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){ v_PrachConfigurationIndex := 157; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 // nSFN mod 1 = 0=> any SFN Subframe 4 v_TimingInfo := cr_TimingInfo(?, (4)); } else { // FR2 v_PrachConfigurationIndex := 156; // @sic R5-217275, R5s211579 sic@ // slots (19,39) in 60kHz - As per 38.211 Table 6.3.3.2-4 // SCS RA = 120kHz, "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nRAslot = 1 v_TimingInfo := ( cr_TimingInfo(?, 4, -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 in 120kHz cr_TimingInfo(?, 9, -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 in 120kHz ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex); f_NR_CellInfo_SetRaResponseWindow(nr_Cell2, sl40); //@sic R5-221437, R5s220125 sic@ //Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); <<SKIPPED CODE>> }

After Change:

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { //@sic R5-197053, R5-198912, R5s191051 sic@ var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo; var integer v_DRBId; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (value) CellGroupConfig v_CellGroupConfig; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; // @sic R5-204436 sic@ var PDSCH_Config v_PDSCH_Config; // @sic R5-204436 sic@ var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; // @sic R5-204436 sic@ var integer v_PrachConfigurationIndex; // @sic R5-213465 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters; // @sic R5-213465 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; // not used (needed when other information of the RRC message needs to be access) // @sic R5-213465 sic@ var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; // to store SYSTEM_IND if it takes over the RRC message // @sic R5-213465 sic@ var template (present) TimingInfo_Type v_TimingInfo; // @sic R5-213465 sic@ timer t_WaitTime := 60.0; //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value? f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); // @sic R5-200256, R5-201212, R5-201646, R5-204436 sic@ //@siclog "Step 4" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 5" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5s201461 sic@ v_PDUSessionInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); // @sic R5-200256 sic@ if (isvalue(v_PDUSessionInfo)) { // @sic R5s200517 sic@ f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 5"); // @sic R5-200256 sic@ } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session"); } //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 7" siclog@ //The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. f_NR_CellInfo_SetSIB1_UseFullResumeID(nr_Cell1, true_); //@siclog "Step 8" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //@siclog "Step 9" siclog@ //Wait for 2.1* modification period second for the UE to receive new system information. v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1); v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod); //@siclog "Step 10" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 11" siclog@ //Check: Does the UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? SRB.receive(car_NR_SRB0_RrcPdu_IND1(nr_Cell1, cr_38508_RRCResumeRequest1(tsc_NR_I_RNTI_Value1))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 12" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); // @sic R5-201646, R5-204436 sic@ //@siclog "Step 13" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 14" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRC_InactiveDef(nr_Cell1, -, false); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2 ,tsc_NR_38508_NextHopChainingCount); //@siclog "Step 15" siclog@ //The SS adjusts cell levels according to row T1 of table 8.1.1.4.1.3.2-1 for FR1 and table 8.1.1.4.1.3.2-2 for FR2. f_NR_SetCellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 16" siclog@ //Check: Does the UE transmit an RRCResumeRequest message with resumeCause set to rna-Update on NR Cell 2 within 60s? t_WaitTime.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))) { t_WaitTime.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 8.1.1.4.1 Step 16"); } [] t_WaitTime.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.1.1.4.1 Step 16"); } } //@siclog "Step 17" siclog@ //The SS transmits an RRCResume message //@sic R5-201646, R5-204436 sic@ v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit; v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config); f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig); //@siclog "Step 18" siclog@ //The UE transmit an RRCResumeComplete message to complete the resume procedure f_NR_RRCResumeComplete_Def(nr_Cell2); //@siclog "Step 19" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? f_NR5GC_CheckDataPath(nr_Cell2, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 19"); // @sic R5-200256, R5s200517 sic@ //@siclog "Step 20" siclog@ //@sic R5-213465 sic@ //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. f_NR_RRC_InactiveDef(nr_Cell2); //@siclog "Steps 21 - 22" siclog@ //@sic R5-214592 sic@ //Void //@siclog "Step 22A" siclog@ //@sic R5-213465, R5-214592 sic@ //The SS changes the prach-ConfigurationIndex in the system information. if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){ v_PrachConfigurationIndex := 91; //WA#WI=984105 //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 // nSFN mod 1 = 0=> any SFN Subframe 4 v_TimingInfo := ( cr_TimingInfo(?, (8)), //WA#WI=984105 cr_TimingInfo(?, (9))); //WA#WI=984105 } else { // FR2 v_PrachConfigurationIndex := 156; // @sic R5-217275, R5s211579 sic@ // slots (19,39) in 60kHz - As per 38.211 Table 6.3.3.2-4 // SCS RA = 120kHz, "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nRAslot = 1 v_TimingInfo := ( cr_TimingInfo(?, 4, -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 in 120kHz cr_TimingInfo(?, 9, -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 in 120kHz ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex); //f_NR_CellInfo_SetRaResponseWindow(nr_Cell2, sl40); //@sic R5-221437, R5s220125 sic@ //WA#WI=984105 //Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); << SKIPPED CODE >> }

MCC160 Implementation:

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC

{

//@siclog "Step 22A" siclog@

//@sic R5-213465, R5-214592 sic@

//The SS changes the prach-ConfigurationIndex in the system information.

if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){

// @sic R5-224201, R5s220929 sic@

v_PrachConfigurationIndex := 91;

if (f_NR_CellInfo_GetIsFDD(nr_Cell2)) {

//FR1 FDD - As per 38.211 Table 6.3.3.2-2

//nSFN mod 4 = 0=> any SFN   Subframe 4/9

v_TimingInfo := ( cr_TimingInfo(?, (4)),

cr_TimingInfo(?, (9))

);

}

else{

//FR1 TDD - As per 38.211 Table 6.3.3.2-3

//nSFN mod 2 = 1=> any odd SFN   Subframe 8/9

v_TimingInfo := ( cr_TimingInfo(?, (8)),

cr_TimingInfo(?, (9))

);

}

} else { // FR2

v_PrachConfigurationIndex := 156; // @sic R5-217275, R5s211579 sic@

// slots (19,39) in 60kHz - As per 38.211 Table 6.3.3.2-4

// SCS RA = 120kHz, "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nRAslot = 1

v_TimingInfo := ( cr_TimingInfo(?, 4, -, cs_SlotTimingInfo_Numerology3(7)),  //Slot 39 in 120kHz

cr_TimingInfo(?, 9, -, cs_SlotTimingInfo_Numerology3(7))   //Slot 79 in 120kHz

);

}

//Set Prach configuration index in SIB1 and in Cell Array

f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex);

f_NR_CellInfo_SetRaResponseWindow(nr_Cell2, sl20); //@sic R5-221437, R5s220125, R5-224201, R5s220929 sic@

//Reconfigure SS with new PRACH parameters

v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2);

f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs))));

//Reconfigure SS to report PRACH preambles

f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable));

}