3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220930

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2666 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As per 38.331 : ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. For Step 0AA in the test case, ra-ResponseWindow is set it as sl40 when prach-ConfigurationIndex is set as 94 for TDD condition. As per the RRC specification requirement , ra-ResponseWindow has to be set as sl20. - At Step 0AA prach-ConfigurationIndex is set as 94 , for which Prach is expected to be received in the subframe = 4/9 ( every odd SFN ). If Prach is received in Subframe =9 and slot = 1 , no issue is observed . But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) . To overcome this issue , it is proposed to make the prach-ConfigurationIndex as 91 for which Prach is expected to be received in the subframe = 8/9 ( every odd SFN ) and there is also no SSB interference during Rar transmission |  |  |  |  |  |  |  |  |
| Summary of change: |  | - At Step 0AA, ra-ResponseWindow is set as sl20 - At Step 0AA, prachConfigurationIndex is modified as 91 in case of TDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for NR testcase 7.1.1.1.2	3

2.1	Correction to the template cs_RACH_ConfigGeneric71112_FR1_TDD	3

2.2	Correction to the function f_TC_7_1_1_1_2_NR_TestBody1	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcase 7.1.1.1.2

## Correction to the template cs_RACH_ConfigGeneric71112_FR1_TDD

| Template name | cs_RACH_ConfigGeneric71112_FR1_TDD |
| --- | --- |
| Reason for change | - As per 38.331 : ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. For Step 0AA in the test case, ra-ResponseWindow is set it as sl40 when prach-ConfigurationIndex is set as 94 for TDD condition. As per the RRC specification requirement , ra-ResponseWindow has to be set as sl20. - At Step 0AA prach-ConfigurationIndex is set as 94 , for which Prach is expected to be received in the subframe = 4/9 ( every odd SFN ). If Prach is received in Subframe =9 and slot = 1 , no issue is observed ;But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) ; to overcome this issue , it is proposed to make the prach-ConfigurationIndex as 91 for which Prach is expected to be received in the subframe = 8/9 ( every odd SFN ) and there is also no SSB interference during Rar transmission |
| Summary of change | - At Step 0AA, ra-ResponseWindow is set as sl20 - At Step 0AA, prachConfigurationIndex is modified as 91 in case of TDD |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted |

Before Change:

template (value) RACH_ConfigGeneric cs_RACH_ConfigGeneric71112_FR1_TDD := cs_NR_RACH_ConfigGenericDef(94,-,-,-,-104,n10,dB2,sl40); /* @status APPROVED (ENDC, NR5GC) */

After Change:

template (value) RACH_ConfigGeneric cs_RACH_ConfigGeneric71112_FR1_TDD := cs_NR_RACH_ConfigGenericDef(91,-,-,-,-104,n10,dB2,sl20); /* @status APPROVED (ENDC, NR5GC) */ //WA#WI=984107

## Correction to the function f_TC_7_1_1_1_2_NR_TestBody1

| Template name | cs_NR_RachProcedureConfig_71112_2 |
| --- | --- |
| Reason for change | - As per 38.331 : ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. For Step 0AA in the test case, ra-ResponseWindow is set it as sl40 when prach-ConfigurationIndex is set as 94 for TDD condition. As per the RRC specification requirement , ra-ResponseWindow has to be set as sl20. - At Step 0AA prach-ConfigurationIndex is set as 94 , for which Prach is expected to be received in the subframe = 4/9 ( every odd SFN ). If Prach is received in Subframe =9 and slot = 1 , no issue is observed ;But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) ; to overcome this issue , it is proposed to make the prach-ConfigurationIndex as 91 for which Prach is expected to be received in the subframe = 8/9 ( every odd SFN ) and there is also no SSB interference during Rar transmission |
| Summary of change | - At Step 0AA, ra-ResponseWindow is set as sl20 - At Step 0AA, prachConfigurationIndex is modified as 91 in case of TDD |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_1_2_NR_TestBody1( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_SYSTEM_IND v_Received_NR_SysInd; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL2; var SubFrameTiming_Type v_TimingUL3; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112(v_NR_PhysicalParameters); var integer v_FrameNum := 0; var integer v_FrameNum2 := 0; var template (present) integer v_SubFrameNum := 0; var integer v_SubFrameNum_Rcvd := 0; var integer v_Slot_Rcvd := 0; var template (present) integer v_PRACH_PreamblesGroupA := (0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,22,23,24,25,26,27); var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var float v_TimerT2 := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 0.96); var float v_TimerT3 := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 1.920); //@sic R5s200823 sic@ var SubFrameTiming_Type v_TimingDRB; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var boolean v_DRB_Flag := false; var boolean v_PRACH_Flag := false; timer t_Watchdog := 5.0; if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_SubFrameNum := (4,9); } else //FR1 FDD { v_SubFrameNum := (2,6,9); } } else { // FR2 // with SCS =120, 10 subframes with each 8 slots. Hence slots 0 ..39 map to sub frames 0..4 v_SubFrameNum := (1,2,3,4,6,7,8,9); } // Enable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // configure RACH procedure suitable for steps 4 to 17. f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // @siclog "Step 1" siclog@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,200); // Timing of DL // Configure SS transmission of Timing advance f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE(-,'011111'B))); //@sic R5s210694 sic@ // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop); // @siclog "Steps 2" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 700); //Time alignemnet is 750ms // @siclog "Steps 3" siclog@ // PDCP SDU =7B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header + 2B Padding Sub PDU =17B=136 bits (an allowed TB size as per 38.523-3 Annex B) t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //cs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_NR(v_TimingDL), {crs_NR_PDCP_SDU_7B} )); // @siclog "Steps 3" siclog@ // preconfigured at SS to not allocate grants // @siclog "Steps 4" siclog@ alt { [ ] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { repeat; }; [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo(?, v_SubFrameNum), v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd { v_FrameNum := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.SFN.Number; v_SubFrameNum_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Subframe.Number; if (not v_IsFR1) {v_Slot_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Slot.SlotOffset.Numerology3;}; //@sic R5s200043 sic@ // FR2 4,9,14,19,24,29,34,39 //@sic R5-198323 R5s200111 R5-201100 R5-201210 sic@ if ((v_IsFR1 and ischosen(v_NR_DuplexMode.TDD) and (v_FrameNum mod 2) == 1 and (v_SubFrameNum_Rcvd == 4 or v_SubFrameNum_Rcvd == 9)) or //TDD (v_IsFR1 and (not ischosen(v_NR_DuplexMode.TDD)) and (v_FrameNum mod 8) == 1 and (v_SubFrameNum_Rcvd == 2 or v_SubFrameNum_Rcvd == 6 or v_SubFrameNum_Rcvd == 9)) or //FDD //Fr2 (( not v_IsFR1 ) and (v_FrameNum mod 4) == 1 and ((v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 0) // slot 8 @sic R5s211590 sic@ or (v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 1) //slot 9 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 2) //slot 18 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 3) //slot 19 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 4) //slot 28 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 5) // slot 29 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 6) // slot 38 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 7) //slot 39 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 0) // slot 48 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 1) //slot 49 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 2) // slot 58 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 3) //slot 59 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 4) // slot 68 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 5) //slot 69 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 6) // slot 78 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 7))//slot 79 )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4 "); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4, wrong frame number used "); } }; }; // @siclog "Steps 5" siclog@ //@sic R5-198323 R5s200111 R5-201100 R5-201210 sic@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo((v_FrameNum, v_FrameNum+2, v_FrameNum+4, v_FrameNum+8), v_SubFrameNum), v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd; v_FrameNum2 := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.SFN.Number; v_SubFrameNum_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Subframe.Number; if (not v_IsFR1) {v_Slot_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Slot.SlotOffset.Numerology3;};//@sic R5s200043 sic@ if ((v_IsFR1 and ischosen(v_NR_DuplexMode.TDD) and (v_FrameNum2 == v_FrameNum or v_FrameNum2 == v_FrameNum+2 )and (v_SubFrameNum_Rcvd == 4 or v_SubFrameNum_Rcvd == 9)) or //TDD (v_IsFR1 and (not ischosen(v_NR_DuplexMode.TDD)) and (v_FrameNum2 == v_FrameNum+8) and (v_SubFrameNum_Rcvd == 2 or v_SubFrameNum_Rcvd == 6 or v_SubFrameNum_Rcvd == 9)) or //FDD //FR2 (( not v_IsFR1 ) and (v_FrameNum2 == v_FrameNum or v_FrameNum2 == v_FrameNum+4)and ((v_SubFrameNum_Rcvd == 0 and v_Slot_Rcvd == 8) // slot 8 or (v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 1) //slot 9 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 2) //slot 18 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 3) //slot 19 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 4) //slot 28 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 5) // slot 29 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 6) // slot 38 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 7) //slot 39 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 0) // slot 48 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 1) //slot 49 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 2) // slot 58 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 3) //slot 59 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 4) // slot 68 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 5) //slot 69 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 6) // slot 78 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 7)) //slot 79 )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 "); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5, wrong frame number used "); } // @siclog "Steps 6" siclog@

After Change:

function f_TC_7_1_1_1_2_NR_TestBody1( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_SYSTEM_IND v_Received_NR_SysInd; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL2; var SubFrameTiming_Type v_TimingUL3; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112(v_NR_PhysicalParameters); var integer v_FrameNum := 0; var integer v_FrameNum2 := 0; var template (present) integer v_SubFrameNum := 0; var integer v_SubFrameNum_Rcvd := 0; var integer v_Slot_Rcvd := 0; var template (present) integer v_PRACH_PreamblesGroupA := (0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,22,23,24,25,26,27); var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var float v_TimerT2 := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 0.96); var float v_TimerT3 := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 1.920); //@sic R5s200823 sic@ var SubFrameTiming_Type v_TimingDRB; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var boolean v_DRB_Flag := false; var boolean v_PRACH_Flag := false; timer t_Watchdog := 5.0; if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_SubFrameNum := (8,9); //WA#WI=984107 } else //FR1 FDD { v_SubFrameNum := (2,6,9); } } else { // FR2 // with SCS =120, 10 subframes with each 8 slots. Hence slots 0 ..39 map to sub frames 0..4 v_SubFrameNum := (1,2,3,4,6,7,8,9); } // Enable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // configure RACH procedure suitable for steps 4 to 17. f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // @siclog "Step 1" siclog@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,200); // Timing of DL // Configure SS transmission of Timing advance f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE(-,'011111'B))); //@sic R5s210694 sic@ // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop); // @siclog "Steps 2" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 700); //Time alignemnet is 750ms // @siclog "Steps 3" siclog@ // PDCP SDU =7B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header + 2B Padding Sub PDU =17B=136 bits (an allowed TB size as per 38.523-3 Annex B) t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //cs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_NR(v_TimingDL), {crs_NR_PDCP_SDU_7B} )); // @siclog "Steps 3" siclog@ // preconfigured at SS to not allocate grants // @siclog "Steps 4" siclog@ alt { [ ] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { repeat; }; [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo(?, v_SubFrameNum), v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd { v_FrameNum := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.SFN.Number; v_SubFrameNum_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Subframe.Number; if (not v_IsFR1) {v_Slot_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Slot.SlotOffset.Numerology3;}; //@sic R5s200043 sic@ // FR2 4,9,14,19,24,29,34,39 //@sic R5-198323 R5s200111 R5-201100 R5-201210 sic@ if ((v_IsFR1 and ischosen(v_NR_DuplexMode.TDD) and (v_FrameNum mod 2) == 1 and (v_SubFrameNum_Rcvd == 8 or v_SubFrameNum_Rcvd == 9)) or //TDD //WA#Wi=984107 (v_IsFR1 and (not ischosen(v_NR_DuplexMode.TDD)) and (v_FrameNum mod 8) == 1 and (v_SubFrameNum_Rcvd == 2 or v_SubFrameNum_Rcvd == 6 or v_SubFrameNum_Rcvd == 9)) or //FDD //Fr2 (( not v_IsFR1 ) and (v_FrameNum mod 4) == 1 and ((v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 0) // slot 8 @sic R5s211590 sic@ or (v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 1) //slot 9 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 2) //slot 18 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 3) //slot 19 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 4) //slot 28 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 5) // slot 29 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 6) // slot 38 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 7) //slot 39 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 0) // slot 48 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 1) //slot 49 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 2) // slot 58 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 3) //slot 59 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 4) // slot 68 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 5) //slot 69 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 6) // slot 78 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 7))//slot 79 )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4 "); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4, wrong frame number used "); } }; }; // @siclog "Steps 5" siclog@ //@sic R5-198323 R5s200111 R5-201100 R5-201210 sic@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo((v_FrameNum, v_FrameNum+2, v_FrameNum+4, v_FrameNum+8), v_SubFrameNum), v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd; v_FrameNum2 := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.SFN.Number; v_SubFrameNum_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Subframe.Number; if (not v_IsFR1) {v_Slot_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Slot.SlotOffset.Numerology3;};//@sic R5s200043 sic@ if ((v_IsFR1 and ischosen(v_NR_DuplexMode.TDD) and (v_FrameNum2 == v_FrameNum or v_FrameNum2 == v_FrameNum+2 )and (v_SubFrameNum_Rcvd == 8 or v_SubFrameNum_Rcvd == 9)) or //TDD //WA#WI=984107 (v_IsFR1 and (not ischosen(v_NR_DuplexMode.TDD)) and (v_FrameNum2 == v_FrameNum+8) and (v_SubFrameNum_Rcvd == 2 or v_SubFrameNum_Rcvd == 6 or v_SubFrameNum_Rcvd == 9)) or //FDD //FR2 (( not v_IsFR1 ) and (v_FrameNum2 == v_FrameNum or v_FrameNum2 == v_FrameNum+4)and ((v_SubFrameNum_Rcvd == 0 and v_Slot_Rcvd == 8) // slot 8 or (v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 1) //slot 9 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 2) //slot 18 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 3) //slot 19 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 4) //slot 28 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 5) // slot 29 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 6) // slot 38 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 7) //slot 39 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 0) // slot 48 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 1) //slot 49 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 2) // slot 58 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 3) //slot 59 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 4) // slot 68 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 5) //slot 69 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 6) // slot 78 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 7)) //slot 79 )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 "); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5, wrong frame number used "); } // @siclog "Steps 6" siclog@