3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220934

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2673 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC UAC test case 11.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 8b1, if the received REGISTRATION REQUEST contents do not match the expected value (e.g. UE sends any registration type other than ‘emergency’) the TC will proceed to step 10a1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an ‘else’ branch to give a fail verdict if incorrect REGISTRATION REQUEST contents are received at step 8b1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A non-conformant UE may be able to pass this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | fl_TC_11_3_7_TestBody |
| --- | --- |
| Reason for change | At step 8b1, if the received REGISTRATION REQUEST contents do not match the expected value (e.g. UE sends any registration type other than ‘emergency’) the TC will proceed to step 10a1. |
| Summary of change | Added an ‘else’ clause to give a fail verdict if incorrect REGISTRATION REQUEST contents are received at step 8b1 |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted. Please see MCC160 additional implementation for verdict of Service Request below. |

Before Change:

function fl_TC_11_3_7_TestBody() runs on NR5GC_PTC { ………………. //@siclog "Step 8b2" siclog@ //@sic R5s210424, R5-214742 sic@ //Steps 5-15 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.5.2.2-2 f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Emergency); SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('011'B),//initial emergency request omit, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message or Registration Request Message Failed"); } ………… }

After Change:

function fl_TC_11_3_7_TestBody() runs on NR5GC_PTC { …………… //@siclog "Step 8b2" siclog@ //@sic R5s210424, R5-214742 sic@ //Steps 5-15 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.5.2.2-2 f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Emergency); SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('011'B),//initial emergency request omit, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message or Registration Request Message Failed"); } ……………. }

Additional comments from MCC160

function fl_TC_11_3_7_TestBody() runs on NR5GC_PTC

{

…

//SERVICE REQUEST

if (match (v_ReceivedMsg.Pdu.Msg, cr_NG_SERVICE_REQUEST (?, ?, ?, omit, omit, omit, *))){ // @sic R5s210731 sic@

//@siclog "Step 8a1" siclog@

//The UE transmits an RRCSetupComplete message AND a SERVICE REQUEST message with 'Service type' set to 'high priority access'.

if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0101'B)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Service Request");

//@siclog "Step 8a2" siclog@

//Steps 4-8 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.9.11.

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg);

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

v_ReceivedAsp.Signalling.Nas[0].Pdu,

-, -, -, -,

crs_SSC_Mode('A'H, '001'B),

-, -, -, -, -, -,

cr_NG_Request_Type('011'B),//initial emergency request

omit,

omit)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

}

else { // @sic R5s220934 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

}

//REGISTRATION REQUEST

…

}