3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220943

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4680 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to functions f_Encode_RRCConnectionReconfiguration_HOToEUTRA and f_Encode_RRCConnectionReconfiguration_HOToEUTRA_CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-07-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect template usage leads to omission of dl_CarrierFreq IE in RRCConnectionReconfiguration message during handover and causing the handover to fail |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced template cs_CarrierFreqEUTRA with cs_CarrierFreqEUTRA_v9e0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2, 8.4.2.4, 8.4.2.7.1, 8.4.2.7.2, 8.4.2.7.3, 8.6.3.4, 8.6.5.1, 8.6.5.1a, 8.6.5.2, 8.6.7.4, 8.6.9.1, 13.4.2.6, 13.4.2.7, 13.4.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_Encode_RRCConnectionReconfiguration_HOToEUTRA |
| --- | --- |
| Reason for change | Incorrect template usage leads to omission of dl_CarrierFreq IE in RRCConnectionReconfiguration message during handover and causing the handover to fail |
| Summary of change | Replaced template cs_CarrierFreqEUTRA with cs_CarrierFreqEUTRA_v9e0 |
| TTCN module | EUTRA_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_Encode_RRCConnectionReconfiguration_HOToEUTRA(CoOrd_EUTRASysInfo_Type p_EUTRASysInfo,

integer p_N := 1,

B3_Type p_NasKsi := '000'B,

boolean p_ProseNotUpdatedFreq := true) return octetstring

{

var bitstring v_Bitstring;

var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_ConvertIntegerToDL_Bandwidth(p_EUTRASysInfo.DL_Bandwidth),

f_ConvertIntegerToUL_Bandwidth(p_EUTRASysInfo.HO.UL_Bandwidth));

var template (value) DRB_ToAddModList v_DRB_ToAddModList := { cs_508_DRB_ToAddMod_DEFAULT_AM(tsc_DRB1) }

var template(omit) CarrierFreqEUTRA v_CarrierFreqEUTRA := omit;

var template(omit) CarrierFreqEUTRA_v9e0 v_CarrierFreqEUTRA_v9e0 := omit;

v_CarrierFreqEUTRA := f_Return_CarrierFreqEUTRA (cs_CarrierFreqEUTRA(p_EUTRASysInfo.Arfcn, omit), p_EUTRASysInfo.UL_Arfcn); // @sic R5-217955 sic@

v_CarrierFreqEUTRA_v9e0 := f_Return_CarrierFreqEUTRA_v9e0 (cs_CarrierFreqEUTRA_v9e0(p_EUTRASysInfo.Arfcn, omit)); // @sic R5-217955 sic@

//<=== SKIPPED ===>

return bit2oct(f_OctetAlignedBitString(v_Bitstring));  // NOTE: bit2oct would add additional bits at the beginning (MSB) whereas here it has be added at the end (LSB)

}

After change:

function f_Encode_RRCConnectionReconfiguration_HOToEUTRA(CoOrd_EUTRASysInfo_Type p_EUTRASysInfo,

integer p_N := 1,

B3_Type p_NasKsi := '000'B,

boolean p_ProseNotUpdatedFreq := true) return octetstring

{

var bitstring v_Bitstring;

var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_ConvertIntegerToDL_Bandwidth(p_EUTRASysInfo.DL_Bandwidth),

f_ConvertIntegerToUL_Bandwidth(p_EUTRASysInfo.HO.UL_Bandwidth));

var template (value) DRB_ToAddModList v_DRB_ToAddModList := { cs_508_DRB_ToAddMod_DEFAULT_AM(tsc_DRB1) }

var template(omit) CarrierFreqEUTRA v_CarrierFreqEUTRA := omit;

var template(omit) CarrierFreqEUTRA_v9e0 v_CarrierFreqEUTRA_v9e0 := omit;

v_CarrierFreqEUTRA := f_Return_CarrierFreqEUTRA (cs_CarrierFreqEUTRA_v9e0(p_EUTRASysInfo.Arfcn, omit), p_EUTRASysInfo.UL_Arfcn); // @sic R5-217955 sic@

v_CarrierFreqEUTRA_v9e0 := f_Return_CarrierFreqEUTRA_v9e0 (cs_CarrierFreqEUTRA_v9e0(p_EUTRASysInfo.Arfcn, omit)); // @sic R5-217955 sic@

//<=== SKIPPED ===>

return bit2oct(f_OctetAlignedBitString(v_Bitstring));  // NOTE: bit2oct would add additional bits at the beginning (MSB) whereas here it has be added at the end (LSB)

}

## 2.2 Change 2

| Function name | f_Encode_RRCConnectionReconfiguration_HOToEUTRA_CA |
| --- | --- |
| Reason for change | Incorrect template usage leads to omission of dl_CarrierFreq IE in RRCConnectionReconfiguration message during handover and causing the handover to fail |
| Summary of change | Replaced template cs_CarrierFreqEUTRA with cs_CarrierFreqEUTRA_v9e0 |
| TTCN module | EUTRA_CapabilityFunctions_CA.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_Encode_RRCConnectionReconfiguration_HOToEUTRA_CA(CoOrd_EUTRASysInfo_Type p_EUTRASysInfo,

CoOrd_EUTRASysInfo_Type p_EUTRASysInfo_Scell

) return octetstring

{

var bitstring v_Bitstring;

var integer v_PrimaryBand := f_EUTRA_GetCA_PrimaryBand(px_EUTRA_CA_BandCombination);

var integer v_SecondaryBand := f_EUTRA_GetCA_SecondaryBand(px_EUTRA_CA_BandCombination);

//@sic R5-169501 sic@

var boolean  v_PcellISTDD := not (f_EUTRA_FrequencyBandIsFDD(v_PrimaryBand)); // If EUTRA is FDD or TDD

var boolean  v_ScellISTDD := not (f_EUTRA_FrequencyBandIsFDD(v_SecondaryBand)); // If EUTRA is FDD or TDD

var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_ConvertIntegerToDL_Bandwidth(p_EUTRASysInfo.DL_Bandwidth),

f_ConvertIntegerToUL_Bandwidth(p_EUTRASysInfo.HO.UL_Bandwidth));

var boolean v_UL_CA := p_EUTRASysInfo.UL_CA; //@sic R5s170737 R5-183277 sic@

var template (omit) CommonSCell_UL_Configuration_Type v_CommonSCell_UL_Configuration := omit;

var template (omit)  DedicatedSCell_UL_Configuration_Type v_DedicatedSCell_UL_Configuration := omit;

var template (value) SCellToAddMod_r10 v_SCellToAddMod;

var SCell_DL_Bandwidth_Type v_SCell_DL_Bandwidth ;

var template(omit) TDD_Config v_TDD_Config := omit;  // omitted in FDD

var template (value) PUCCH_ConfigDedicated_v1020 v_PUCCH_ConfigDedicated :=cs_508_PUCCH_ConfigDedicated_v1020_1TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX);

//@sic R5-171366 sic@

var template (omit)  LAA_SCellConfiguration_r13 v_LAA_SCellConfiguration := omit;

var template(omit) CarrierFreqEUTRA v_CarrierFreqEUTRA := omit; //@sic R5-172905 sic@

var template(omit) CarrierFreqEUTRA_v9e0 v_CarrierFreqEUTRA_v9e0 := omit;

var boolean v_LAA_Scell := (v_SecondaryBand == 46);

v_CarrierFreqEUTRA := f_Return_CarrierFreqEUTRA (cs_CarrierFreqEUTRA(p_EUTRASysInfo.Arfcn, omit), p_EUTRASysInfo.UL_Arfcn); // @sic R5-217955 sic@

v_CarrierFreqEUTRA_v9e0 := f_Return_CarrierFreqEUTRA_v9e0 (cs_CarrierFreqEUTRA_v9e0(p_EUTRASysInfo.Arfcn, omit)); // @sic R5-217955 sic@

//<=== SKIPPED ===>

return bit2oct(f_OctetAlignedBitString(v_Bitstring));  // NOTE: bit2oct would add additional bits at the beginning (MSB) whereas here it has be added at the end (LSB)

}

After change:

function f_Encode_RRCConnectionReconfiguration_HOToEUTRA_CA(CoOrd_EUTRASysInfo_Type p_EUTRASysInfo,

CoOrd_EUTRASysInfo_Type p_EUTRASysInfo_Scell

) return octetstring

{

var bitstring v_Bitstring;

var integer v_PrimaryBand := f_EUTRA_GetCA_PrimaryBand(px_EUTRA_CA_BandCombination);

var integer v_SecondaryBand := f_EUTRA_GetCA_SecondaryBand(px_EUTRA_CA_BandCombination);

//@sic R5-169501 sic@

var boolean  v_PcellISTDD := not (f_EUTRA_FrequencyBandIsFDD(v_PrimaryBand)); // If EUTRA is FDD or TDD

var boolean  v_ScellISTDD := not (f_EUTRA_FrequencyBandIsFDD(v_SecondaryBand)); // If EUTRA is FDD or TDD

var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_ConvertIntegerToDL_Bandwidth(p_EUTRASysInfo.DL_Bandwidth),

f_ConvertIntegerToUL_Bandwidth(p_EUTRASysInfo.HO.UL_Bandwidth));

var boolean v_UL_CA := p_EUTRASysInfo.UL_CA; //@sic R5s170737 R5-183277 sic@

var template (omit) CommonSCell_UL_Configuration_Type v_CommonSCell_UL_Configuration := omit;

var template (omit)  DedicatedSCell_UL_Configuration_Type v_DedicatedSCell_UL_Configuration := omit;

var template (value) SCellToAddMod_r10 v_SCellToAddMod;

var SCell_DL_Bandwidth_Type v_SCell_DL_Bandwidth ;

var template(omit) TDD_Config v_TDD_Config := omit;  // omitted in FDD

var template (value) PUCCH_ConfigDedicated_v1020 v_PUCCH_ConfigDedicated :=cs_508_PUCCH_ConfigDedicated_v1020_1TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX);

//@sic R5-171366 sic@

var template (omit)  LAA_SCellConfiguration_r13 v_LAA_SCellConfiguration := omit;

var template(omit) CarrierFreqEUTRA v_CarrierFreqEUTRA := omit; //@sic R5-172905 sic@

var template(omit) CarrierFreqEUTRA_v9e0 v_CarrierFreqEUTRA_v9e0 := omit;

var boolean v_LAA_Scell := (v_SecondaryBand == 46);

v_CarrierFreqEUTRA := f_Return_CarrierFreqEUTRA (cs_CarrierFreqEUTRA_v9e0(p_EUTRASysInfo.Arfcn, omit), p_EUTRASysInfo.UL_Arfcn); // @sic R5-217955 sic@

v_CarrierFreqEUTRA_v9e0 := f_Return_CarrierFreqEUTRA_v9e0 (cs_CarrierFreqEUTRA_v9e0(p_EUTRASysInfo.Arfcn, omit)); // @sic R5-217955 sic@

//<=== SKIPPED ===>

return bit2oct(f_OctetAlignedBitString(v_Bitstring));  // NOTE: bit2oct would add additional bits at the beginning (MSB) whereas here it has be added at the end (LSB)

}