3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220954

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4683 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for LTE RRC test case 8.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following implementation of prose CR R5-221221, a Conformant UE operating at Rel-15 or later fail the capability checking as it may not include phyLayerParameters_v1530 if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are not supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an auxilliary function to allow phyLayerParameters_v1530 to be ommitted if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE Rel-15 or later may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | UE_EUTRA_Capability_v1540_IEs |
| --- | --- |
| Reason for change | Following implementation of prose CR R5-221221, a Conformant UE operating at Rel-15 or later fail the capability checking as it may not include phyLayerParameters_v1530 if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are not supported. |
| Summary of change | Add an auxilliary function to allow phyLayerParameters_v1530 to be ommitted if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are not supported. |
| TTCN module | RRC_OtherFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

template UE_EUTRA_Capability_v1440_IEs cr_UE_EUTRA_Capability_v1440_IEs (template UE_EUTRA_Capability_v1540_IEs p_UE_EUTRA_Capability_v1540_IEs := *) := { /* @sic R5s180552 BASELINE MOVING 2018 sic@ */ /* @status APPROVED (LTE) */ lwa_Parameters_v1440 := ?, mac_Parameters_v1440 := ?, nonCriticalExtension := { phyLayerParameters_v1450 := *, rf_Parameters_v1450 := *, otherParameters_v1450 := ?, ue_CategoryDL_v1450 := *, // included in checks of categories nonCriticalExtension := { ue_CategoryDL_v1460 := *, // included in checks of categories otherParameters_v1460 := ?, nonCriticalExtension := { irat_ParametersNR_r15 := *, featureSetsEUTRA_r15 := *, pdcp_ParametersNR_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v1510 := *, tdd_Add_UE_EUTRA_Capabilities_v1510 := *, nonCriticalExtension := { measParameters_v1520 := ?, nonCriticalExtension := { measParameters_v1530 := f_Derive_measParameters_v1530(), // @sic R5-194084 sic@ otherParameters_v1530 := f_Derive_otherParameters_v1530(), // @sic R5-194084 sic@ neighCellSI_AcquisitionParameters_v1530 := *, mac_Parameters_v1530 := *, phyLayerParameters_v1530 := { // @sic R5-221221 sic@ stti_SPT_Capabilities_r15 := *, ce_Capabilities_r15 := { ce_CRS_IntfMitig_r15 := *, ce_CQI_AlternativeTable_r15 := *, ce_PDSCH_FlexibleStartPRB_CE_ModeA_r15 := f_Derive_CE_PDSCH_FlexibleStartPRB_CE_ModeA_r15(), // @sic R5-221221 sic@ ce_PDSCH_FlexibleStartPRB_CE_ModeB_r15 := *, ce_PDSCH_64QAM_r15 := *, ce_PUSCH_FlexibleStartPRB_CE_ModeA_r15 := f_Derive_CE_PUSCH_FlexibleStartPRB_CE_ModeA_r15(), // @sic R5-221221 sic@ ce_PUSCH_FlexibleStartPRB_CE_ModeB_r15 := *, ce_PUSCH_SubPRB_Allocation_r15 := *, ce_UL_HARQ_ACK_Feedback_r15 := * }, shortCQI_ForSCellActivation_r15 := *, mimo_CBSR_AdvancedCSI_r15 := *, crs_IntfMitig_r15 := *, ul_PowerControlEnhancements_r15 := *, urllc_Capabilities_r15 := *, altMCS_Table_r15 := * }, // phyLayerParameters-v1530 rf_Parameters_v1530 := *, pdcp_Parameters_v1530 := *, ue_CategoryDL_v1530 := *, // no checks defined yet ue_BasedNetwPerfMeasParameters_v1530 := *, rlc_Parameters_v1530 := *, sl_Parameters_v1530 := *, extendedNumberOfDRBs_r15 := *, reducedCP_Latency_r15 := *, laa_Parameters_v1530 := *, ue_CategoryUL_v1530 := *, // no checks defined yet fdd_Add_UE_EUTRA_Capabilities_v1530 := *, tdd_Add_UE_EUTRA_Capabilities_v1530 := *, nonCriticalExtension := p_UE_EUTRA_Capability_v1540_IEs /* @sic R5s201386 BASELINE MOVING 2020 sic@ */ } ifpresent // UE-EUTRA-Capability-v1530-IEs } ifpresent // UE-EUTRA-Capability-v1520-IEs } ifpresent // UE-EUTRA-Capability-v1510-IEs } ifpresent // UE-EUTRA-Capability-v1460-IEs } ifpresent // UE-EUTRA-Capability-v1450-IEs } ifpresent;

After Change:

template UE_EUTRA_Capability_v1440_IEs cr_UE_EUTRA_Capability_v1440_IEs (template UE_EUTRA_Capability_v1540_IEs p_UE_EUTRA_Capability_v1540_IEs := *) := { /* @sic R5s180552 BASELINE MOVING 2018 sic@ */ /* @status APPROVED (LTE) */ lwa_Parameters_v1440 := ?, mac_Parameters_v1440 := ?, nonCriticalExtension := { phyLayerParameters_v1450 := *, rf_Parameters_v1450 := *, otherParameters_v1450 := ?, ue_CategoryDL_v1450 := *, // included in checks of categories nonCriticalExtension := { ue_CategoryDL_v1460 := *, // included in checks of categories otherParameters_v1460 := ?, nonCriticalExtension := { irat_ParametersNR_r15 := *, featureSetsEUTRA_r15 := *, pdcp_ParametersNR_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v1510 := *, tdd_Add_UE_EUTRA_Capabilities_v1510 := *, nonCriticalExtension := { measParameters_v1520 := ?, nonCriticalExtension := { measParameters_v1530 := f_Derive_measParameters_v1530(), // @sic R5-194084 sic@ otherParameters_v1530 := f_Derive_otherParameters_v1530(), // @sic R5-194084 sic@ neighCellSI_AcquisitionParameters_v1530 := *, mac_Parameters_v1530 := *, phyLayerParameters_v1530 := f_Derive_phyLayerParameters_v1530(), // changes for @sic R5-221221 sic@ rf_Parameters_v1530 := *, pdcp_Parameters_v1530 := *, ue_CategoryDL_v1530 := *, // no checks defined yet ue_BasedNetwPerfMeasParameters_v1530 := *, rlc_Parameters_v1530 := *, sl_Parameters_v1530 := *, extendedNumberOfDRBs_r15 := *, reducedCP_Latency_r15 := *, laa_Parameters_v1530 := *, ue_CategoryUL_v1530 := *, // no checks defined yet fdd_Add_UE_EUTRA_Capabilities_v1530 := *, tdd_Add_UE_EUTRA_Capabilities_v1530 := *, nonCriticalExtension := p_UE_EUTRA_Capability_v1540_IEs /* @sic R5s201386 BASELINE MOVING 2020 sic@ */ } ifpresent // UE-EUTRA-Capability-v1530-IEs } ifpresent // UE-EUTRA-Capability-v1520-IEs } ifpresent // UE-EUTRA-Capability-v1510-IEs } ifpresent // UE-EUTRA-Capability-v1460-IEs } ifpresent // UE-EUTRA-Capability-v1450-IEs } ifpresent;

## Change 2

| Function name | UE_EUTRA_Capability_v1540_IEs f_Derive_phyLayerParameters_v1530 |
| --- | --- |
| Reason for change | Following implementation of prose CR R5-221221, a Conformant UE operating at Rel-15 or later fail the capability checking as it may not include phyLayerParameters_v1530 if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are not supported. |
| Summary of change | Add an auxilliary function to allow phyLayerParameters_v1530 to be ommitted if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are not supported. |
| TTCN module | RRC_OtherFunctions.ttcn |
| MCC160 Comment | Accepted |

New function:

function f_Derive_phyLayerParameters_v1530() return template PhyLayerParameters_v1530_Type { var template PhyLayerParameters_v1530_Type v_PhyLayerParameters_v1530 := omit; v_PhyLayerParameters_v1530 := { stti_SPT_Capabilities_r15 := *, ce_Capabilities_r15 := { ce_CRS_IntfMitig_r15 := *, ce_CQI_AlternativeTable_r15 := *, ce_PDSCH_FlexibleStartPRB_CE_ModeA_r15 := f_Derive_CE_PDSCH_FlexibleStartPRB_CE_ModeA_r15(), // @sic R5-221221 sic@ ce_PDSCH_FlexibleStartPRB_CE_ModeB_r15 := *, ce_PDSCH_64QAM_r15 := *, ce_PUSCH_FlexibleStartPRB_CE_ModeA_r15 := f_Derive_CE_PUSCH_FlexibleStartPRB_CE_ModeA_r15(), // @sic R5-221221 sic@ ce_PUSCH_FlexibleStartPRB_CE_ModeB_r15 := *, ce_PUSCH_SubPRB_Allocation_r15 := *, ce_UL_HARQ_ACK_Feedback_r15 := * }, shortCQI_ForSCellActivation_r15 := *, mimo_CBSR_AdvancedCSI_r15 := *, crs_IntfMitig_r15 := *, ul_PowerControlEnhancements_r15 := *, urllc_Capabilities_r15 := *, altMCS_Table_r15 := * } if (pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH) { return v_PhyLayerParameters_v1530; } else { return v_PhyLayerParameters_v1530 ifpresent; } }

Additional comment from MCC160

New type definition:

// ASN.1 types needed to handle otherParameters_v1530_IEs @sic R5s220954 sic@ type UE_EUTRA_Capability_v1530_IEs.phyLayerParameters_v1530 PhyLayerParameters_v1530_Type; /* @status */