3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220957

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2685 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_NR_AddMod_1SCell_ENDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the function f_NR_AddMod_1SCell_ENDC, Ul grant configuration is started for Scell if p_UL_CA is set to True . In a case where the UE is not supporting PUCCH on Scell, the UE will not have PUCCH configuration to initiate SR. Hence it is proposed to add p_UL_CA_ConfigPUCCH also ,as part of check before configuring Ul grant on Scell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_NR_AddMod_1SCell_ENDC , before calling f_NR_ULGrantConfiguration_Start , (p_UL_CA and p_UL_CA_ConfigPUCCH) condition is checked |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.8.1.ENDC, 7.1.1.3.8.2.ENDC, 7.1.1.3.8.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1	Correction to the function f_NR_AddMod_1SCell_ENDC	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Correction to the function f_NR_AddMod_1SCell_ENDC

| Template name | f_NR_AddMod_1SCell_ENDC |
| --- | --- |
| Reason for change | In the function f_NR_AddMod_1SCell_ENDC, Ul grant configuration is started for Scell if p_UL_CA is set to True . In a case where the UE is not supporting PUCCH on Scell, the UE will not have PUCCH configuration to initiate SR. Hence it is proposed to add p_UL_CA_ConfigPUCCH also ,as part of check before configuring Ul grant on Scell. |
| Summary of change | In the function f_NR_AddMod_1SCell_ENDC , before calling f_NR_ULGrantConfiguration_Start , (p_UL_CA and p_UL_CA_ConfigPUCCH) condition is checked |
| TTCN module | NR_CommonFunctions_ENDC_CA.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_AddMod_1SCell_ENDC(NR_CellId_Type p_PSCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex := tsc_SCellIndex_2, template (omit) NR_RadioBearerList_Type p_SS_RadioBearerList := omit, template (omit) NR_RLC_BearerToAddModList_Type p_NR_RLC_BearerToAddModList := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, boolean p_UL_CA := false, template(omit) ServingCellConfig.sCellDeactivationTimer p_SCellDeactivationTimer := omit, boolean p_UL_CA_ConfigPUCCH := true ) runs on NR_BASE_PTC { //@sic R5-214764 : added p_UL_CA_ConfigPUCCH sic@ var template(value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, p_SCellIndex, p_UL_CA, p_SCellDeactivationTimer, -, p_UL_CA_ConfigPUCCH); if (isvalue(p_NR_RLC_BearerToAddModList)) { v_SCGConfig.rlc_BearerToAddModList:= p_NR_RLC_BearerToAddModList; } f_NR_ConfigPCell_AddRelSCells(p_PSCell, {p_SCell}, tsc_SCellIndex_1, cs_TimingInfo_Now, f_NR_CellInfo_GetRNTI(p_PSCell), p_SS_RadioBearerList); f_NR_ConfigSCell(p_SCell,v_SCGConfig, cs_TimingInfo_Now); //Send RRC Reconfiguration on E-UTRA side f_NR_SendRRCReconfigurationContentsToEUTRA(p_RadioBearerConfig, v_SCGConfig); if (p_UL_CA) { f_NR_ULGrantConfiguration_Start(p_SCell); } }

After Change:

function f_NR_AddMod_1SCell_ENDC(NR_CellId_Type p_PSCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex := tsc_SCellIndex_2, template (omit) NR_RadioBearerList_Type p_SS_RadioBearerList := omit, template (omit) NR_RLC_BearerToAddModList_Type p_NR_RLC_BearerToAddModList := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, boolean p_UL_CA := false, template(omit) ServingCellConfig.sCellDeactivationTimer p_SCellDeactivationTimer := omit, boolean p_UL_CA_ConfigPUCCH := true ) runs on NR_BASE_PTC { //@sic R5-214764 : added p_UL_CA_ConfigPUCCH sic@ var template(value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, p_SCellIndex, p_UL_CA, p_SCellDeactivationTimer, -, p_UL_CA_ConfigPUCCH); if (isvalue(p_NR_RLC_BearerToAddModList)) { v_SCGConfig.rlc_BearerToAddModList:= p_NR_RLC_BearerToAddModList; } f_NR_ConfigPCell_AddRelSCells(p_PSCell, {p_SCell}, tsc_SCellIndex_1, cs_TimingInfo_Now, f_NR_CellInfo_GetRNTI(p_PSCell), p_SS_RadioBearerList); f_NR_ConfigSCell(p_SCell,v_SCGConfig, cs_TimingInfo_Now); //Send RRC Reconfiguration on E-UTRA side f_NR_SendRRCReconfigurationContentsToEUTRA(p_RadioBearerConfig, v_SCGConfig); if (p_UL_CA and p_UL_CA_ConfigPUCCH) { //WA#WI=971353 f_NR_ULGrantConfiguration_Start(p_SCell); } }