3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220987

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2702 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC CAG Testcase 6.5.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Similar to the change for TC 6.5.2.1 in R5s220857, TC 6.5.2.2 requires that the function f_NR_CellInfo_SetPLMN be called prior to F_NR_CellInfo_Set_SIB1_SetPNI_NPN_IdentityInfoList to properly configure the CagInfoList IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList f_NR_CellInfo_SetPLMN; Called: f_NR_CellInfo_SetPLMN f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | function f_TC_6_5_2_2_NR5GC() |
| --- | --- |
| Reason for change | Similar to the change for TC 6.5.2.1 in R5s220857, TC 6.5.2.2 requires that the function f_NR_CellInfo_SetPLMN be called prior to F_NR_CellInfo_Set_SIB1_SetPNI_NPN_IdentityInfoList to properly configure the CagInfoList IEs. |
| Summary of change | Instead of calling f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList f_NR_CellInfo_SetPLMN; Called: f_NR_CellInfo_SetPLMN f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList; |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_5_2_2_NR5GC() runs on NR5GC_PTC { //CAG Selection in Automatic Mode var NG_NAS_MSG_Indication_Type v_ReceivedMsg; f_NR5GC_Init(NR_2); //Cell2 and Cell4 are CAG Cells. default CAG-ID value used for Cell2, value 2 used for Cell4 f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell4, 2); f_NR_CellInfo_SetPLMN(nr_Cell2, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell4, tsc_NR_HPLMN_003_21);//PLMN3 f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//PLMN2 // @sic R5-220531 sic@ f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell4, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -88, -88); //FR2 FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); //CAG Cell f_NR_CellConfig_Def(nr_Cell4); //CAG Cell f_NR_CellConfig_Def(nr_Cell11); //The UE is in Automatic PLMN selection mode. f_UT_AutomaticPLMN_Select (UT); //Make the pre-test condition satisfied v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1. fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_EmptyCAGInfoList); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); //SS adjusts cell levels according to row T0 of table 6.5.2.2.3.2-1 for FR1 and table 6.5.2.2.3.2-2 for FR2 //f_NR_SetCellPower (nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //Anritsu TTCN CR R5s220864 f_NR_TestBody_Set(true); fl_TC_6_5_2_2_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); }

After Change:

function f_TC_6_5_2_2_NR5GC() runs on NR5GC_PTC { //CAG Selection in Automatic Mode var NG_NAS_MSG_Indication_Type v_ReceivedMsg; f_NR5GC_Init(NR_2); f_NR_CellInfo_SetPLMN(nr_Cell2, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell4, tsc_NR_HPLMN_003_21);//PLMN3 f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//PLMN2 // @sic R5-220531 sic@ //Cell2 and Cell4 are CAG Cells. default CAG-ID value used for Cell2, value 2 used for Cell4 f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell4, 2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell4, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -88, -88); //FR2 FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); //CAG Cell f_NR_CellConfig_Def(nr_Cell4); //CAG Cell f_NR_CellConfig_Def(nr_Cell11); //The UE is in Automatic PLMN selection mode. f_UT_AutomaticPLMN_Select (UT); //Make the pre-test condition satisfied v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1. fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_EmptyCAGInfoList); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); //SS adjusts cell levels according to row T0 of table 6.5.2.2.3.2-1 for FR1 and table 6.5.2.2.3.2-2 for FR2 //f_NR_SetCellPower (nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //Anritsu TTCN CR R5s220864 f_NR_TestBody_Set(true); fl_TC_6_5_2_2_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); }