3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221010

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0860 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS XCAP test case 15.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-09-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following implementation of R5-216570, only the 1st rule in a ruleset in the simservs document is checked. It is possible for some deivces that the supplementary service flavours can only be activated in some combinations, e.g. for test case 15.10 a device may activate rules for both CFNRc (not reachable) and CFNL (not logged in). In this case the condition to bec ehcked may be in the 2nd rule in a ruleset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes made for the f_SupplementaryService_Check() to traverse through all the rules in a ruleset configured by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail some test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Change 1	3

Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_SupplementaryService_Check() |
| --- | --- |
| Reason for change | Following implementation of R5-216570, only the 1st rule in a ruleset in the simservs document is checked. It is possible for some deivces that the supplementary service flavours can only be activated in some combinations, e.g. for test case 15.10 a device may activate rules for both CFNRc (not reachable) and CFNL (not logged in). In this case the condition to bec ehcked may be in the 2nd rule in a ruleset. |
| Summary of change | Changes made for the f_SupplementaryService_Check() to traverse through all the rules in a ruleset configured by the UE. |
| TTCN module | IMS_XCAP_Simserv.ttcn |
| MCC160 Comment | Rejected: The test case prose of XCAP test cases shall specify exactly which rules are expected. If there are any optional rules e.g. for test case 15.10 then this shall be specified in the prose. Currently according to 34.229-1/5 there is no test case specifying more than one rule. The test case prose shall be changed to specify optional rules where needed. TTCN will be changed accordingly after agreement by RAN5. |

Before Change

function f_SupplementaryService_Check(Ruleset p_Ruleset, SS_Type p_SupplementaryService, boolean p_Deactivated := false) runs on IMS_PTC return boolean { var boolean v_RulesetOK := true; var RuleType v_Rule; var XSD.AnyURI v_TargetAddress; // check ruleset for communication forwarding, communication barring select (p_SupplementaryService) { case (SS_CFU, SS_CFNRy, SS_CFB, SS_CFNL, SS_CFNRc) { v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_TargetAddress := px_XCAP_TargetUri; v_RulesetOK := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); } case (SS_BAIC, SS_BAIC_ANONYMOUS, SS_BIC_ROAM, SS_BAOC, SS_BOIC, SS_BOIC_EXHC, SS_BARRING_OUT_ROAM) { // @sic R5-153995 sic@ v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_RulesetOK := fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated); } case (SS_BAIC_EXCEPT_SPECIFIC_USER) { v_RulesetOK := (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option1(p_Ruleset) or (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule1(p_Ruleset) and fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule2(p_Ruleset))); } } return v_RulesetOK; }

After Change

function f_SupplementaryService_Check(Ruleset p_Ruleset, SS_Type p_SupplementaryService, boolean p_Deactivated := false) runs on IMS_PTC return boolean { var boolean v_RulesetOK := true; var RuleType v_Rule; Var integer i; var boolean v_Rulecheck; // boolean to varify the rule outcome var XSD.AnyURI v_TargetAddress; // check ruleset for communication forwarding, communication barring select (p_SupplementaryService) { case (SS_CFU, SS_CFNRy, SS_CFB, SS_CFNL, SS_CFNRc) { v_TargetAddress := px_XCAP_TargetUri; v_RulesetOK := false; for (i := 0; i < lengthof(p_Ruleset.rule_list); i := i + 1) { v_Rule := p_Ruleset.rule_list[i]; fl_IMS_XCAP_CheckRuleId(v_Rule); v_Rulecheck := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); if(v_Rulecheck){ v_RulesetOK := true; } } } case (SS_BAIC, SS_BAIC_ANONYMOUS, SS_BIC_ROAM, SS_BAOC, SS_BOIC, SS_BOIC_EXHC, SS_BARRING_OUT_ROAM) { // @sic R5-153995 sic@ v_RulesetOK := false; for (i := 0; i < lengthof(p_Ruleset.rule_list); i := i + 1) { v_Rule := p_Ruleset.rule_list[i]; fl_IMS_XCAP_CheckRuleId(v_Rule); v_Rulecheck := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); if(v_Rulecheck){ v_RulesetOK := true; } } } case (SS_BAIC_EXCEPT_SPECIFIC_USER) { v_RulesetOK := (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option1(p_Ruleset) or (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule1(p_Ruleset) and fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule2(p_Ruleset))); } } return v_RulesetOK; }