3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221043

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0168 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of POS testcase 9.3.1.2.19s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add POS test case 9.3.1.2.19s to the IWD_22wk23 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.3.1.2.19s required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.19s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 37.571-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1	Verification Test Summary	2

2	Changes required to verify test case 9.3.1.2.19s	2

2.1 Correction to cs_NR_GapConfig_OTDOA	2

2.2. Correction to f_POS_NR_Set_DlPrsAssistanceDataPerTRP	3

2.3 Correction to LPP_Templates	5

2.4 Correction to f_POS_NR_SimulateDlPrsPOSifSupported	6

2.5 Correction to f_POS_NR_CellConfig_DlPrs	7

2.6 Correction to f_POS_NR_Set_DlPrsAssistanceDataPerTRP	8

2.7 Correction to f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList	9

3	Branches executed	12

4	Execution Log Files	12

4.1	Mediatek MT6879 UE	12

5	References	13

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of POS test case 9.3.1.2.19s which is part of the POS test suite in iWD_TTCN3-B2020-09_D22wk23.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	9.3.1.2.19s

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Mediatek MT6879 UE

Verification Status:	PASS

# Changes required to verify test case 9.3.1.2.19s

## 2.1 Correction to cs_NR_GapConfig_OTDOA

| Function name | cs_NR_GapConfig_OTDOA |
| --- | --- |
| Reason for change | According to Table 8.3.1-3 from 3GPP 37.571-2, the MeasGapConfig for NR PRS based test cases must configure a gapOffset of 3 for the UE. However, according to Table 8.4.1.6-3, the Slot offset is 11 ms. The mismatch between these 2 values causes a conformant UE to fail the test case. Therefore, this CR proposes to change the value of gapOffset from 3 to 10. |
| Summary of change | Change the value of gapOffset from 3 to 10 in cs_NR_GapConfig |
| TTCN module | NR5GC_Positioning_Templates.ttcn |
| MCC160 Comment | Accepted, conditional to approval of prose CR at RAN5#97 |

Before

<<SKIPPED CODE>>

template (omit) MeasConfig cds_NR_MeasConfig_MeasGap (template (omit) MeasGapConfig p_MeasGapConfig)

modifies cs_NR_MeasConfig_Base:=

{ /* @status    APPROVED (POS) */

// Acc Table 8.3.1-2 TS 37.571-2

measGapConfig := p_MeasGapConfig

};

template (value) GapConfig cs_NR_GapConfig_OTDOA := cs_NR_GapConfig(3, ms3, ms160, ms0); /* @status    APPROVED (POS) */

template (value) AdditionalInformation cs_RoutingInfoNR (octetstring p_RoutingId) :=

{ /* @status    APPROVED (POS) */

/* 24.501 cl. 9.11.2.1 */

// @sic R5s201391 sic@

iei := '24'O,

iel := '04'O,

additionalInfo := p_RoutingId

};

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

template (omit) MeasConfig cds_NR_MeasConfig_MeasGap (template (omit) MeasGapConfig p_MeasGapConfig)

modifies cs_NR_MeasConfig_Base:=

{ /* @status    APPROVED (POS) */

// Acc Table 8.3.1-2 TS 37.571-2

measGapConfig := p_MeasGapConfig

};

template (value) GapConfig cs_NR_GapConfig_OTDOA := cs_NR_GapConfig(10, ms3, ms160, ms0); /* @status    APPROVED (POS) */

template (value) AdditionalInformation cs_RoutingInfoNR (octetstring p_RoutingId) :=

{ /* @status    APPROVED (POS) */

/* 24.501 cl. 9.11.2.1 */

// @sic R5s201391 sic@

iei := '24'O,

iel := '04'O,

additionalInfo := p_RoutingId

};

<<SKIPPED CODE>>

## 2.2. Correction to f_POS_NR_Set_DlPrsAssistanceDataPerTRP

| Function name | f_POS_NR_Set_DlPrsAssistanceDataPerTRP |
| --- | --- |
| Reason for change | According to 3GPP 38.508-1 Table 4.4.2-2, the Physical cell identity for NR Cell 1 is 0. However, the assistance data from 37.571-2 Table 8.4.1.6-1 assigns 1 to nr-PhysCellID-r16. This causes UE not to report DL PRS measurements. |
| Summary of change | Change the value of nr-PhysCellID-r16 from 1 to 0 in cs_DlPrsAssistanceDataPerTRP for NR Cell 1 |
| TTCN module | NR5GC_Positioning_Testcases.ttcn |
| MCC160 Comment | Accepted, conditional to prose CR agreement at RAN5#97. |

Before

<<SKIPPED CODE>>

function f_POS_NR_Set_DlPrsAssistanceDataPerTRP(NR_CellId_Type p_CellId) runs on NR_BASE_PTC return template (value) NR_DL_PRS_AssistanceDataPerTRP_r16

{

var template (value) NR_DL_PRS_AssistanceDataPerTRP_r16 v_DlPrs_AssistanceDataPerTRP;

var NCGI_r15 v_CellGlobalId := f_POS_Get_CellGlobalId(p_CellId);

var ARFCN_ValueNR v_FrequencySSB := f_NR_CellInfo_GetFrequencySSB(p_CellId);

var NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 v_PeriodicityAndOffset := f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId);

select(p_CellId){

case (nr_Cell1) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(0, 1, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset));

}

case (nr_Cell2) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(1, 2, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset));

}

case (nr_Cell3) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(2, 3, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset));

}

case else {

FatalError (__FILE__, __LINE__, "Invalid Cell ID");

}

}

return v_DlPrs_AssistanceDataPerTRP;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_Set_DlPrsAssistanceDataPerTRP(NR_CellId_Type p_CellId) runs on NR_BASE_PTC return template (value) NR_DL_PRS_AssistanceDataPerTRP_r16

{

var template (value) NR_DL_PRS_AssistanceDataPerTRP_r16 v_DlPrs_AssistanceDataPerTRP;

var NCGI_r15 v_CellGlobalId := f_POS_Get_CellGlobalId(p_CellId);

var ARFCN_ValueNR v_FrequencySSB := f_NR_CellInfo_GetFrequencySSB(p_CellId);

var NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 v_PeriodicityAndOffset := f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId);

select(p_CellId){

case (nr_Cell1) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(0, 0, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 0));

}

case (nr_Cell2) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(1, 2, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 1));

}

case (nr_Cell3) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(2, 3, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 2));

}

case else {

FatalError (__FILE__, __LINE__, "Invalid Cell ID");

}

}

return v_DlPrs_AssistanceDataPerTRP;

}

<<SKIPPED CODE>>

## 2.3 Correction to LPP_Templates

| Function name | LPP _Templates |
| --- | --- |
| Reason for change | According to 3GPP 37.571-2 Table 8.4.1.6-3, the value of dl-PRS-SequenceID-r16 is 0 for all the TRPs. For intrafrequency DL PRS test cases, since the PRS sequences are on the same time, frequency and encoded with same sequence ID (0), a conformant UE may not decode PRS sequences from different Physical Cell IDs and DL PRS IDs. Therefore, a different dl-PRS-SequenceID-r16 value is needed per TRP |
| Summary of change | Modify templates cs_NR_DL_PRS_Info_Def, cs_NR_DL_PRS_ResourceSet and cs_NR_DL_PRS_Resource to include p_PrsSequenceId as an input parameter |
| TTCN module | LPP_Templates.ttcn |
| MCC160 Comment | Accepted, conditional to prose CR agreement at RAN5#97. |

Before

<<SKIPPED CODE>>

template (value) NR_DL_PRS_Info_r16 cs_NR_DL_PRS_Info_Def(NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 p_PeriodicityAndOffset) :=

{//Acc. Table 8.4.1.6-3: NR-DL-PRS-Info

nr_DL_PRS_ResourceSetList_r16 := {cs_NR_DL_PRS_ResourceSet(p_PeriodicityAndOffset)}

};

template (value) NR_DL_PRS_ResourceSet_r16 cs_NR_DL_PRS_ResourceSet(NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 p_PeriodicityAndOffset) :=

{

nr_DL_PRS_ResourceSetID_r16 := 0,

dl_PRS_Periodicity_and_ResourceSetSlotOffset_r16 := p_PeriodicityAndOffset,

dl_PRS_ResourceRepetitionFactor_r16 := omit,

dl_PRS_ResourceTimeGap_r16 := omit,

dl_PRS_NumSymbols_r16 := n2,

dl_PRS_MutingOption1_r16 := omit,

dl_PRS_MutingOption2_r16 := omit,

dl_PRS_ResourcePower_r16 := 0,

dl_PRS_ResourceList_r16 := {cs_NR_DL_PRS_Resource}

};

template (value) NR_DL_PRS_Resource_r16 cs_NR_DL_PRS_Resource :=

{

nr_DL_PRS_ResourceID_r16 := 0,

dl_PRS_SequenceID_r16 := 0,

dl_PRS_CombSizeN_AndReOffset_r16 := {

n2_r16 := 0

},

dl_PRS_ResourceSlotOffset_r16 := 0,

dl_PRS_ResourceSymbolOffset_r16 := 0,

dl_PRS_QCL_Info_r16 := omit

};

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

template (value) NR_DL_PRS_Info_r16 cs_NR_DL_PRS_Info_Def(NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 p_PeriodicityAndOffset, integer p_PrsSequenceId) :=

{//Acc. Table 8.4.1.6-3: NR-DL-PRS-Info

nr_DL_PRS_ResourceSetList_r16 := {cs_NR_DL_PRS_ResourceSet(p_PeriodicityAndOffset, p_PrsSequenceId)}

};

template (value) NR_DL_PRS_ResourceSet_r16 cs_NR_DL_PRS_ResourceSet(NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 p_PeriodicityAndOffset, integer p_PrsSequenceId) :=

{

nr_DL_PRS_ResourceSetID_r16 := 0,

dl_PRS_Periodicity_and_ResourceSetSlotOffset_r16 := p_PeriodicityAndOffset,

dl_PRS_ResourceRepetitionFactor_r16 := omit,

dl_PRS_ResourceTimeGap_r16 := omit,

dl_PRS_NumSymbols_r16 := n2,

dl_PRS_MutingOption1_r16 := omit,

dl_PRS_MutingOption2_r16 := omit,

dl_PRS_ResourcePower_r16 := 0,

dl_PRS_ResourceList_r16 := {cs_NR_DL_PRS_Resource(p_PrsSequenceId)}

};

template (value) NR_DL_PRS_Resource_r16 cs_NR_DL_PRS_Resource(integer p_PrsSequenceId) :=

{

nr_DL_PRS_ResourceID_r16 := 0,

dl_PRS_SequenceID_r16 := p_PrsSequenceId,

dl_PRS_CombSizeN_AndReOffset_r16 := {

n2_r16 := 0

},

dl_PRS_ResourceSlotOffset_r16 := 0,

dl_PRS_ResourceSymbolOffset_r16 := 0,

dl_PRS_QCL_Info_r16 := omit

};<<SKIPPED CODE>>

## 2.4 Correction to f_POS_NR_SimulateDlPrsPOSifSupported

| Function name | f_POS_NR_SimulateDlPrsPOSifSupported |
| --- | --- |
| Reason for change | According to 3GPP 37.571-2 Table 8.4.1.6-3, the value of dl-PRS-SequenceID-r16 is 0 for all the TRPs. For intrafrequency DL PRS test cases, since the PRS sequences are on the same time, frequency and encoded with same sequence ID (0), a conformant UE may not decode PRS sequences from different Physical Cell IDs and DL PRS IDs. Therefore, a different dl-PRS-SequenceID-r16 value is needed per TRP |
| Summary of change | Include the template change from section 2.3 and set the value of p_PrsSequenceId based on the NR Cell |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted, conditional to prose CR agreement at RAN5#97. |

Before

<<SKIPPED CODE>>

function f_POS_NR_SimulateDlPrsPOSifSupported(NR_CellId_Type p_CellId) runs on NR5GC_POS_PTC

{

var template (value) NR_DL_PRS_Info_r16 v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId));

if (pc_Multi_RTT or pc_UEA_DL_AoD or pc_UEA_DL_TDOA) {

//Configure DL-PRS

f_POS_NR_SS_DlPrsCellConfig(p_CellId, cas_NR_CellConfig_DlPrs_REQ(p_CellId,

cs_DlPrsFreqLayerConfig,

cs_DlPrsTRPConfig(v_DlPrsInfo)));

}

if (pc_UEA_DL_TDOA) {

f_POS_NR_CellConfig_DlPrs(nr_Cell2);

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_SimulateDlPrsPOSifSupported(NR_CellId_Type p_CellId) runs on NR5GC_POS_PTC

{

var template (value) NR_DL_PRS_Info_r16 v_DlPrsInfo;

select (p_CellId) {

case (nr_Cell1) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 0);

}

case (nr_Cell2) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 1);

}

case (nr_Cell3) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 2);

}

case else { FatalError(__FILE__, __LINE__, "Invalid NR CellId"); }

}

if (pc_Multi_RTT or pc_UEA_DL_AoD or pc_UEA_DL_TDOA) {

//Configure DL-PRS

f_POS_NR_SS_DlPrsCellConfig(p_CellId, cas_NR_CellConfig_DlPrs_REQ(p_CellId,

cs_DlPrsFreqLayerConfig,

cs_DlPrsTRPConfig(v_DlPrsInfo)));

}

if (pc_UEA_DL_TDOA) {

f_POS_NR_CellConfig_DlPrs(nr_Cell2);

}

}<<SKIPPED CODE>>

## 2.5 Correction to f_POS_NR_CellConfig_DlPrs

| Function name | f_POS_NR_CellConfig_DlPrs |
| --- | --- |
| Reason for change | According to 3GPP 37.571-2 Table 8.4.1.6-3, the value of dl-PRS-SequenceID-r16 is 0 for all the TRPs. For intrafrequency DL PRS test cases, since the PRS sequences are on the same time, frequency and encoded with same sequence ID (0), a conformant UE may not decode PRS sequences from different Physical Cell IDs and DL PRS IDs. Therefore, a different dl-PRS-SequenceID-r16 value is needed per TRP |
| Summary of change | Include the template change from section 2.3 and set the value of p_PrsSequenceId based on the NR Cell |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted, conditional to prose CR agreement at RAN5#97. |

Before

<<SKIPPED CODE>>

function f_POS_NR_CellConfig_DlPrs(NR_CellId_Type p_CellId) runs on NR5GC_POS_PTC

{

var template (value) NR_DL_PRS_Info_r16 v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId));

f_NR_CellConfig_Def(p_CellId);

//Configure DL-PRS

f_POS_NR_SS_DlPrsCellConfig(p_CellId, cas_NR_CellConfig_DlPrs_REQ(p_CellId,

cs_DlPrsFreqLayerConfig,

cs_DlPrsTRPConfig(v_DlPrsInfo)));

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CellConfig_DlPrs(NR_CellId_Type p_CellId) runs on NR5GC_POS_PTC

{

var template (value) NR_DL_PRS_Info_r16 v_DlPrsInfo;

select (p_CellId) {

case (nr_Cell1) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 0);

}

case (nr_Cell2) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 1);

}

case (nr_Cell3) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 2);

}

case else { FatalError(__FILE__, __LINE__, "Invalid NR CellId"); }

}

f_NR_CellConfig_Def(p_CellId);

//Configure DL-PRS

f_POS_NR_SS_DlPrsCellConfig(p_CellId, cas_NR_CellConfig_DlPrs_REQ(p_CellId,

cs_DlPrsFreqLayerConfig,

cs_DlPrsTRPConfig(v_DlPrsInfo)));

}

<<SKIPPED CODE>>

## 2.6 Correction to f_POS_NR_Set_DlPrsAssistanceDataPerTRP

| Function name | f_POS_NR_Set_DlPrsAssistanceDataPerTRP |
| --- | --- |
| Reason for change | According to 3GPP 37.571-2 Table 8.4.1.6-3, the value of dl-PRS-SequenceID-r16 is 0 for all the TRPs. For intrafrequency DL PRS test cases, since the PRS sequences are on the same time, frequency and encoded with same sequence ID (0), a conformant UE may not decode PRS sequences from different Physical Cell IDs and DL PRS IDs. Therefore, a different dl-PRS-SequenceID-r16 value is needed per TRP |
| Summary of change | Include the template change from section 2.3 and set the value of p_PrsSequenceId based on the NR Cell |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted, conditional to prose CR agreement at RAN5#97. |

Before

<<SKIPPED CODE>>

function f_POS_NR_Set_DlPrsAssistanceDataPerTRP(NR_CellId_Type p_CellId) runs on NR_BASE_PTC return template (value) NR_DL_PRS_AssistanceDataPerTRP_r16

{

var template (value) NR_DL_PRS_AssistanceDataPerTRP_r16 v_DlPrs_AssistanceDataPerTRP;

var NCGI_r15 v_CellGlobalId := f_POS_Get_CellGlobalId(p_CellId);

var ARFCN_ValueNR v_FrequencySSB := f_NR_CellInfo_GetFrequencySSB(p_CellId);

var NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 v_PeriodicityAndOffset := f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId);

select(p_CellId){

case (nr_Cell1) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(0, 1, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset));

}

case (nr_Cell2) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(1, 2, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset));

}

case (nr_Cell3) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(2, 3, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset));

}

case else {

FatalError (__FILE__, __LINE__, "Invalid Cell ID");

}

}

return v_DlPrs_AssistanceDataPerTRP;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_Set_DlPrsAssistanceDataPerTRP(NR_CellId_Type p_CellId) runs on NR_BASE_PTC return template (value) NR_DL_PRS_AssistanceDataPerTRP_r16

{

var template (value) NR_DL_PRS_AssistanceDataPerTRP_r16 v_DlPrs_AssistanceDataPerTRP;

var NCGI_r15 v_CellGlobalId := f_POS_Get_CellGlobalId(p_CellId);

var ARFCN_ValueNR v_FrequencySSB := f_NR_CellInfo_GetFrequencySSB(p_CellId);

var NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 v_PeriodicityAndOffset := f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId);

select(p_CellId){

case (nr_Cell1) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(0, 1, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 0));

}

case (nr_Cell2) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(1, 2, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 1));

}

case (nr_Cell3) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(2, 3, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 2));

}

case else {

FatalError (__FILE__, __LINE__, "Invalid Cell ID");

}

}

return v_DlPrs_AssistanceDataPerTRP;

}

<<SKIPPED CODE>>

## 2.7 Correction to f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList

| Function name | f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList |
| --- | --- |
| Reason for change | Current implementation for f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList does not take into account that following parameters are optional (from 3GPP 37.355): maxNumOfDL-PRS-ResProcessedPerSlot-r16 SEQUENCE { scs15-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs30-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs60-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs120-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, Current implementation requires UE must report all 4 SCS combinations. This leads into test case failures when testing FR1, since UE will not report SCS 120 for FR1 bands. |
| Summary of change | In the final if statement, report at least one of the 4 SCS combinations, replacing and by or |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Already implemented as part of R5s2201041. |

Before

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList (NR_DL_PRS_ProcessingCapability_r16.prs_ProcessingCapabilityBandList_r16 p_NrDlPrsProcessingCapabilityBandList) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer v_Index;

var boolean v_Found_SupportedBandwidthPRS_FR1 := false;

var boolean v_Found_SupportedBandwidthPRS_FR2 := false;

var boolean v_Found_Dl_PRS_BufferType := false;

var boolean v_Found_DurationOfPRS_ProcessingSymbols := false;

var boolean v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := false;

for (v_Index := 0; v_Index < lengthof(p_NrDlPrsProcessingCapabilityBandList); v_Index:=v_Index+1) {

fl_CheckNRBandCapability(p_NrDlPrsProcessingCapabilityBandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS

if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1 == pc_supportedBandwidthPRS_FR1)) {

v_Found_SupportedBandwidthPRS_FR1 := true;

}

if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2 == pc_supportedBandwidthPRS_FR2)) {

v_Found_SupportedBandwidthPRS_FR2 := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_BufferType_r16 == pc_dl_PRS_BufferType) {

v_Found_Dl_PRS_BufferType := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSysmbols_r16 == pc_durationOfPRS_ProcessingSymbols) {

v_Found_DurationOfPRS_ProcessingSymbols := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSymbolsInEveryTms_r16 == pc_PRS_ProcessingSymbolsInEveryTms) {

v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS15)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS30)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS60)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS120)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := true;

}

}

if (not ((v_Found_SupportedBandwidthPRS_FR1 or v_Found_SupportedBandwidthPRS_FR2) and v_Found_Dl_PRS_BufferType and v_Found_DurationOfPRS_ProcessingSymbols and

v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms and v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 and v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 and

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 and v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for Prs_ProcessingCapabilityBandList");

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList (NR_DL_PRS_ProcessingCapability_r16.prs_ProcessingCapabilityBandList_r16 p_NrDlPrsProcessingCapabilityBandList) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer v_Index;

var boolean v_Found_SupportedBandwidthPRS_FR1 := false;

var boolean v_Found_SupportedBandwidthPRS_FR2 := false;

var boolean v_Found_Dl_PRS_BufferType := false;

var boolean v_Found_DurationOfPRS_ProcessingSymbols := false;

var boolean v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := false;

for (v_Index := 0; v_Index < lengthof(p_NrDlPrsProcessingCapabilityBandList); v_Index:=v_Index+1) {

fl_CheckNRBandCapability(p_NrDlPrsProcessingCapabilityBandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS

if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1 == pc_supportedBandwidthPRS_FR1)) {

v_Found_SupportedBandwidthPRS_FR1 := true;

}

if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2 == pc_supportedBandwidthPRS_FR2)) {

v_Found_SupportedBandwidthPRS_FR2 := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_BufferType_r16 == pc_dl_PRS_BufferType) {

v_Found_Dl_PRS_BufferType := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSysmbols_r16 == pc_durationOfPRS_ProcessingSymbols) {

v_Found_DurationOfPRS_ProcessingSymbols := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSymbolsInEveryTms_r16 == pc_PRS_ProcessingSymbolsInEveryTms) {

v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS15)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS30)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS60)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS120)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := true;

}

}

if (not ((v_Found_SupportedBandwidthPRS_FR1 or v_Found_SupportedBandwidthPRS_FR2) and v_Found_Dl_PRS_BufferType and v_Found_DurationOfPRS_ProcessingSymbols and

v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms and (v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 or

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120))) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for Prs_ProcessingCapabilityBandList");

}

}

<<SKIPPED CODE>>

# Branches executed

## This POS TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n71” using NIA1 Integrity and NEA1 ciphering algorithms

# Execution Log Files

## Mediatek MT6879 UE

The Mediatek MT6879 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_9_3_1_2_19s_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221044: Supporting information for addition of POS testcase 9.3.1.2.19s in FR1 |
| --- | --- |