3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221057

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2726 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR5GC NR-DC test case 8.2.3.14.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 3 “reestablishPdcp” should be set “true” in RadioBearerConfig, as per 38.523-1 table 8.2.3.14.2.3.3-2A, this is currently omitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set reestablishPdcp to “true” at step 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.14.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish() |
| --- | --- |
| Reason for change | At step 3 “reestablishPdcp” should be set “true” in RadioBearerConfig, as per 38.523-1 table 8.2.3.14.2.3.3-2A |
| Summary of change | Pass new paramter for pdcpReestablish=”true” to template cs_NR_PDCP_Config_Split Note : This function currently only used for 8.2.3.14.2. step 2. |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish( NR_CellId_Type p_MCG_PCellId, NR_CellId_Type p_Source_PSCellId, NR_CellId_Type p_Target_PSCellId, boolean p_RBConfig_KeyChange := true, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template(omit) SK_Counter p_SK_Counter := omit ) runs on NR5GC_PTC { ………………….. v_CellGroupConfigSCG.rlc_BearerToAddModList[0].reestablishRLC := true_; v_RadioBearerConfig2 := cs_NRDRB_SplitBearer(omit, v_SCG_DRB_Id, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, omit, tsc_NR_CellGroupId_SCG), secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( v_CellGroupConfigSCG, v_CellGroupConfigMCG, v_RadioBearerConfig2, -, p_SK_Counter ); // Add SCG DRB configured in proxy (split SCG DRB) v_SS_MCG_RadioBearerList[lengthof(v_SS_MCG_RadioBearerList)] := cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_Proxy(cs_RlcBearerRouting_NR(p_Target_PSCellId)), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id))); ……………………….. }

After Change

function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish( NR_CellId_Type p_MCG_PCellId, NR_CellId_Type p_Source_PSCellId, NR_CellId_Type p_Target_PSCellId, boolean p_RBConfig_KeyChange := true, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template(omit) SK_Counter p_SK_Counter := omit ) runs on NR5GC_PTC { …………………………… v_CellGroupConfigSCG.rlc_BearerToAddModList[0].reestablishRLC := true_; v_RadioBearerConfig2 := cs_NRDRB_SplitBearer(omit, v_SCG_DRB_Id, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, omit, tsc_NR_CellGroupId_SCG), secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get(), true_); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( v_CellGroupConfigSCG, v_CellGroupConfigMCG, v_RadioBearerConfig2, -, p_SK_Counter ); // Add SCG DRB configured in proxy (split SCG DRB) v_SS_MCG_RadioBearerList[lengthof(v_SS_MCG_RadioBearerList)] := cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_Proxy(cs_RlcBearerRouting_NR(p_Target_PSCellId)), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id))); …………………….. }

## Change 2

| Template name | Template cs_NRDRB_SplitBearer |
| --- | --- |
| Reason for change | At step 3 “reestablishPdcp” should be set “true” in RadioBearerConfig, as per 38.523-1 table 8.2.3.14.2.3.3-2A |
| Summary of change | Take new paramter for pdcpReestablish and pass to cs_NR_DRB_ToAddMod |
| TTCN module | NR_RRC_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change

template (value) RadioBearerConfig cs_NRDRB_SplitBearer(template (omit) DRB_ToAddMod.cnAssociation p_CN_Association, DRB_Identity p_DRBId, template (omit) PDCP_Config p_PDCP_Config, NR_KeyToUse_Type p_KeyToUse := secondary, CipheringAlgorithm p_CipheringAlgorithm, template (omit) IntegrityProtAlgorithm p_IntegrityProtAlgorithm ) := /* @status APPROVED (NR5GC) */ // @sic R5s210888: added p_PDCP_Config sic@ cs_38508_RadioBearerConfigDef(-, { cs_NR_DRB_ToAddMod(p_CN_Association,p_DRBId, -, -, p_PDCP_Config)}, //NESPCT-11897 -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(p_CipheringAlgorithm, p_IntegrityProtAlgorithm), p_KeyToUse));

After Change

template (value) RadioBearerConfig cs_NRDRB_SplitBearer(template (omit) DRB_ToAddMod.cnAssociation p_CN_Association, DRB_Identity p_DRBId, template (omit) PDCP_Config p_PDCP_Config, NR_KeyToUse_Type p_KeyToUse := secondary, CipheringAlgorithm p_CipheringAlgorithm, template (omit) IntegrityProtAlgorithm p_IntegrityProtAlgorithm, template (omit) DRB_ToAddMod.reestablishPDCP p_ReestablishPDCP := omit ) := /* @status APPROVED (NR5GC) */ // @sic R5s210888: added p_PDCP_Config sic@ cs_38508_RadioBearerConfigDef(-, { cs_NR_DRB_ToAddMod(p_CN_Association,p_DRBId, p_ReestablishPDCP, -, p_PDCP_Config)}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(p_CipheringAlgorithm, p_IntegrityProtAlgorithm), p_KeyToUse));