3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221066

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2732 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ENDC testcase 8.2.4.1.1.2 in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add ENDC test case 8.2.4.1.1.2 in FR2 to the IWD_22wk23 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to ENDC test case 8.2.4.1.1.2 required for agreement. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.1.1.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	f_NR_GetCA_IntraBandFrequency	3

3	Branches executed	4

4	Execution Log Files	4

4.1	Qualcomm  Snapdragon X70 5G Modem-RF System	4

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of ENDC test case 8.2.4.1.1.2 in FR2 which is part of the ENDC test suite in iWD_TTCN3-B2020-09_D22wk23.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.2.4.1.1.2.ENDC

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Qualcomm  Snapdragon X70 5G Modem-RF System

Verification Status:	PASS

# Corrections required

## f_NR_GetCA_IntraBandFrequency

| Function name | f_NR_GetCA_IntraBandFrequency |
| --- | --- |
| Reason for change | According to TS 38.508-1, for CA_n261(XA), Table 4.3.1.2.1.5-2 needs to be used. In this Table for CBW = 100, the carrierBandwidth is 66 as well as the rest of the parameters are specific to this CBW. However, TTCN has incorrectly taken the carrierBandwidth for CBW = 200 as well as coding wrong values for the rest of the parameters. |
| Summary of change | Implemented the values in Table 4.3.1.2.1.5-2 for CBW = 100 |
| TTCN module | NR_CA_FrequencyFunctions |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_GetCA_IntraBandFrequency ( NR_CA_BandCombination_Type p_NR_CA_BandCombination_Type ) return template(value) NR_Frequencyf1tof4_Type { <<SKIPPED CODE>> case( CA_n261_2A ) { //@sic R5-206306, R5-206861 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2071387 , v_FreqBandIndicatorNR , 2070875 , 0 , kHz120 , 132 ), 100 , kHz120 , 8 , 1 , 22 , 14 , 8 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz120 , 132 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2083771 , v_FreqBandIndicatorNR , 2077325 , 504 , kHz120 , 132 ), 100 , kHz120 , 2 , 1 , 517 , 5 , 8 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz120 , 132 , v_P_Max), 100 ); } case else { FatalError (__FILE__, __LINE__, "Invalid NR-SA band combination") } } return v_NR_Frequencyf1tof4; }

After Change:

function f_NR_GetCA_IntraBandFrequency ( NR_CA_BandCombination_Type p_NR_CA_BandCombination_Type ) return template(value) NR_Frequencyf1tof4_Type { <<SKIPPED CODE>> //WA# v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2071387 , v_FreqBandIndicatorNR , 2070875 , 0 , kHz120 , 66 ), 100 , kHz120 , 4 , 1 , 22 , 7 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz120 , 66 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2083771 , v_FreqBandIndicatorNR , 2071277 , 504 , kHz120 , 66 ), 100 , kHz120 , 7 , 1 , 1020 , 2 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz120 , 66 , v_P_Max), 100 ); } case else { FatalError (__FILE__, __LINE__, "Invalid NR-SA band combination") } } return v_NR_Frequencyf1tof4; }

# Branches executed

## This ENDC TC was executed in Band combination “CA_n261(2A)” using NEA2/NIA2  algorithms”.

# Execution Log Files

## Qualcomm  Snapdragon X70 5G Modem-RF System

The Qualcomm Snapdragon X70 5G Modem-RF System UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_8_2_4_1_1_2_ENDC_FR2_QC.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221067: Supporting information for addition of ENDC TC 8.2.4.1.1.2 in FR2. |
| --- | --- |