3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221095

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0862 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_IMS_CC_StartCall |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest, TEI15_Test |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Due to the implementation of R5-225362 some reasoning has been added to coordination messages in some cases. Test cases 7.16 and 7.17 miss the reasoning for MT video call provoking a mismatch once the coordination message for video call is being established, failing the test case. This can be fixed just adding the proper reasoning to coordination message as follows: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added “Start MT Video IMS” to expected coordination message to establish a MR Video call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.16.NR5GC and 7.17.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Correction to function f_IMS_CC_StartCall	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_IMS_CC_StartCall

| Function name | f_IMS_CC_StartCall |
| --- | --- |
| Reason for change | Due to the implementation of R5-225362 some reasoning has been added to coordination messages in some cases. Test cases 7.16 and 7.17 miss the reasoning for MT video call provoking a mismatch and failing the establishment of the call. |
| Summary of change | Added “Start MT Video IMS” to expected coordination message to establish a MR Video call. |
| TTCN module | IMS\Common\IMS_CC_CommonFunctions.ttcn |
| MCC160 Comment | This will result in an error in IMS_Testsuite_EUTRA. Alternative solution implemented: Removed this reasoning from f_NR5GC_IPCAN_MT_VideoCall |

Before Change:

<<SKIPPED CODE>> function f_IMS_CC_StartCall(IMS_TestProcedure_Type p_TestProcedure, charstring p_SpeechAndVideoCallUri := px_IMS_CalleeUri, CallType_Type p_CallType := normalCall) runs on IMS_PTC { /* @sic new parameters p_SpeechAndVideoCallUri and p_CallType to support conference calls sic@ */ // @sic R5s160855 removed return type sic@ f_IMS_IPCAN_StartProcedure(IPCAN, p_TestProcedure); select (p_TestProcedure) { case (IPCAN_MO_SpeechCall) { select (p_CallType) { case (normalCall) { f_UT_RequestIMSCall(MMI, p_SpeechAndVideoCallUri); // @sic R5s130333, R5-134070 sic@ } case (normalCall_ReturnId){ f_UT_RequestIMSCall(MMI, p_SpeechAndVideoCallUri, CNF_REQUIRED); // @sic R5s130333, R5-134070, R5s160855 sic@ } case (conferenceCall) { f_UT_InitiateConferenceCall(MMI); // @sic R5-155177 sic@ } case (normalCallOIR) { // @sic R5-160975 sic@ f_UT_RequestIMSCallOIR(MMI, p_SpeechAndVideoCallUri); } } } case (IPCAN_MO_VideoCall) { f_UT_RequestIMSVideoCall(MMI, p_SpeechAndVideoCallUri); // @sic R5-134070 sic@ } case (IPCAN_MT_SpeechCall, IPCAN_MT_VideoCall) { f_IMS_IPCAN_WaitForTrigger(IPCAN); // wait for RRC connection to be established } case (IPCAN_EmergencyCall_NormalService, IPCAN_EmergencyCall_OptionalPDNRelease) { // @sic R5s150091 sic@ @sic R5s180023: IPCAN_EmergencyCall_OptionalPDNRelease sic@ f_UT_RequestIMSEmergencyCall(MMI, px_EmergencyCallNumber); // @sic R5-134070 sic@ } case else { FatalError(__FILE__, __LINE__, ""); } } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_CC_StartCall(IMS_TestProcedure_Type p_TestProcedure, charstring p_SpeechAndVideoCallUri := px_IMS_CalleeUri, CallType_Type p_CallType := normalCall) runs on IMS_PTC { /* @sic new parameters p_SpeechAndVideoCallUri and p_CallType to support conference calls sic@ */ // @sic R5s160855 removed return type sic@ f_IMS_IPCAN_StartProcedure(IPCAN, p_TestProcedure); select (p_TestProcedure) { case (IPCAN_MO_SpeechCall) { select (p_CallType) { case (normalCall) { f_UT_RequestIMSCall(MMI, p_SpeechAndVideoCallUri); // @sic R5s130333, R5-134070 sic@ } case (normalCall_ReturnId){ f_UT_RequestIMSCall(MMI, p_SpeechAndVideoCallUri, CNF_REQUIRED); // @sic R5s130333, R5-134070, R5s160855 sic@ } case (conferenceCall) { f_UT_InitiateConferenceCall(MMI); // @sic R5-155177 sic@ } case (normalCallOIR) { // @sic R5-160975 sic@ f_UT_RequestIMSCallOIR(MMI, p_SpeechAndVideoCallUri); } } } case (IPCAN_MO_VideoCall) { f_UT_RequestIMSVideoCall(MMI, p_SpeechAndVideoCallUri); // @sic R5-134070 sic@ } case (IPCAN_MT_SpeechCall/*, IPCAN_MT_VideoCall*/) { f_IMS_IPCAN_WaitForTrigger(IPCAN); // wait for RRC connection to be established } case (IPCAN_MT_VideoCall) { f_IMS_IPCAN_WaitForTrigger(IPCAN,"Start MT Video IMS"); } case (IPCAN_EmergencyCall_NormalService, IPCAN_EmergencyCall_OptionalPDNRelease) { // @sic R5s150091 sic@ @sic R5s180023: IPCAN_EmergencyCall_OptionalPDNRelease sic@ f_UT_RequestIMSEmergencyCall(MMI, px_EmergencyCallNumber); // @sic R5-134070 sic@ } case else { FatalError(__FILE__, __LINE__, ""); } } } <<SKIPPED CODE>>

#### MCC160 Implementation

function f_NR5GC_IPCAN_MT_VideoCall(IMS_IPCAN_CO_ORD_PORT p_Port,

NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type

{

var TriggerResult_Type v_Result := NORMAL;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo;

f_NR_Paging_Def(p_NR_CellId, RRC_IDLE);

v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B); // @sic R5s220235 sic@

f_IMS_IPCAN_SendCoOrdMsg(p_Port, cms_IMS_IPCAN_Trigger()); // @sic R5-225362 sic@

v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port);  // wait until IMS PTC is ready to configure video DRB

if (v_Result == NORMAL) {                               // => procedure shall not be aborted

f_NR5GC_ModifyPDUSession_AddVideo(p_NR_CellId, valueof(v_GSM_MobilityInfo));

}

f_IMS_IPCAN_SendCoOrdMsg(p_Port);

return v_Result;

}