3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221096

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2750 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC test case 8.2.7.2.1 in FR1+FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 8.2.7.2.1 to the IWD_22wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.2.7.2.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function fl_TC_8_2_7_2_1_TestBody()	4

2.2	Function f_NR5GC_GetRadioBearerList ()	8

3	Branches executed	10

4	Execution Log Files	10

4.1	Qualcomm Snapdragon X70 5G Modem-RF System	10

5	References	11

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.2.7.2.1  which is part of the NR5GC test suite in iwd-TTCN3-B2020-09_D22wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

  - Verification Test Summary

Test Case: 	8.2.7.2.1

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System

Verification Status:	PASS

- Corrections required

  - Function fl_TC_8_2_7_2_1_TestBody()

| Function name | fl_TC_8_2_7_2_1_TestBody () |
| --- | --- |
| Reason for change | - In the current Implementation of the test case , at step 1 , f_NR_RRC_InactiveDef f_NR_SS_SRBs_DRBs_Release , release SRB1,2 , and NR-DC Context can not be maintained by SS in the absence of SRB1,2. Hence it is proposed to switch the SCG DRB release before calling f_NR_RRC_InactiveDef - Ul grant is stopped in the function f_NR_RRC_InactiveDef , UL grant has to be started again before handling resume complete message - v_RadioBearerConfig has to be filled properly containing SRB2 and MCG DRB with reestablish Pdcp set ( this is sent along with resume Peer Message at Step 4 ) |
| Summary of change | - SCG DRB release is done before calling f_NR_RRC_InactiveDef - Ul grant is started back before handling resume complete message - In the resume DL peer Message , v_RadioBearerConfig has to be filled properly containing SRB2 and MCG DRB with reestablish Pdcp set |
| TTCN module | RRC_Resume_NRDC_NR5GC.ttcn |
| MCC160 Comment | 1.Not needed, there is not such inter-dependency specified in the NR test model / ASPs. 2.Accepted 3.Accepted Please see MCC160 final implementation below. |

Before Change:

function fl_TC_8_2_7_2_1_TestBody() runs on NR5GC_PTC { var integer v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(nr_Cell10); var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) SDAP_Config v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG := f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG)); var template (value) DRB_ToAddMod v_DRBToAddMod_SCG; var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(false, -, true); var template (value) RadioBearerConfig v_RadioBearerConfig2; var template (value) CellGroupConfig v_CellGroupConfigMCG := f_NR_GetSCGConfigResume(nr_Cell1); var template(omit) octetstring v_EncodedSCG_MCG := bit2oct(encvalue(v_CellGroupConfigMCG)); var template (value) CellGroupConfig v_CellGroupConfigSCG; var template (value) NR_RadioBearerList_Type v_SS_ConfigMCG := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList(), true); var template (value) NR_RadioBearerList_Type v_SS_ConfigSCG; var template (value) RRCResume_v1560_IEs v_RRCResume_v1560Ext; //@siclog "Step 1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //Release SCG DRB in SS f_NR_SS_CommonRadioBearerConfig(nr_Cell10, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_SCG_DRB_Id))}); //@siclog "Step 2" siclog@ //The SS waits for 5 sec and transmits a Paging message including a matched identity (correct fullI-RNTI). f_Delay(5.0); f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 3" siclog@ //UE transmits an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1); //@siclog "Step 4" siclog@ //The SS transmits an RRCResume message. // Initialise variablmes for SCG v_DRBToAddMod_SCG := cs_NR_DRB_ToAddMod(omit, v_SCG_DRB_Id, true_, -, omit); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, {v_DRBToAddMod_SCG}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get_SCG(), f_NR_AS_IntegrityAlgorithm_Get_SCG()), secondary)); v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(nr_Cell10, v_SCG_DRB_Id); v_RRCResume_v1560Ext := f_NRDC_BuildRRCResumeSCG_v1560(v_CellGroupConfigSCG, v_RadioBearerConfig2, -, 0, true_); v_SS_ConfigSCG := { cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsDRB), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)), v_SDAP_ConfigurationSCG) }; f_ReconfigureSS_DRBs_NRDC( nr_Cell1, nr_Cell10, -, v_SS_ConfigMCG, v_SS_ConfigSCG, MCG_SCG_SecondaryKeys ); f_NR_SS_SystemIndCtrlConfig(nr_Cell10, cds_NR_SystemIndCtrl_RachPreamble(enable)); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCResume(-, v_EncodedSCG_MCG, v_RadioBearerConfig, -, v_RRCResume_v1560Ext))); interleave { //@siclog "Step 5" siclog@ //The UE transmits an RRCResumeComplete message. [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete(-, cr_RRCResumeComplete_v1610(?)))) {} [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell10, -, ?)) {} } f_NR_SS_SystemIndCtrlConfig(nr_Cell10, cds_NR_SystemIndCtrl_RachPreamble(disable)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } }

After Change:

function fl_TC_8_2_7_2_1_TestBody() runs on NR5GC_PTC { var integer v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(nr_Cell10); var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) SDAP_Config v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG := f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG)); var template (value) DRB_ToAddMod v_DRBToAddMod_SCG; var template (value) RadioBearerConfig v_RadioBearerConfig /*:= f_NR5GC_GetExisting_RadioBearerConfig(true, -, true)*/; //WA#WI=998159 var template (value) RadioBearerConfig v_RadioBearerConfig2; var template (value) CellGroupConfig v_CellGroupConfigMCG := f_NR_GetSCGConfigResume(nr_Cell1); var template(omit) octetstring v_EncodedSCG_MCG := bit2oct(encvalue(v_CellGroupConfigMCG)); var template (value) CellGroupConfig v_CellGroupConfigSCG; var template (value) NR_RadioBearerList_Type v_SS_ConfigMCG := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList(), true); var template (value) NR_RadioBearerList_Type v_SS_ConfigSCG; var template (value) RRCResume_v1560_IEs v_RRCResume_v1560Ext; var NR_DRBInfoList_Type v_ExistingDRBs; //WA#WI=998159 var integer i,j; //WA#WI=998159 var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit; //WA#WI=998159 //@siclog "Step 1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. f_NR_SS_CommonRadioBearerConfig(nr_Cell10, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_SCG_DRB_Id))}); //WA#WI=998159 f_NR_RRC_InactiveDef(nr_Cell1); //Release SCG DRB in SS //f_NR_SS_CommonRadioBearerConfig(nr_Cell10, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_SCG_DRB_Id))}); //WA#WI=998159 //@siclog "Step 2" siclog@ //The SS waits for 5 sec and transmits a Paging message including a matched identity (correct fullI-RNTI). f_Delay(5.0); f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 3" siclog@ //UE transmits an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1); f_NR_ULGrantConfiguration_Start(nr_Cell1); //WA#WI=988940 //@siclog "Step 4" siclog@ //The SS transmits an RRCResume message. // Initialise variablmes for SCG v_DRBToAddMod_SCG := cs_NR_DRB_ToAddMod(omit, v_SCG_DRB_Id, true_, -, omit); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, {v_DRBToAddMod_SCG}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get_SCG(), f_NR_AS_IntegrityAlgorithm_Get_SCG()), secondary)); v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(nr_Cell10, v_SCG_DRB_Id); v_RRCResume_v1560Ext := f_NRDC_BuildRRCResumeSCG_v1560(v_CellGroupConfigSCG, v_RadioBearerConfig2, -, 0, true_); v_SS_ConfigSCG := { cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsDRB), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)), v_SDAP_ConfigurationSCG) }; f_ReconfigureSS_DRBs_NRDC( nr_Cell1, nr_Cell10, -, v_SS_ConfigMCG, v_SS_ConfigSCG, MCG_SCG_SecondaryKeys ); f_NR_SS_SystemIndCtrlConfig(nr_Cell10, cds_NR_SystemIndCtrl_RachPreamble(enable)); v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); j := 0; //WA#WI=988940 for (i:= 0; i < lengthof(v_ExistingDRBs); i := i+1) { if(v_ExistingDRBs[i].DC_SCG == false) //WA#WI=988940 { v_DRB_ToAddModList[j] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); j := j+1; //WA#WI=988940 } } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(2)}, v_DRB_ToAddModList); //WA#WI=988940 SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCResume(-, v_EncodedSCG_MCG, v_RadioBearerConfig, -, v_RRCResume_v1560Ext))); interleave { //@siclog "Step 5" siclog@ //The UE transmits an RRCResumeComplete message. [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete(-, cr_RRCResumeComplete_v1610(?)))) {} [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell10, -, ?)) {} } f_NR_SS_SystemIndCtrlConfig(nr_Cell10, cds_NR_SystemIndCtrl_RachPreamble(disable)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } }

  - Function f_NR5GC_GetRadioBearerList ()

| Function name | f_NR5GC_GetRadioBearerList() |
| --- | --- |
| Reason for change | In the current Implementation , the implementation of p_ExcludeSCG_DRB is not taking care of all the possibilities Current Implementation is having the condition if (not(p_ExcludeSCG_DRB and valueof(p_DrbInfoList[i].DC_SCG) == false)) before filling the radiobearerList. In case of SCG bearer , If not (p_ExcludeSCG_DRB = 1 and valueof(p_DrbInfoList[i].DC_SCG) == false)), then radio bearer List will be filled with SCG DRB entry though we configured excludeScg_Drb as True ( this is not correct ) And in case of MCG bearer If not (p_ExcludeSCG_DRB = 1 and valueof(p_DrbInfoList[i].DC_SCG) == false) We will be excluding the MCG DRB entry in v_RadioBearerList |
| Summary of change | To address the above mentioned Issue the condition is changed before filling the v_RadioBearerList The proposed condition will do below things - If exclude SCG DRB is true and the bearer detected is having DC_SCG set to False then it will be filled in v_RadioBearerList - In case of p_ExcludeSCG_DRB not being set , v_RadioBearerList will be filled with all entries |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted in principle, but changed it to: if (not(p_ExcludeSCG_DRB and valueof(p_DrbInfoList[i].DC_SCG))) |

Before Change:

function f_NR5GC_GetRadioBearerList(template (value) NR_DRBInfoList_Type p_DrbInfoList, boolean p_ExcludeSCG_DRB := false ) return template (value) NR_RadioBearerList_Type { var template (value) NR_RadioBearerList_Type v_RadioBearerList; var integer i; var integer k := 0; // @sic R5s211604 sic@ for (i:= 0; i < lengthof(p_DrbInfoList); i:=i+1) { if (not(p_ExcludeSCG_DRB and valueof(p_DrbInfoList[i].DC_SCG) == false)) { v_RadioBearerList[k] := p_DrbInfoList[i].SS_Config; k:=k+1; } } return v_RadioBearerList; }

After Change:

function f_NR5GC_GetRadioBearerList(template (value) NR_DRBInfoList_Type p_DrbInfoList, boolean p_ExcludeSCG_DRB := false ) return template (value) NR_RadioBearerList_Type { var template (value) NR_RadioBearerList_Type v_RadioBearerList; var integer i; var integer k := 0; // @sic R5s211604 sic@ for (i:= 0; i < lengthof(p_DrbInfoList); i:=i+1) { if( (p_ExcludeSCG_DRB and valueof(p_DrbInfoList[i].DC_SCG) == false) or (not p_ExcludeSCG_DRB)) //WA#WI=998159 //if (not(p_ExcludeSCG_DRB and valueof(p_DrbInfoList[i].DC_SCG) == false)) //WA#WI=998159 { v_RadioBearerList[k] := p_DrbInfoList[i].SS_Config; k:=k+1; } } return v_RadioBearerList; }

MCC160 final implementation

These changes are additional comment received from Keysight and accepted in principle.

Please see MCC160 final implementation below.

Initialize the SRB+DRBs configurations for resume before paging the UE at step 2 to avoid SS may delay sending of RRCResume at step 4 and cause T319 to expire at the UE.

import from CommonAspDefs all;

function fl_TC_8_2_7_2_1_TestBody() runs on NR5GC_PTC

{

var integer v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG();

var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(nr_Cell10);

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN);

var template (value) SDAP_Config v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5});

var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG := f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG));

var template (value) DRB_ToAddMod v_DRBToAddMod_SCG;

var template (value) RadioBearerConfig v_RadioBearerConfig; // @sic R5s221096 sic@

var template (value) RadioBearerConfig v_RadioBearerConfig2;

var template (value) CellGroupConfig v_CellGroupConfigMCG := f_NR_GetSCGConfigResume(nr_Cell1);

var template(omit) octetstring v_EncodedSCG_MCG := bit2oct(encvalue(v_CellGroupConfigMCG));

var template (value) CellGroupConfig v_CellGroupConfigSCG;

var template (value) NR_RadioBearerList_Type v_SS_ConfigMCG := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList(), true);

var template (value) NR_RadioBearerList_Type v_SS_ConfigSCG;

var template (value) RRCResume_v1560_IEs v_RRCResume_v1560Ext;

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); // @sic R5s221096 sic@

var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit; // @sic R5s221096 sic@

var integer i; // @sic R5s221096 sic@

var integer j := 0; // @sic R5s221096 sic@

//@siclog "Step 1" siclog@

//The SS transmits an RRCRelease message with suspendConfig.

f_NR_RRC_InactiveDef(nr_Cell1);

//Release SCG DRB in SS

f_NR_SS_CommonRadioBearerConfig(nr_Cell10, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_SCG_DRB_Id))});

// Prepare SRB+DRBs configurations for step 4

// Configure RadioBearerConfig with condition RESUME

for (i:= 0; i < lengthof(v_ExistingDRBs); i :=i+1) { // @sic R5s221096 sic@

if(v_ExistingDRBs[i].DC_SCG == false){

v_DRB_ToAddModList[j] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit);

j := j+1;

}

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList);

// Initialise variablmes for SCG

v_DRBToAddMod_SCG := cs_NR_DRB_ToAddMod(omit, v_SCG_DRB_Id, true_, -, omit);

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, {v_DRBToAddMod_SCG}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get_SCG(), f_NR_AS_IntegrityAlgorithm_Get_SCG()), secondary));

v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(nr_Cell10, v_SCG_DRB_Id);

v_RRCResume_v1560Ext := f_NRDC_BuildRRCResumeSCG_v1560(v_CellGroupConfigSCG, v_RadioBearerConfig2, -, 0, true_);

v_SS_ConfigSCG := { cs_NR_SS_DRB_Config(v_SCG_DRB_Id,

cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsDRB),

cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)),

v_SDAP_ConfigurationSCG) };

//@siclog "Step 2" siclog@

//The SS waits for 5 sec and transmits a Paging message including a matched identity (correct fullI-RNTI).

f_Delay(5.0);

f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE);

//@siclog "Step 3" siclog@

//UE transmits an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value.

f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1);

//@siclog "Step 4" siclog@

//The SS transmits an RRCResume message.

// Switch on UL grant assignments

f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s221096 sic@

f_ReconfigureSS_DRBs_NRDC( nr_Cell1, nr_Cell10, -, v_SS_ConfigMCG, v_SS_ConfigSCG, MCG_SCG_SecondaryKeys );

f_NR_SS_SystemIndCtrlConfig(nr_Cell10, cds_NR_SystemIndCtrl_RachPreamble(enable));

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCResume(-, v_EncodedSCG_MCG, v_RadioBearerConfig, -, v_RRCResume_v1560Ext)));

interleave {

//@siclog "Step 5" siclog@

//The UE transmits an RRCResumeComplete message.

[] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete(-, cr_RRCResumeComplete_v1610(?)))) {}

[] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell10, -, ?)) {}

}

f_NR_SS_SystemIndCtrlConfig(nr_Cell10, cds_NR_SystemIndCtrl_RachPreamble(disable));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

}

TTCN module: TestcaseProperties.ttcn

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } case ("TC_8_1_6_1_2_7_NR5GC") { return true; } case ("TC_8_1_6_1_3_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } case ("TC_8_1_6_1_4_5_NR5GC") { return true; } case ("TC_8_1_6_1_4_6_NR5GC") { return true; } case ("TC_8_1_6_1_4_7_NR5GC") { return true; } case ("TC_8_1_6_1_4_8_NR5GC") { return true; } case ("TC_8_2_7_2_1_NR5GC") { return true; } } return false; }

- Branches executed

## This NR TC was executed in NR-DC FR1+FR2 Band Combination DC_n79A_n257A with EEA1/EIA1

- Execution Log Files

  - Qualcomm Snapdragon X70 5G Modem-RF System

The Snapdragon X70 5G Modem-RF System passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_2_7_2_1_Qualcomm.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s221097: Supporting information for addition of NR-DC TC 8.2.7.2.1 |
| --- | --- |