3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221100

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2752 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for function fl_Check_featureSets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Variable v_Index is not update inside while loop. This cause TC stack in while loop. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For each while loop include increamention for variable v_Index. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC MCC160: affect also testcase 8.2.1.1.2.NEDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_Check_featureSets |
| --- | --- |
| Reason for change | Variable v_Index is not update inside while loop. This cause TC stack in while loop. |
| Summary of change | For each while loop include increamention for variable v_Index |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_Check_featureSets (UE_NR_Capability p_NRCapability) { var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index := 0; var boolean v_Found_dynamicSwitchSUL := false; var boolean v_Found_intraFreqDAPS_r16 := false; var boolean v_Found_pusch_RepetitionTypeB_r16 := false; v_Received_NRCapabilityMsg := p_NRCapability; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink)) and (not v_Found_dynamicSwitchSUL)) { // check featureSetsUplink if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink[v_Index].dynamicSwitchSUL)) { v_Found_dynamicSwitchSUL := true; } } if (not f_NR_CompareCapability_vs_PICS(pc_dynamicSwitch_SUL, v_Found_dynamicSwitchSUL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_dynamicSwitch_SUL not correctly set"); } v_Index := 0; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610)) and (not v_Found_intraFreqDAPS_r16)) { // check featureSetsDownlink_v1610 if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610[v_Index].intraFreqDAPS_r16)) { v_Found_intraFreqDAPS_r16 := true; } } if (not f_NR_CompareCapability_vs_PICS(pc_intraFreqDAPS_r16, v_Found_intraFreqDAPS_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_intraFreqDAPS_r16 not correctly set"); } v_Index := 0; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610)) and (not v_Found_pusch_RepetitionTypeB_r16)) { // check featureSetsUplink_v1610 if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610[v_Index].pusch_RepetitionTypeB_r16)) { v_Found_pusch_RepetitionTypeB_r16 := true; } } if (not f_NR_CompareCapability_vs_PICS(pc_pusch_RepetitionTypeB_r16, v_Found_pusch_RepetitionTypeB_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_pusch_RepetitionTypeB_r16 not correctly set"); } }

After Change:

function fl_Check_featureSets (UE_NR_Capability p_NRCapability) { var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index := 0; var boolean v_Found_dynamicSwitchSUL := false; var boolean v_Found_intraFreqDAPS_r16 := false; var boolean v_Found_pusch_RepetitionTypeB_r16 := false; v_Received_NRCapabilityMsg := p_NRCapability; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink)) and (not v_Found_dynamicSwitchSUL)) { // check featureSetsUplink if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink[v_Index].dynamicSwitchSUL)) { v_Found_dynamicSwitchSUL := true; } v_Index := v_Index + 1; } if (not f_NR_CompareCapability_vs_PICS(pc_dynamicSwitch_SUL, v_Found_dynamicSwitchSUL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_dynamicSwitch_SUL not correctly set"); } v_Index := 0; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610)) and (not v_Found_intraFreqDAPS_r16)) { // check featureSetsDownlink_v1610 if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610[v_Index].intraFreqDAPS_r16)) { v_Found_intraFreqDAPS_r16 := true; } v_Index := v_Index + 1; } if (not f_NR_CompareCapability_vs_PICS(pc_intraFreqDAPS_r16, v_Found_intraFreqDAPS_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_intraFreqDAPS_r16 not correctly set"); } v_Index := 0; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610)) and (not v_Found_pusch_RepetitionTypeB_r16)) { // check featureSetsUplink_v1610 if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610[v_Index].pusch_RepetitionTypeB_r16)) { v_Found_pusch_RepetitionTypeB_r16 := true; } v_Index := v_Index + 1; } if (not f_NR_CompareCapability_vs_PICS(pc_pusch_RepetitionTypeB_r16, v_Found_pusch_RepetitionTypeB_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_pusch_RepetitionTypeB_r16 not correctly set"); } }