3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221105

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2757 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At steps 3 and 22, the DRB configuration given to the SS will only include the entry for the DRB under test, but the UE may be configured with some additional DRBs if multiple PDU sessions were established during the preamble. This can mismatch cause issues as the SS may not send an RLC acknowledgement to a PDCP status report triggered on these DRBs following handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes done to pass list of all active DRBs, with 1st entry configured for MAC transparent mode, to function f_NR_RLC_ReconfigurationHO() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test case in multi-PDU session operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_1_9_1_NR_TestBody() |
| --- | --- |
| Reason for change | At steps 3 and 22, the DRB configuration given to the SS will only include the entry for the DRB under test, but the UE may be configured with some additional DRBs if multiple PDU sessions were established during the preamble. This can mismatch cause issues as the SS may not send an RLC acknowledgement to a PDCP status report triggered on these DRBs following handover. |
| Summary of change | Changes done to pass list of all active DRBs, with 1st entry configured for MAC transparent mode, to function f_NR_RLC_ReconfigurationHO() |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Accepted, according to RAN5 agreed prose CR R5-227418. |

Before Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5S210138 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_UL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; var SubFrameTiming_Type v_Timing; ……………… //@siclog "Step 7" siclog@ //@siclog "Steps 3-4a" siclog@ //@sic R5s210950 sic@ f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, -, -, omit), v_SS_DRB_Config, true, v_CellGroupConfig);//@sic R5-213132 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s210896 R5-215171 sic@ ……………….. //@siclog "Steps 22" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell //@sic R5s210950 sic@ v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, -, -, -,v_NR_HarqProcessConfig_UL);//@sic R5s220669 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, v_DciUlInfo, -, -); f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, -, -, omit),v_SS_DRB_Config,true, v_CellGroupConfig); //@sic R5-204418 R5s210138 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList,v_NR_HarqProcessConfig_UL); //Reconfiguring MAC to normal mode //@sic R5s220687 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); }

After Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); v_SS_Drb_ConfigList[0]:= cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5S210138 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_UL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; var SubFrameTiming_Type v_Timing; …………………… //@siclog "Step 7" siclog@ //@siclog "Steps 3-4a" siclog@ //@sic R5s210950 sic@ f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, -, -, omit), ,v_SS_Drb_ConfigList, true, v_CellGroupConfig);//@sic R5-213132 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s210896 R5-215171 sic@ ………………………………. //@siclog "Steps 22" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell //@sic R5s210950 sic@ v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, -, -, -,v_NR_HarqProcessConfig_UL);//@sic R5s220669 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, v_DciUlInfo, -, -); f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, -, -, omit),v_SS_DRB_ConfigList,true, v_CellGroupConfig);//@sic R5-204418 R5s210138 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList,v_NR_HarqProcessConfig_UL); //Reconfiguring MAC to normal mode //@sic R5s220687 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); }

## Change 2

| Function name | f_NR_RLC_ReconfigurationHO() |
| --- | --- |
| Reason for change | At steps 3 and 22, the DRB configuration given to the SS will only include the entry for the DRB under test, but the UE may be configured with some additional DRBs if multiple PDU sessions were established during the preamble. This can mismatch cause issues as the SS may not send an RLC acknowledgement to a PDCP status report triggered on these DRBs following handover. |
| Summary of change | Change paramter type to take list of DRBs for SS config, instead of single DRB. Note : Some changes for compilation will be needed in other test cases using this function : 7.1.2.2.6, 7.1.2.3.6, 7.1.2.3.8 |
| TTCN module | NR5GC_Handover.ttcn |
| MCC160 Comment | Accepted According to RAN5 agreed prose CR R5-227420 for TC 7.1.2.2.6 and RAN5 agreed prose CR R5-227421 for TC 7.1.2.3.11 |

Before Change:

function f_NR_RLC_ReconfigurationHO(NR_CellId_Type p_NR_CellId, DRB_Identity p_DRBId, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList, template (value) PDCP_Config p_PDCP_Config, template (omit) NR_RadioBearer_Type p_SS_Drb_Config := omit, boolean p_OmitRecoverPDCP := false, template (omit) CellGroupConfig p_CellGroupConfig := omit ) runs on NR5GC_PTC return SubFrameTiming_Type { //@sic R5-194296 R5-204512 sic@ var SubFrameTiming_Type v_Timing; v_Timing := f_NR_RLC_ReconfigurationHO_Steps1_8( p_NR_CellId, p_DRBId, p_RLC_BearerConfigList, p_PDCP_Config, p_SS_Drb_Config, p_OmitRecoverPDCP, p_CellGroupConfig); f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10(p_NR_CellId); return v_Timing; }

After Change:

function f_NR_RLC_ReconfigurationHO(NR_CellId_Type p_NR_CellId, DRB_Identity p_DRBId, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList, template (value) PDCP_Config p_PDCP_Config, template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList:= omit, boolean p_OmitRecoverPDCP := false, template (omit) CellGroupConfig p_CellGroupConfig := omit ) runs on NR5GC_PTC return SubFrameTiming_Type { //@sic R5-194296 R5-204512 sic@ var SubFrameTiming_Type v_Timing; v_Timing := f_NR_RLC_ReconfigurationHO_Steps1_8( p_NR_CellId, p_DRBId, p_RLC_BearerConfigList, p_PDCP_Config, p_SS_Drb_ConfigList, p_OmitRecoverPDCP, p_CellGroupConfig); f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10(p_NR_CellId); return v_Timing; }

## Change 3

| Function name | f_NR_RLC_ReconfigurationHO_Steps1_8() |
| --- | --- |
| Reason for change | At steps 3 and 22, the DRB configuration given to the SS will only include the entry for the DRB under test, but the UE may be configured with some additional DRBs if multiple PDU sessions were established during the preamble. This can mismatch cause issues as the SS may not send an RLC acknowledgement to a PDCP status report triggered on these DRBs following handover. |
| Summary of change | Change paramter type to take list of DRBs for SS config, instead of single DRB. |
| TTCN module | NR5GC_Handover.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_RLC_ReconfigurationHO_Steps1_8(NR_CellId_Type p_NR_CellId, DRB_Identity p_DRBId, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList, template (value) PDCP_Config p_PDCP_Config, template (omit) NR_RadioBearer_Type p_SS_Drb_Config := omit, boolean p_OmitRecoverPDCP := false, template (omit) CellGroupConfig p_CellGroupConfig := omit ) runs on NR5GC_PTC return SubFrameTiming_Type { var template (value) CellGroupConfig v_CellGroupConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RadioBearer_Type v_SS_Drb_Config; var template (value) DL_DCCH_Message v_RRCReconfiguration; ……………………… v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext); if (isvalue(p_SS_Drb_Config)) { // @sic R5s190905_r1 sic@ v_SS_Drb_Config := p_SS_Drb_Config; } else { v_SS_Drb_Config := cs_NR_SS_RadioBearer_AM_Test(p_DRBId, -); // size18; } v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, tsc_C_RNTI_Value3); v_Timing := f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step1_8(p_NR_CellId, v_RRCReconfiguration, false, {v_SS_Drb_Config}, v_RachProcedureConfig,tsc_C_RNTI_Value3, -, -, -); return v_Timing; }

After Change:

function f_NR_RLC_ReconfigurationHO_Steps1_8(NR_CellId_Type p_NR_CellId, DRB_Identity p_DRBId, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList, template (value) PDCP_Config p_PDCP_Config, template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit, boolean p_OmitRecoverPDCP := false, template (omit) CellGroupConfig p_CellGroupConfig := omit ) runs on NR5GC_PTC return SubFrameTiming_Type { var template (value) CellGroupConfig v_CellGroupConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) DL_DCCH_Message v_RRCReconfiguration; …………………….. v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext); if (isvalue(p_SS_Drb_ConfigList)) { // @sic R5s190905_r1 sic@ v_SS_Drb_ConfigList := p_SS_Drb_ConfigList; } else { v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_AM_Test(p_DRBId, -)}; // size18; } v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, tsc_C_RNTI_Value3); v_Timing := f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step1_8(p_NR_CellId, v_RRCReconfiguration, false, v_SS_Drb_ConfigList, v_RachProcedureConfig,tsc_C_RNTI_Value3, -, -, -); return v_Timing; }