3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221111

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2761 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 11.4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current implementation of the function fl_TC_11_4_5_NR5GC_TestBody after Step 17 the UE will be in idle state. As a result the postamble should be done in state STATE_IDLE_1A. This needs to be corected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_TC_11_4_5_NR5GC, called f_NR_Postamble(nr_Cell1, STATE_IDLE_1A /*WA#11_4_5 STATE_CONNECTED_3A*/); instead of f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the function f_TC_11_4_5_NR5GC	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required

  - Correction to the function f_TC_11_4_5_NR5GC

| Template name | f_TC_11_4_5_NR5GC |
| --- | --- |
| Reason for change | In the current implementation of the function fl_TC_11_4_5_NR5GC_TestBody after Step 17 the UE will be in idle state. As a result the postamble should be done in state STATE_IDLE_1A. This needs to be corected. |
| Summary of change | In the function f_TC_11_4_5_NR5GC, called f_NR_Postamble(nr_Cell1, STATE_IDLE_1A /*WA#11_4_5 STATE_CONNECTED_3A*/); instead of f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function f_TC_11_4_5_NR5GC() runs on NR5GC_PTC { //5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming f_NR5GC_Init(NR_1); //Create and configure cells//@sic R5s210875 sic@ f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11);//@sic R5-217820 sic@ //Set power for the cells//@sic R5s210875 sic@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1 //In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_11_4_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);//@sic R5s210875 sic@ }

After Change:

function f_TC_11_4_5_NR5GC() runs on NR5GC_PTC { //5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming f_NR5GC_Init(NR_1); //Create and configure cells//@sic R5s210875 sic@ f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11);//@sic R5-217820 sic@ //Set power for the cells//@sic R5s210875 sic@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1 //In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_11_4_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A /*WA#11_4_5 STATE_CONNECTED_3A*/);//@sic R5s210875 sic@ }