3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221119

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2768 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC PDCP test case 7.1.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 8 and 24, the DRB configuration given to the SS will only include the entry for the DRB under test, but the UE may be configured with some additional DRBs if multiple PDU sessions were established during the preamble. This can mismatch cause issues as the SS may not send an RLC acknowledgement to a PDCP status report triggered on these DRBs following handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include full list of configured DRBs in the v_SS_Drb_ConfigList DRB config list, used at steps 8 and 24. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test casein multi-PDU session operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_3_4_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | At step 8 and 24, the DRB configuration given to the SS will only include the entry for the DRB under test, but the UE may be configured with some additional DRBs if multiple PDU sessions were established during the preamble. This can mismatch cause issues as the SS may not send an RLC acknowledgement to a PDCP status report triggered on these DRBs following handover. |
| Summary of change | Include full list of configured DRBs in the v_SS_Drb_ConfigList DRB config list, used at steps 8 and 24. |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment | Accepted {… } removed v_SS_Drb_ConfigList[0] := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None, -, v_T_PollRetransmit)}; |

Before Change:

function fl_TC_7_1_3_4_1_NR5GC_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR5GC_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var template (omit) MasterKeyUpdate v_MasterKeyUpdate:= cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); //@sic R5-202618 sic@ var template (value) CellGroupConfig v_CellGroupConfig1; var template (value) CellGroupConfig v_CellGroupConfig2; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms150; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //Initialise RLC_BearerConfig with re-establishment var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None, -, v_T_PollRetransmit)};// @sic R5s200690sic@ //@sic R5s200690 sic@ //CellGroupCongfig with condition PCell_change v_CellGroupConfig1 := f_NR_GetCellGroupConfigHO( nr_Cell2, v_RLC_BearerToAddModList, tsc_C_RNTI_Value3, omit ); v_CellGroupConfig2 := f_NR_GetCellGroupConfigHO( nr_Cell1, v_RLC_BearerToAddModList, tsc_C_RNTI_Value1, omit ); p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; f_TC_7_1_3_4_1_NR_Step1to7(p_NR_PDCP_State); //@siclog "Step 8" siclog@ //The SS requests UE to make a handover to NR Cell 2 f_NR5GC_PDCP_IntraNR_HO_InterCell(nr_Cell1, nr_Cell2, -,true, v_MasterKeyUpdate,v_CellGroupConfig1,v_SS_Drb_ConfigList); // @sic R5s200690sic@ f_TC_7_1_3_4_1_NR_Step10to23(p_NR_PDCP_State,true); //@sic R5-204363 sic@ //@sicR5s200690 sic@ DRB Configuration in target Cell v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None, -, v_T_PollRetransmit)}; //@siclog "Step 23-24" siclog@ //The SS requests UE to make a handover to NR Cell 1 with the NR RRCReconfiguration sent on NR Cell 2 with key change. f_NR5GC_PDCP_IntraNR_HO_InterCell(nr_Cell2,nr_Cell1, tsc_C_RNTI_Value1, true, v_MasterKeyUpdate,v_CellGroupConfig2,v_SS_Drb_ConfigList);// @sic R5s200690sic@ f_TC_7_1_3_4_1_NR_Step25to30(p_NR_PDCP_State,true);//@sic R5-204363 sic@ }

After Change:

function fl_TC_7_1_3_4_1_NR5GC_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR5GC_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var template (omit) MasterKeyUpdate v_MasterKeyUpdate:= cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); //@sic R5-202618 sic@ var template (value) CellGroupConfig v_CellGroupConfig1; var template (value) CellGroupConfig v_CellGroupConfig2; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms150; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //Initialise RLC_BearerConfig with re-establishment var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); v_SS_Drb_ConfigList[0] := f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None, -, v_T_PollRetransmit);// @sic R5s200690sic@ //@sic R5s200690 sic@ //CellGroupCongfig with condition PCell_change v_CellGroupConfig1 := f_NR_GetCellGroupConfigHO( nr_Cell2, v_RLC_BearerToAddModList, tsc_C_RNTI_Value3, omit ); v_CellGroupConfig2 := f_NR_GetCellGroupConfigHO( nr_Cell1, v_RLC_BearerToAddModList, tsc_C_RNTI_Value1, omit ); p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; f_TC_7_1_3_4_1_NR_Step1to7(p_NR_PDCP_State); //@siclog "Step 8" siclog@ //The SS requests UE to make a handover to NR Cell 2 f_NR5GC_PDCP_IntraNR_HO_InterCell(nr_Cell1, nr_Cell2, -,true, v_MasterKeyUpdate,v_CellGroupConfig1,v_SS_Drb_ConfigList); // @sic R5s200690sic@ f_TC_7_1_3_4_1_NR_Step10to23(p_NR_PDCP_State,true); //@sic R5-204363 sic@ //@sicR5s200690 sic@ DRB Configuration in target Cell v_SS_Drb_ConfigList[0] := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None, -, v_T_PollRetransmit)}; //@siclog "Step 23-24" siclog@ //The SS requests UE to make a handover to NR Cell 1 with the NR RRCReconfiguration sent on NR Cell 2 with key change. f_NR5GC_PDCP_IntraNR_HO_InterCell(nr_Cell2,nr_Cell1, tsc_C_RNTI_Value1, true, v_MasterKeyUpdate,v_CellGroupConfig2,v_SS_Drb_ConfigList);// @sic R5s200690sic@ f_TC_7_1_3_4_1_NR_Step25to30(p_NR_PDCP_State,true);//@sic R5-204363 sic@ }