3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221120

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4692 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT testcase 22.5.18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect cell ID is used when resetting of radio bearers at Step 67 |  |  |  |  |  |  |  |  |
| Summary of change: |  | At Step 67, radio bearers of nbiot_Cell11 is reset in place of nbiot_Cell1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN will not be in-line with prose |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_22_5_18_Body |
| --- | --- |
| Reason for change | Incorrect cell ID is used when resetting of radio bearers at Step 67 |
| Summary of change | At Step 67, radio bearers of nbiot_Cell11 is reset in place of nbiot_Cell1 |
| TTCN module | NBIOT\22_5\NBIOT_NAS_Attach.ttcn |
| MCC160 Comment | Accepted |

Before change:

function fl_TC_22_5_18_Body() runs on NBIOT_PTC

{

//<=== SKIPPED ===>

//@siclog "Step 62 - 65B" siclog@

// Steps 62-63: Steps 2 to 3 of the NB-IoT Control Plane CIoT MT user data transfer non-SMS transport procedure (TS 36.508 8.1.5A.2.3) are executed.

// Steps 64-65: Void

// Step 65A : Check: Does the UE transmit a CONTROL PLANE SERVICE REQUEST according to step 4a1a1 or 4a1b1 in the generic procedure in TS 36.508 8.1.5A.2.3?

// Step 65B: Steps 4a1b1 to 5 or only step 5 of the NB-IoT Control Plane CIoT MT user data transfer non-SMS transport procedure (TS 36.508 8.1.5A.2.3) are executed

f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5(nbiot_Cell11);

//@siclog "Step 66" siclog@

//The SS releases the RRC connection.

f_NBIOT_RRC_ConnectionRelease(nbiot_Cell11);

//@siclog "Step 67" siclog@

// The UE is switched off - Note 1: This implies detaching of the UE, releasing of the RRC connection and resetting of the radio bearers at the SS side.

f_NBIOT_SS_SRBs_DRBs_Reset (nbiot_Cell1, CONTROL_PLANE);

f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell11, CONTROL_PLANE, RRC_IDLE);

//@siclog "Step 68" siclog@

// Set the cell type of Ncell 1 to the "Serving cell"

// Set the cell type of Ncell 11 to the "Non-Suitable cell".

v_CellPowerList := {

cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE),

cs_NB_CellPower(nbiot_Cell11, tsc_NonSuitableNBIOTCellRS_EPRE)

};

f_NBIOT_SetCellPowerList(v_CellPowerList);

//<=== SKIPPED ===>

}

After change:

function fl_TC_22_5_18_Body() runs on NBIOT_PTC

{

//<=== SKIPPED ===>

//@siclog "Step 62 - 65B" siclog@

// Steps 62-63: Steps 2 to 3 of the NB-IoT Control Plane CIoT MT user data transfer non-SMS transport procedure (TS 36.508 8.1.5A.2.3) are executed.

// Steps 64-65: Void

// Step 65A : Check: Does the UE transmit a CONTROL PLANE SERVICE REQUEST according to step 4a1a1 or 4a1b1 in the generic procedure in TS 36.508 8.1.5A.2.3?

// Step 65B: Steps 4a1b1 to 5 or only step 5 of the NB-IoT Control Plane CIoT MT user data transfer non-SMS transport procedure (TS 36.508 8.1.5A.2.3) are executed

f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5(nbiot_Cell11);

//@siclog "Step 66" siclog@

//The SS releases the RRC connection.

f_NBIOT_RRC_ConnectionRelease(nbiot_Cell11);

//@siclog "Step 67" siclog@

// The UE is switched off - Note 1: This implies detaching of the UE, releasing of the RRC connection and resetting of the radio bearers at the SS side.

f_NBIOT_SS_SRBs_DRBs_Reset (nbiot_Cell11, CONTROL_PLANE);

f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell11, CONTROL_PLANE, RRC_IDLE);

//@siclog "Step 68" siclog@

// Set the cell type of Ncell 1 to the "Serving cell"

// Set the cell type of Ncell 11 to the "Non-Suitable cell".

v_CellPowerList := {

cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE),

cs_NB_CellPower(nbiot_Cell11, tsc_NonSuitableNBIOTCellRS_EPRE)

};

f_NBIOT_SetCellPowerList(v_CellPowerList);

//<=== SKIPPED ===>

}