3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221124

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2772 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC RRC measurement test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According e.g. table Table 8.2.5.1.2.3.3-2, measResultFreqList should be “not checked”. Current implementation uses the default value “{cr_MeasResultFreqFailNR} ifpresent” which has expected values of “rsrpResult_r15” and “rsrqResult_r15” set to “0”. Pass “*” to cr_FailureReportSCG_NR_r15 for p_MeasResultFreqFail |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass “*” to cr_FailureReportSCG_NR_r15 for p_MeasResultFreqFail |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.1.1.ENDC, 8.2.5.2.1.ENDC, 8.2.5.3.1.ENDC, 8.2.5.4.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | e.g. fl_TC_8_2_5_1_1_ENDC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | According e.g. table Table 8.2.5.1.2.3.3-2, measResultFreqList should be “not checked”. Current implementation uses the default value “{cr_MeasResultFreqFailNR} ifpresent” which has expected values of “rsrpResult_r15” and “rsrqResult_r15” set to “0”. |
| Summary of change | Pass “*” to cr_FailureReportSCG_NR_r15 for p_MeasResultFreqFail Note : Same correction needed for 8.2.5.2.1.ENDC, 8.2.5.3.1.ENDC, 8.2.5.4.1.ENDC |
| TTCN module | EUTRA_RRC_Templates.ttcn |
| MCC160 Comment | Accepted. Same solution applied also in: 7.1.3.2.1.ENDC, 7.1.3.2.2.ENDC, 7.1.3.2.3.ENDC, 8.2.3.18.2.ENDC |

Before Change:

function fl_TC_8_2_5_1_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { // @sic R5-187795 sic@ timer t_Timer := 5.0; f_EUTRA_NR_WaitForCoOrd_Trigger(NR); t_Timer.start; alt{ // @siclog "Step 4" siclog@ // Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformationNR message with failureType set to 'randomAccessProblem'? [] SRB.receive (car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(randomAccessProblem)))) {// verdict pass the UE transmit SCGFailureInformationNR message in the next 5 sec f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_Timer.stop; } [] t_Timer.timeout {// verdict fail the UE doesn't transmit SCGFailureInformationNR message in the next 5 sec f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } }

After Change:

function fl_TC_8_2_5_1_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { // @sic R5-187795 sic@ timer t_Timer := 5.0; f_EUTRA_NR_WaitForCoOrd_Trigger(NR); t_Timer.start; alt{ // @siclog "Step 4" siclog@ // Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformationNR message with failureType set to 'randomAccessProblem'? [] SRB.receive (car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(randomAccessProblem, *)))) {// verdict pass the UE transmit SCGFailureInformationNR message in the next 5 sec f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_Timer.stop; } [] t_Timer.timeout {// verdict fail the UE doesn't transmit SCGFailureInformationNR message in the next 5 sec f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } }

MCC160 implementation

module RRCReconfigFailure_ENDC_EUTRA

function fl_TC_8_2_5_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{ // @sic R5-190024 sic@

timer t_Timer := 5.0;

// @siclog "Step 1" siclog@

f_Delay(2.0);

// The SS changes NR Cell 1 parameter to non-suitable "Off"

// Send trigger to NR to change power level of NR Cell1

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Step 2" siclog@

// Void.

// @siclog "Step 3" siclog@

// Check: Does the UE transmit in the next 5 sec (NOTE 1) a SCGFailureInformationNR message with failureType set to 't310-Expiry'?

t_Timer.start;

alt{

[] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(t310_Expiry,*,omit)))) /* @sic R5s221124 sic@ */

{

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3");

t_Timer.stop;

}

[] t_Timer.timeout

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3");

}

}

}

. . .

function fl_TC_8_2_5_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

timer t_WatchDog := 5.0; // 5 sec (arbitrary value) for a SCGFailureInformationNR message

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

// @siclog "Step 4" siclog@

// Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformationNR message with failureType set to "rlc-MaxNumRetx"?

t_WatchDog.start;

alt {

[] t_WatchDog.timeout

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); // No SCGFailureInformationNR received

}

[] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(rlc_MaxNumRetx, *)))) /* @sic R5s221124 sic@ */

{

t_WatchDog.stop;

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

}

} // alt

// Send trigger to NR to finish the test case

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

}

. . .

function fl_TC_8_2_5_4_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{ // @sic R5s180647 change 8 sic@

var float v_DelayBeforeRRCConnectionReconfiguration := 5.0;   // 5s for the NR side to provide necessary information

var float v_T304Min_1sec := f_EUTRA_SetTimerToleranceMin (eutra_Cell1, rrcTimer, 1.0);

var SubFrameTiming_Type v_SubframeTiming;

var template (value) TimingInfo_Type v_TimingInfo;

timer t_T304Expiry;

timer t_WatchDog;

v_SubframeTiming := f_EUTRA_GetNextSendOccasion(eutra_Cell1, float2int(v_DelayBeforeRRCConnectionReconfiguration * 1000.0));  // send RRCConnectionReconfiguration 5.0 from now on

t_T304Expiry.start(v_DelayBeforeRRCConnectionReconfiguration + v_T304Min_1sec);                                               // t_T304Expiry expires v_T304Min_1sec after RRCConnectionReconfiguration has been sent

t_WatchDog.start(v_DelayBeforeRRCConnectionReconfiguration + v_T304Min_1sec + 5.0);                                           // 5.0s for the UE to send FailureReportSCG after expiry of T304

v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_SubframeTiming);

// @siclog "Step 1" siclog@

f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, SCG_RBConfig, -, -, -, -, -, -, -, v_TimingInfo);

// @siclog "Step 2" siclog@

t_T304Expiry.timeout;

// @siclog "Step 3" siclog@

// Check: Does the UE transmit on PCell (E-UTRA Cell 1) in the next 5 sec (arbitrary value) a SCGFailureInformationNR message with failureType set to 'synchReconfigFailure-SCG'?

alt {

[] t_WatchDog.timeout

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3");

}

[] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(synchReconfigFailureSCG, *)))) // @sic R5-191897 R5s221124 sic@

{

t_WatchDog.stop;

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3");

}

}

}

module RRC_CPC_ENDC_EUTRA

function fl_TC_8_2_3_18_2_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var template (omit) octetstring v_NR_SCGConfig := omit;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params;

var PhysCellId v_PhysCellId_Cell2;

var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def;

var template (value) MeasConfig v_MeasConfig;

timer t_WatchDog := 10.0;

// @siclog "Step 1" siclog@

// The SS transmits an RRCConnectionReconfiguration message containing NR RRCReconfiguration message to perform conditional PSCell change with ConditionalReconfiguration to set NR Cell 2 as target candidate cell.

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); //Rx. reconfiguration contents from NR side

f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig, -, -, omit);

v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR);

f_EUTRA38_Security_Set(v_Security_Params);

// @siclog "Step 2" siclog@

// The UE transmits an RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on E-UTRA Cell 1

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Step 3" siclog@

// SS adjusts the cell-specific reference signal level according to row "T1" -  no power level change in EUTRA side

// @siclog "Step 4" siclog@

// Wait for 5s to ensure that T304 expires.

// NOTE: SS does not respond to the UE transmitted RACH preambles on NR Cell 2.

f_Delay(5.0);

// @siclog "Step 5" siclog@

// Check: Does UE transmit the SCGFailureInformationNR message with failureType set to 'synchReconfigFailureSCG' in E-UTRA Cell 1?

t_WatchDog.start;

alt {

[] t_WatchDog.timeout

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5");

}

[] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(synchReconfigFailureSCG, *)))) /* @sic R5s221124 sic@ */

{

t_WatchDog.stop;

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5: UE transmit the SCGFailureInformationNR message with failureType set to synchReconfigFailureSCG in E-UTRA Cell 1");

}

}

. . .

module PDCP_ENDC_EUTRA

function fl_EUTRA_PDCP_Integrity(IntegrityProtAlgorithm_Type p_AS_IntegrityProtAlgorithm) runs on EUTRA_5GS_PTC

{//@sic R5-202614 sic@

var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2;

if (pc_srb3){

v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3;

}

f_EUTRA38_Init(EN_DC, c1);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

//Set integrity algorithm

f_EUTRA_AS_IntegrityAlgorithm_Set(p_AS_IntegrityProtAlgorithm);//@sic R5-202614 sic@

//Preamble to enter UE in state 2A . NR PDCP is configured on SRB1 and SRB2 and SRB 3

//@sic R5-195339 sic@

f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_SCG, RRC_CONNECTED);//@sic R5s190476 sic@ ch 4.8 @sic R5s211168 sic@

f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP (eutra_Cell1, v_SecondaryConfig,1); //@sic R5s190476 sic@ ch 4.9

f_EUTRA_TestBody_Set(true);

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

//@siclog "Step 1a1-1a2" siclog@

//@sic R5-212370 sic@

//IF Connectivity is EN-DC or NGEN-DC, the SS sends EUTRA RRC UECapabilityEnquiry including RAT-Type eutra message to the UE.

//Check: Does the UE send a EUTRA RRC UECapabilityInformation message?

//@sic R5s201545 sic@

//@sic R5-206327 sic@

f_EUTRA_CheckUeCapabilityEnquiry(eutra_Cell1);

if (pc_srb3){

//@siclog "Step 2a4" siclog@

//Does the UE send SCGFailureInformationNR with failureType as srb3-IntegrityFailure on SRB1?

SRB.receive (car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(srb3_IntegrityFailure, *)))) /* @sic R5s221124 sic@ */

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

}

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

f_EUTRA_TestBody_Set(false);

f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204475 sic@

// If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell!

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble"));

}