3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221125

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0864 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS emergency call test case 10.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following the implementation of CR R5-225426, the IMS PTC now sends IPCAN_ENDPROCEDURE (configuration = IPCAN_SpeechAndEmergencyCall, procedure = IPCAN_EmergencyCall_NormalService) after step 18, to trigger release of the emergency voice bearer. Then later after 180 ringing is sent for the next emergency call, it sends a trigger to IPCAN from the function f_IMS_CC_TriggerDedicatedBearerActivation() but since IPCAN state machine has ended its procedure and is back in its main loop, it will not accept any coordination trigger. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced the trigger with an IPCAN_STARTPROCEDURE(configuration = IPCAN_SpeechAndEmergencyCall, procedure = IPCAN_EmergencyCall_NormalService) command to start a new IPCAN procedure, to add the emergency voice bearer for the 2nd call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_10_6_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Following the implementation of CR R5-225426, the IMS PTC now sends IPCAN_ENDPROCEDURE (configuration = IPCAN_SpeechAndEmergencyCall, procedure = IPCAN_EmergencyCall_NormalService) after step 18, to trigger release of the emergency voice bearer. Then later after 180 ringing is sent for the next emergency call, it sends a trigger to IPCAN from the function f_IMS_CC_TriggerDedicatedBearerActivation() but since IPCAN state machine has ended its procedure and is back in its main loop, it will not accept any coordination trigger. |
| Summary of change | Replaced the trigger with an IPCAN_STARTPROCEDURE(configuration = IPCAN_SpeechAndEmergencyCall, procedure = IPCAN_EmergencyCall_NormalService) command to start a new IPCAN procedure, to add the emergency voice bearer for the 2nd call. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted in principle, but still used the function f_IMS_CC_TriggerDedicatedBearerActivation: f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_EmergencyCall_NormalService, IPCAN_SpeechAndEmergencyCall); Implemented under R5s221136 |

Before Change:

function f_TC_10_6_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ … … … // Step 3 of Annex A.6. SS sends a 180 Ringing. IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine180, f_IMS_InviteResponse_180_MessageHeaderTX(v_InviteRequest, -, -, -, v_EmergencyType)))); f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MT_SpeechCall); // @sic R5-225426 sic@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_EmergencyCall_NormalService, // Step 4 of Annex A.6. SS responds to the invite INVITE with 200 OK including the SDP record. v_SDP_Message_Step4 := f_IMS_BuildSDP_EmergencyCallAnswer_TX(v_AudioCodecInfo.Fmt, v_AudioCodecInfo.Codec); /* @sic R5s141244: no origin address to be handed over anymore sic@ @sic R5s170327 sic@ */ v_MessageHeader_InviteResponse_200 := f_IMS_OtherResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, cs_ContentTypeSDP, tsc_IMS_Emergency_TelUri, -, -, v_EmergencyType); v_CalleeContactSipUri := f_Addr_Union_GetSipUrl(v_MessageHeader_InviteResponse_200.contact.contactBody.contactAddresses[0].addressField); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader_InviteResponse_200, cs_MessageBody_SDP(v_SDP_Message_Step4)))); … … … }

After Change:

function f_TC_10_6_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ … … … // Step 3 of Annex A.6. SS sends a 180 Ringing. IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine180, f_IMS_InviteResponse_180_MessageHeaderTX(v_InviteRequest, -, -, -, v_EmergencyType)))); f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_EmergencyCall_NormalService, IPCAN_SpeechAndEmergencyCall); //ADDED // Step 4 of Annex A.6. SS responds to the invite INVITE with 200 OK including the SDP record. v_SDP_Message_Step4 := f_IMS_BuildSDP_EmergencyCallAnswer_TX(v_AudioCodecInfo.Fmt, v_AudioCodecInfo.Codec); /* @sic R5s141244: no origin address to be handed over anymore sic@ @sic R5s170327 sic@ */ v_MessageHeader_InviteResponse_200 := f_IMS_OtherResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, cs_ContentTypeSDP, tsc_IMS_Emergency_TelUri, -, -, v_EmergencyType); v_CalleeContactSipUri := f_Addr_Union_GetSipUrl(v_MessageHeader_InviteResponse_200.contact.contactBody.contactAddresses[0].addressField); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader_InviteResponse_200, cs_MessageBody_SDP(v_SDP_Message_Step4)))); … … … }