3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221128

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0866 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS emergency call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In some test cases e.g. 10.3.NR5GC and 10.6.NR5GC in which there is an emergency call made and released during an ongoing MO call, the function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media() is called from IPCAN following release of the emergency call. In this case there will be 2 active PDU sessions which are both using QFI 7. Currently the starting list of DRBs will only include those related to the IMS PDU session, not the emergency PDU session. Also the code in this function can select the wrong DRB ID as it is only checking for the QFI, not the PDU session ID. This can cause the wrong DRB to be released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to search through all DRBs of all PDU Sessions, existing function f_NR5GC_MobileInfo_GetDRBInfoList() is used and the type of the variable is changed from “IntegerList_Type” to “NR_DRBInfoList_Type”. The check for QFI 7 is extended to also check the PDU session, to cover this case where there are 2 active PDU sessions both this QFI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.NR5GC, 10.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_NR5GC_ModifyPDUSession_RemoveIMS_Media() |
| --- | --- |
| Reason for change | In some test cases e.g. 10.3.NR5GC and 10.6.NR5GC in which there is an emergency call made and released during an ongoing MO call, the function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media() is called from IPCAN following release of the emergency call. In this case there will be 2 active PDU sessions which are both using QFI 7. Currently the starting list of DRBs will only include those related to the IMS PDU session, not the emergency PDU session. Also the code in this function can select the wrong DRB ID as it is only checking for the QFI, not the PDU session ID. This can cause the wrong DRB to be released. |
| Summary of change | In order to search through all DRBs of all PDU Sessions, existing function f_NR5GC_MobileInfo_GetDRBInfoList() is used and the type of the variable is changed from “IntegerList_Type” to “NR_DRBInfoList_Type”. The check for QFI 7 is extended to also check the PDU session, to cover this case where there are 2 active PDU sessions both this QFI |
| TTCN module | NR5GC_IMSProcedures |
| MCC160 Comment | Not Needed. Alternative solution implemented under R5s221134 |

Before Change:

function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(NR_CellId_Type p_NR_CellId, ListOfMedia_Type p_MediaType, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForIMSPDUSession(); var NR_DRBInfo_Type v_DRBInfo; var IntegerList_Type v_DRB_ToReleaseList; var template (value) QoS_RuleList v_QoS_Rules; var template (value) QoS_FlowList v_QoS_Flows; var boolean v_RemoveVoice := false; var boolean v_RemoveVideo := false; var integer i; var integer j := 0; for (i := 0; i < lengthof(p_MediaType); i := i+1) { // @sic R5-217798 sic@ select (p_MediaType[i]) { case(IPCAN_SpeechCall, IPCAN_EmergencyCall) { // @sic R5-225426 sic@ v_RemoveVoice := true; } case(IPCAN_VideoCall) { v_RemoveVideo := true; } } } // @sic R5-216150 sic@ if (v_RemoveVoice) { // @sic R5-217798 sic@ v_QoS_Rules[j] := cs_QoS_DeleteRule('03'O); // @sic R5s210073, R5-213439 sic@ v_QoS_Flows[j] := cs_QoS_DeleteFlow('000111'B); // @sic R5-213439 sic@ j := j+1; } if (v_RemoveVideo) { v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O); // @sic R5s220500 sic@ v_QoS_Flows[j] := cs_QoS_DeleteFlow('001000'B); } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, cs_QoS_Rules(v_QoS_Rules, '7A'O), cs_QoSFlowDescr(v_QoS_Flows)); j := 0; // reset index // @sic R5-211171 sic@ for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_DRBIdList[i])); if (not (v_DRBInfo.DefaultDRB)) { if ((v_RemoveVoice and (v_DRBInfo.SDAP_Config.mappedQoS_FlowsToAdd[0] == 7)) or v_RemoveVideo and (v_DRBInfo.SDAP_Config.mappedQoS_FlowsToAdd[0] == 8)) { // @sic R5-217798 sic@ v_DRB_ToReleaseList[j] := v_DRBInfo.DRB_Id; j := j+1; } } } v_ReceivedAsp := f_NR_RRCReconfigReleaseDRBs(p_NR_CellId, v_DRB_ToReleaseList, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend)); // @sic R5s220595 sic@ if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } if (v_RemoveVoice) { // @sic R5-217798 sic@ f_IP_RtpRtcpLoopback(IP, valueof(p_GSM_MobilityInfo.PdnIndex), tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ } if (v_RemoveVideo) { // @sic R5-217798 sic@ f_IP_RtpRtcpLoopback(IP, valueof(p_GSM_MobilityInfo.PdnIndex), tsc_IMS_MediaPort_M2, omit); // @sic R5s210363 sic@ } }

After Change:

function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(NR_CellId_Type p_NR_CellId, ListOfMedia_Type p_MediaType, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var NR_DRBInfoList_Type v_DRBList := f_NR5GC_MobileInfo_GetDRBInfoList(); var NR_DRBInfo_Type v_DRBInfo; var IntegerList_Type v_DRB_ToReleaseList; var template (value) QoS_RuleList v_QoS_Rules; var template (value) QoS_FlowList v_QoS_Flows; var boolean v_RemoveVoice := false; var boolean v_RemoveVideo := false; var integer i; var integer j := 0; for (i := 0; i < lengthof(p_MediaType); i := i+1) { // @sic R5-217798 sic@ select (p_MediaType[i]) { case(IPCAN_SpeechCall, IPCAN_EmergencyCall) { // @sic R5-225426 sic@ v_RemoveVoice := true; } case(IPCAN_VideoCall) { v_RemoveVideo := true; } } } // @sic R5-216150 sic@ if (v_RemoveVoice) { // @sic R5-217798 sic@ v_QoS_Rules[j] := cs_QoS_DeleteRule('03'O); // @sic R5s210073, R5-213439 sic@ v_QoS_Flows[j] := cs_QoS_DeleteFlow('000111'B); // @sic R5-213439 sic@ j := j+1; } if (v_RemoveVideo) { v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O); // @sic R5s220500 sic@ v_QoS_Flows[j] := cs_QoS_DeleteFlow('001000'B); } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, cs_QoS_Rules(v_QoS_Rules, '7A'O), cs_QoSFlowDescr(v_QoS_Flows)); j := 0; // reset index // @sic R5-211171 sic@ for (i := 0; i < lengthof(v_DRBList); i := i+1) { v_DRBInfo := v_DRBList[i]; if (not (v_DRBInfo.DefaultDRB)) { if ((v_RemoveVoice and (v_DRBInfo.SDAP_Config.mappedQoS_FlowsToAdd[0] == 7) and (v_DRBInfo.SDAP_Config.pdu_Session == oct2int(p_GSM_MobilityInfo.SessionId))) or v_RemoveVideo and (v_DRBInfo.SDAP_Config.mappedQoS_FlowsToAdd[0] == 8) { // @sic R5-217798 sic@ //anupams@22wk37 v_DRB_ToReleaseList[j] := v_DRBInfo.DRB_Id; j := j+1; } } } v_ReceivedAsp := f_NR_RRCReconfigReleaseDRBs(p_NR_CellId, v_DRB_ToReleaseList, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend)); // @sic R5s220595 sic@ if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } if (v_RemoveVoice) { // @sic R5-217798 sic@ f_IP_RtpRtcpLoopback(IP, valueof(p_GSM_MobilityInfo.PdnIndex), tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ } if (v_RemoveVideo) { // @sic R5-217798 sic@ f_IP_RtpRtcpLoopback(IP, valueof(p_GSM_MobilityInfo.PdnIndex), tsc_IMS_MediaPort_M2, omit); // @sic R5s210363 sic@ } }