3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221134

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0867 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With implementation of R5-225426, now voice media can be removed from an emergency call once the call is terminated but keep the PDU established. To accomplish that, function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media is called, function used previously to remove voice and video media for normal IMS calls, it selects from a DRB list media to be released. The problem comes when the only DRB list provided is related to IMS PDU, DRB list should be assigned depending on the nature of the call: IMS PDU DRB list for normal calls or emergency PDU list for emergency calls. Wth current implementation, DRB list is taken always from IMS PDU session, this can be fixed taking IMS or emergency PDU session depending on type of the call to be released. An additional typo change has been added to this CR document, explained in correction 1.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Assigned the correct DRB list for media removal depending on call type: normal calls or emergency calls. Corrected verdict. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.NR5GC and other NR IMS emergency test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	4

2.1	Correction to function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media	4

2.2	Correction to function f_NR5GC_ModifyPDUSession_RemoveEmergency	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media

| Function name | fl_NR5GC_ModifyPDUSession_RemoveIMS_Media |
| --- | --- |
| Reason for change | The only DRB list provided is related to IMS PDU, DRB list should be assigned depending on the nature of the call: IMS PDU DRB list for normal calls or emergency PDU list for emergency calls. Wth current implementation, DRB list is taken always from IMS PDU session, this can be fixed taking IMS or emergency PDU session depending on type of the call to be released. |
| Summary of change | Assigned the correct DRB list for media removal depending on call type: normal calls or emergency calls. |
| TTCN module | Common\NR5GC\NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(NR_CellId_Type p_NR_CellId, ListOfMedia_Type p_MediaType, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForIMSPDUSession(); var NR_DRBInfo_Type v_DRBInfo; var IntegerList_Type v_DRB_ToReleaseList; var template (value) QoS_RuleList v_QoS_Rules; var template (value) QoS_FlowList v_QoS_Flows; var boolean v_RemoveVoice := false; var boolean v_RemoveVideo := false; var integer i; var integer j := 0; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(NR_CellId_Type p_NR_CellId, ListOfMedia_Type p_MediaType, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(valueof(p_GSM_MobilityInfo.SessionId))); var NR_DRBInfo_Type v_DRBInfo; var IntegerList_Type v_DRB_ToReleaseList; var template (value) QoS_RuleList v_QoS_Rules; var template (value) QoS_FlowList v_QoS_Flows; var boolean v_RemoveVoice := false; var boolean v_RemoveVideo := false; var integer i; var integer j := 0; <<SKIPPED CODE>>

## Correction to function f_NR5GC_ModifyPDUSession_RemoveEmergency

| Function name | f_NR5GC_ModifyPDUSession_RemoveEmergency |
| --- | --- |
| Reason for change | Call released for this function is an emergency call not a video call. |
| Summary of change | Changed the type of call released. |
| TTCN module | Common\NR5GC\NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> function f_NR5GC_ModifyPDUSession_RemoveEmergency(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(p_NR_CellId, {IPCAN_EmergencyCall}, valueof(v_GSM_MobilityInfo)); // @sic R5-225426 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Video Call Release"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_NR5GC_ModifyPDUSession_RemoveEmergency(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(p_NR_CellId, {IPCAN_EmergencyCall}, valueof(v_GSM_MobilityInfo)); // @sic R5-225426 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); } <<SKIPPED CODE>>