3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221143

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2785 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If EUTRA band is greater than maxFBI then "freqBandIndicator-v9e0" should be included in the SIB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include "freqBandIndicator-v9e0" in SIB1 forEUTRA bands greater than or equal to maxFBI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.10.NR5GC, 6.2.3.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2 Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_3_10_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | If EUTRA band is greater than maxFBI then "freqBandIndicator-v9e0" should be included in the SIB1. |
| Summary of change | Include "freqBandIndicator-v9e0" in SIB1 forEUTRA bands greater than or equal to maxFBI. |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted but implemented differently – see proposed implementation below |

Before Change:

function f_TC_6_2_3_10_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { …………….. //Modify SIB1 scheduling as per Table 6.2.3.10.3.3-1: SystemInformationBlockType1 for cell 1 - as per ccombination 1 // Acc to Table 6.2.3.10.3.3-1: schedulingInfoList-v12j0 includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix) => SIB2+SIB24 => remove SIB3 in schedulingInfoList v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); // Add schedulingInfoList-v12j0 acc to Table 6.2.3.10.3.3-1 v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); //Modify SI list v_SIs := f_EUTRA_CellInfo_GetSysinfoSIs(eutra_Cell1); …………….. }

After Change:

function f_TC_6_2_3_10_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ……………….. //Modify SIB1 scheduling as per Table 6.2.3.10.3.3-1: SystemInformationBlockType1 for cell 1 - as per ccombination 1 // Acc to Table 6.2.3.10.3.3-1: schedulingInfoList-v12j0 includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix) => SIB2+SIB24 => remove SIB3 in schedulingInfoList v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); // Add schedulingInfoList-v12j0 acc to Table 6.2.3.10.3.3-1 if(px_ePrimaryFrequencyBand >= maxFBI){ v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(px_ePrimaryFrequencyBand, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); } else{ v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); } f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); //Modify SI list v_SIs := f_EUTRA_CellInfo_GetSysinfoSIs(eutra_Cell1); ………………. }

MCC160 proposed implementation: create a new functon f_EUTRA_CellInfo_SetSysInfo_SIB1_v12j0 and use it inside the test cases.

/*

* @desc      Assign the parameterised nonCriticalExtension SystemInformationBlockType1_v8h0_IEs value in SIB1 of the cell

* @param     p_CellId

*/

function f_EUTRA_CellInfo_SetSysInfo_SIB1_v12j0 ( EUTRA_CellId_Type p_CellId,

template (value) SystemInformationBlockType1_v12j0_IEs p_SystemInformationBlockType1_v12j0

) runs on EUTRA_PTC

{ //@sic R5s221143 sic@

var template(value) SystemInformationBlockType1_v8h0_IEs v_SIB1_v8h0;

var integer v_Band := f_EUTRA_CellInfo_GetBand(p_CellId);

if (v_Band >= maxFBI){

v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(v_Band, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(p_SystemInformationBlockType1_v12j0)))));

} else {

v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(p_SystemInformationBlockType1_v12j0)))));

}

f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(p_CellId, v_SIB1_v8h0);

}

//==========================================================================================

function f_TC_6_2_3_10_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC

{ // Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE / schedulingInfoList-v12j0

var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency;

var template(value) SI_List_Type  v_SIs;

var template(value) SchedulingInfoList_v12j0 v_SchedulingInfoList_v12j0_2SI := cs_SchedulingInfoList_v12j0_2SI;

var template(value) SystemInformationBlockType1_v12j0_IEs v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(v_SchedulingInfoList_v12j0_2SI);

f_EUTRA38_IRAT_Init(c31);  //Use combination c31 to have SI[2] already initialised with SIB24

// System information combination 1 as defined in TS 36.508 clause 4.4.3.1 is used in E-UTRA Cell 1.

//Set SIB24 as per Table 6.2.3.10.3.3-3: SystemInformationBlockType24 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.10.3.2-3)

//Update SIB24 first before changing the scheduling

f_EUTRA_CellInfo_SetSIB24_CellReselectionPriority_r15(eutra_Cell1, 3);

f_EUTRA_CellInfo_SetSIB24_ThreshX_Low_r15(eutra_Cell1, 10);

// Set SIB3 as per Table 6.2.3.10.3.3-2: SystemInformationBlockType3 of EUTRA Cell 1

f_EUTRA_CellInfo_SetSysInfo_ThreshServingLow (eutra_Cell1, 10);

//Modify SIB1 scheduling as per Table 6.2.3.10.3.3-1: SystemInformationBlockType1 for cell 1  - as per ccombination 1

// Acc to Table 6.2.3.10.3.3-1: schedulingInfoList-v12j0 includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix) => SIB2+SIB24 => remove SIB3 in schedulingInfoList

v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1));

f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity));

// Add schedulingInfoList-v12j0 acc to Table 6.2.3.10.3.3-1

f_EUTRA_CellInfo_SetSysInfo_SIB1_v12j0(eutra_Cell1, v_SystemInformationBlockType1_v12j0); //@sic R5s221143 sic@

//Modify SI list

v_SIs := f_EUTRA_CellInfo_GetSysinfoSIs(eutra_Cell1);

v_SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[1] :=   v_SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0];

v_SIs := {v_SIs[0], v_SIs[1]};

f_EUTRA_CellInfo_SetSysinfoSIs(eutra_Cell1, v_SIs);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

//Preamble: finishes with EUTRA cell on (NR cell off) and co-ord msg sent to say UE has completed registration on EUTRA

f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble");               // Wait for UE to register on NR first

f_EUTRA38_NR5GC_Preamble (eutra_Cell1, TESTMode_OFF, RRC_CONNECTED); // @sic R5-216166 sic@ Go on to EUTRA RRC_CONNECTED while NR is already off

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble"));    // inform NR side about registration on both sides

f_EUTRA_TestBody_Set(true);

fl_TC_6_2_3_10and11_NR5GC_EUTRA_TestBody ();

f_EUTRA_TestBody_Set(false);

// take down the cell now, UE finished postamble on NR side

f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble");

f_EUTRA_ReleaseAllCells();

}

## 2.2 Change 2

| Function name | f_TC_6_2_3_11_NR5GC_EUTRA |
| --- | --- |
| Reason for change | If EUTRA band is greater than maxFBI then "freqBandIndicator-v9e0" should be included in the SIB1. |
| Summary of change | Include "freqBandIndicator-v9e0" in SIB1 forEUTRA bands greater than or equal to maxFBI. |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted but implemented differently – see proposed implementation below similar as in Change 1 |

Before Change

function f_TC_6_2_3_11_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ……………. v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); //set schedulingInfoListExt-r12 in SIB1 with sib24 v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(omit, cs_SchedulingInfoListExt_1SI(v_ChannelBandwidthDependency.SIB24_periodicity,{sibType24_v1530})); v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit,cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); // Modify sysinfo UE off with the scheduling including the extended SI sib24 // The schuling in SIB1 includes only 2 SI, the extended scheduling include 1 SI - provide to SS all 3 SIs (in v_SiSchedul) f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul); ……………. }

After Change

function f_TC_6_2_3_11_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { …………… v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); //set schedulingInfoListExt-r12 in SIB1 with sib24 v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(omit, cs_SchedulingInfoListExt_1SI(v_ChannelBandwidthDependency.SIB24_periodicity,{sibType24_v1530})); if(px_ePrimaryFrequencyBand >= maxFBI){ v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(px_ePrimaryFrequencyBand, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); } else{ v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); } f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); // Modify sysinfo UE off with the scheduling including the extended SI sib24 // The schuling in SIB1 includes only 2 SI, the extended scheduling include 1 SI - provide to SS all 3 SIs (in v_SiSchedul) f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul); ……………………. }

MCC160 Implementation

function f_TC_6_2_3_11_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE / schedulingInfoListExt-r12 var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency; var template (value) AllSiSchedul_Type v_SiSchedul; var template (value) EUTRA_CellInfo_Type v_CellInfo; var SysinfoType_Type v_SysinfoType; var template(value) SystemInformationBlockType1_v12j0_IEs v_SystemInformationBlockType1_v12j0; // @sic R5s210433 sic@ f_EUTRA38_IRAT_Init(c31); // Use syinfo combination 31 to get SIB24 initialised automatically //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Set SIB24 as per Table 6.2.3.11.3.3-3: SystemInformationBlockType24 of E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.11.3.2-3) //Update SIB24 first before changing the scheduling f_EUTRA_CellInfo_SetSIB24_CellReselectionPriority_r15(eutra_Cell1, 3); f_EUTRA_CellInfo_SetSIB24_ThreshX_Low_r15(eutra_Cell1, 10); // Set SIB3 Table 6.2.3.11.3.3-2: SystemInformationBlockType3 of E-UTRA Cell 1 (preamble and all steps , Table 6.2.3.11.3.2-3) f_EUTRA_CellInfo_SetSysInfo_ThreshServingLow (eutra_Cell1, 10); //Retreive the SI scheduling ASP info with 3 SI: sib2, sib3, sib24 => to be used when reconfiguring SS with 3 SI v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1); v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(eutra_Cell1); v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(valueof(v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1), f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1), v_SysinfoType, valueof(v_CellInfo.Frequency.DL_ChBandwidth)); //Modify SIB1 scheduling as per Table 6.2.3.11.3.3-1: SystemInformationBlockType1 for cell 1 - as per ccombination 1: SI[0]=sib2 SI[1]=sib3 // Acc to Table 6.2.3.10.3.3-1: schedulingInfoList-v12j0 includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix) => SIB2+SIB24 => remove SIB3 in schedulingInfoList v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); //set schedulingInfoListExt-r12 in SIB1 with sib24 v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(omit, cs_SchedulingInfoListExt_1SI(v_ChannelBandwidthDependency.SIB24_periodicity,{sibType24_v1530})); f_EUTRA_CellInfo_SetSysInfo_SIB1_v12j0(eutra_Cell1, v_SystemInformationBlockType1_v12j0); //@sic R5s221143 sic@ // Modify sysinfo UE off with the scheduling including the extended SI sib24 // The schuling in SIB1 includes only 2 SI, the extended scheduling include 1 SI - provide to SS all 3 SIs (in v_SiSchedul) f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul); //Preamble: finishes with EUTRA cell on (NR cell off) and co-ord msg sent to say UE has completed registration on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Wait for UE to register on NR first f_EUTRA38_NR5GC_Preamble (eutra_Cell1, TESTMode_OFF, RRC_CONNECTED); // @sic R5-216166 sic@ Go on to EUTRA RRC_CONNECTED while NR is already off f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); // inform NR side about registration on both sides f_EUTRA_TestBody_Set(true); fl_TC_6_2_3_10and11_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }