3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221146

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2787 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 7.1.1.4.2.x test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current configuration have RLC poll retransmit timer set to 80msec (38.508-1 default settings) and the SR repitition set to 40msec. The SS is also configured to provide Uplink grant for the DL data based on the second SR reception. SS DL data when transmitted will not receive RLC ACK until the second SR reception is received and UL grant is provided to send the loopback data along with the RLC ACK. This could lead to a situation where the RLC retransmission may happen if the differnce between sending of DL data and second SR reception is around 80msec. In order to avoid RLC retransmission it is proposed to change the t-PollRetransmit to 160msec The changes require an update to 38.523-1 and a draft prose CR is provided |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the ttcn code to set the t-PollRetransmit to 160msec |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.1.NR5GC, 7.1.1.4.2.3.NR5GC, 7.1.1.4.2.4.NR5GC, 7.1.1.4.2.5.NR5GC, 7.1.1.4.2.1.ENDC, 7.1.1.4.2.3.ENDC, 7.1.1.4.2.4.ENDC, 7.1.1.4.2.5. .ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required Uplink TBS test cases	2

2.1	Correction to f_TC_7_1_1_4_2_1_ENDC_NR	2

2.2	Correction to f_TC_7_1_1_4_2_3_ENDC_NR	5

2.3	Correction to f_TC_7_1_1_4_2_4_ENDC_NR	7

2.4	Correction to f_TC_7_1_1_4_2_5_ENDC_NR	10

2.5	Correction to f_TC_7_1_1_4_2_1_NR5GC()	12

2.6	Correction to f_TC_7_1_1_4_2_3_NR5GC	14

2.7	Correction to f_TC_7_1_1_4_2_4_NR5GC	17

2.8	Correction to f_TC_7_1_1_4_2_5_NR5GC	20

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required Uplink TBS test cases

## Correction to f_TC_7_1_1_4_2_1_ENDC_NR

| Function name | f_TC_7_1_1_4_2_1_ENDC_NR |
| --- | --- |
| Reason for change | The current configuration have RLC poll retransmit timer set to 80msec (38.508-1 default settings) and the SR repitition set to 40msec. The SS is also configured to provide Uplink grant for the DL data based on the second SR reception. SS DL data when transmitted will not receive RLC ACK until the second SR reception is received and UL grant is provided to send the loopback data along with the RLC ACK. This could lead to a situation where the RLC retransmission may happen if the differnce between sending of DL data and second SR reception is around 80msec. In order to avoid RLC retransmission it is proposed to change the t-PollRetransmit to 160msec |
| Summary of change | v_T_PollRetransmit is updated to 160msec and v_SS_Drb_ConfigList,v_RLC_BearerConfigList is updated and provided to SS and peer message to configure v_T_PollRetransmit in f_NR_ENDC_ConfigAM_UM |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |
| MCC 160 Comment R1 | Accepted as improvement based on approved prose CR R5-227416 |

Before Change:

function f_TC_7_1_1_4_2_1_ENDC_NR() runs on ENDC_NR_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711421; //Init Cell parameters f_ENDC_NR_Init(); f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_0_And_1_0, omit, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_4_2_1_ENDC_NR() runs on ENDC_NR_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711421; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms160; // (changed timer poll to 160 from 80 / var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DL_t_Reassembly_Type v_T_Reassembly; //Init Cell parameters f_ENDC_NR_Init(); f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_0_And_1_0, omit, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, tsc_NR_DRB2, -, -, -, -,-,v_T_PollRetransmit)}; //(changed timer poll to 160 from 80 / v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(tsc_NR_DRB2, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0,v_SS_Drb_ConfigList,v_RLC_BearerConfigList); //(changed timer poll to 160 from 80 / // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

## Correction to f_TC_7_1_1_4_2_3_ENDC_NR

| Function name | f_TC_7_1_1_4_2_3_ENDC_NR |
| --- | --- |
| Reason for change | Same as Change 1.1 |
| Summary of change | v_T_PollRetransmit is updated to 160msec and v_SS_Drb_ConfigList,v_RLC_BearerConfigList is updated and provided to SS and peer message to configure v_T_PollRetransmit in f_NR_ENDC_ConfigAM_UM |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |
| MCC 160 Comment R1 | Accepted as improvement based on approved prose CR R5-227416 |

Before Change:

function f_TC_7_1_1_4_2_3_ENDC_NR() runs on ENDC_NR_PTC { // 7.1.1.4.2.3 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711423; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_ENDC_NR_Init(); /* if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0; } */ //@sic R5s201479 sic@ f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, omit, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true);

After Change:

function f_TC_7_1_1_4_2_3_ENDC_NR() runs on ENDC_NR_PTC { // 7.1.1.4.2.3 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711423; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms160; //WA#WI=978604 (changed timer poll to 160 from 80 / var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DL_t_Reassembly_Type v_T_Reassembly; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_ENDC_NR_Init(); /* if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0; } */ //@sic R5s201479 sic@ f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, omit, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, tsc_NR_DRB2, -, -, -, -,-,v_T_PollRetransmit)}; // (changed timer poll to 160 from 80 / v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(tsc_NR_DRB2, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0,v_SS_Drb_ConfigList,v_RLC_BearerConfigList); //(changed timer poll to 160 from 80 / // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true);

## Correction to f_TC_7_1_1_4_2_4_ENDC_NR

| Function name | f_TC_7_1_1_4_2_4_ENDC_NR |
| --- | --- |
| Reason for change | Same as change 1.1 |
| Summary of change | v_T_PollRetransmit is updated to 160msec and v_SS_Drb_ConfigList,v_RLC_BearerConfigList is updated and provided to SS and peer message to configure v_T_PollRetransmit in f_NR_ENDC_ConfigAM_UM |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |
| MCC 160 Comment R1 | Accepted as improvement based on approved prose CR R5-227416 |

Before Change:

function f_TC_7_1_1_4_2_4_ENDC_NR() runs on ENDC_NR_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711424; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //Init Cell parameters f_ENDC_NR_Init(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0; } f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true);

After Change:

function f_TC_7_1_1_4_2_4_ENDC_NR() runs on ENDC_NR_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711424; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms160; //(changed timer poll to 160 from 80 / var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DL_t_Reassembly_Type v_T_Reassembly; //Init Cell parameters f_ENDC_NR_Init(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0; } f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, tsc_NR_DRB2, -, -, -, -,-,v_T_PollRetransmit)}; // (changed timer poll to 160 from 80 / v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(tsc_NR_DRB2, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0,v_SS_Drb_ConfigList,v_RLC_BearerConfigList); //(changed timer poll to 160 from 80 / // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true);

## Correction to f_TC_7_1_1_4_2_5_ENDC_NR

| Function name | f_TC_7_1_1_4_2_5_ENDC_NR |
| --- | --- |
| Reason for change | Same as change 1.1 |
| Summary of change | v_T_PollRetransmit is updated to 160msec and v_SS_Drb_ConfigList,v_RLC_BearerConfigList is updated and provided to SS and peer message to configure v_T_PollRetransmit in f_NR_ENDC_ConfigAM_UM |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |
| MCC 160 Comment R1 | Accepted as improvement based on approved prose CR R5-227416 |

Before Change:

function f_TC_7_1_1_4_2_5_ENDC_NR() runs on ENDC_NR_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711425; //Init Cell parameters f_ENDC_NR_Init(); f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_0_And_1_0, omit, omit,enabled);//@sic R5s201144 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters, 27, true); //@sic R5s201144 sic@ f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_4_2_5_ENDC_NR() runs on ENDC_NR_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711425; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms160; (changed timer poll to 160 from 80 / var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DL_t_Reassembly_Type v_T_Reassembly; //Init Cell parameters f_ENDC_NR_Init(); f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_0_And_1_0, omit, omit,enabled);//@sic R5s201144 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, tsc_NR_DRB2, -, -, -, -,-,v_T_PollRetransmit)}; (changed timer poll to 160 from 80 / v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(tsc_NR_DRB2, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0,v_SS_Drb_ConfigList,v_RLC_BearerConfigList); // (changed timer poll to 160 from 80 / // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters, 27, true); //@sic R5s201144 sic@ f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

## Correction to f_TC_7_1_1_4_2_1_NR5GC()

| Function name | f_TC_7_1_1_4_2_1_NR5GC() |
| --- | --- |
| Reason for change | Same as change 1.1 |
| Summary of change | v_T_PollRetransmit is updated to 160msec and v_SS_Drb_ConfigList,v_RLC_BearerConfigList is updated and provided to SS and peer message v_RLC_BearerConfigList is passed as a parameter to f_NR5GC_RRC_ConnectedState3N_Def |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |
| MCC 160 Comment R1 | Accepted as improvement based on approved prose CR R5-227416 |

Before Change:

function f_TC_7_1_1_4_2_1_NR5GC() runs on NR5GC_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled var DRB_Identity v_DRBId ; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711421; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s190826 sic@ //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_0_And_1_0, omit, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //@sic R5s190827 sic@ v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)}; //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

After Change:

function f_TC_7_1_1_4_2_1_NR5GC() runs on NR5GC_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled var DRB_Identity v_DRBId ; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711421; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s190826 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms160; // (changed timer poll to 160 from 80 / var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DL_t_Reassembly_Type v_T_Reassembly; //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_0_And_1_0, omit, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //@sic R5s190827 sic@ v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None,-,v_T_PollRetransmit)}; //WA#WI=978604 (changed timer poll to 160 from 80 / v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); //(changed timer poll to 160 from 80 / //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime v_RLC_BearerConfigList, // (changed timer poll to 160 from 80 / -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

## Correction to f_TC_7_1_1_4_2_3_NR5GC

| Function name | f_TC_7_1_1_4_2_3_NR5GC |
| --- | --- |
| Reason for change | Same change as 1.1 |
| Summary of change | v_T_PollRetransmit is updated to 160msec and v_SS_Drb_ConfigList,v_RLC_BearerConfigList is updated and provided to SS and peer message v_RLC_BearerConfigList is passed as a parameter to f_NR5GC_RRC_ConnectedState3N_Def |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |
| MCC 160 Comment R1 | Accepted as improvement based on approved prose CR R5-227416 |

Before Change:

function f_TC_7_1_1_4_2_3_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.3 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled var DRB_Identity v_DRBId ; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711423; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_NR5GC_Init(NR_1); //if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0;}; //@sic R5s200421 sic@ f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, omit, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)}; //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

After Change:

function f_TC_7_1_1_4_2_3_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.3 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled var DRB_Identity v_DRBId ; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711423; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms160; // (changed timer poll to 160 from 80 / var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DL_t_Reassembly_Type v_T_Reassembly; //Init Cell parameters f_NR5GC_Init(NR_1); //if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0;}; //@sic R5s200421 sic@ f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, omit, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None,-,v_T_PollRetransmit)}; // (changed timer poll to 160 from 80 / v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime v_RLC_BearerConfigList, //(changed timer poll to 160 from 80 / -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

## Correction to f_TC_7_1_1_4_2_4_NR5GC

| Function name | f_TC_7_1_1_4_2_4_NR5GC |
| --- | --- |
| Reason for change | Same change as 1.1 |
| Summary of change | v_T_PollRetransmit is updated to 160msec and v_SS_Drb_ConfigList,v_RLC_BearerConfigList is updated and provided to SS and peer message v_RLC_BearerConfigList is passed as a parameter to f_NR5GC_RRC_ConnectedState3N_Def |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |
| MCC 160 Comment R1 | Accepted as improvement based on approved prose CR R5-227416 |

Before Change:

function f_TC_7_1_1_4_2_4_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var DRB_Identity v_DRBId; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711424; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //Init Cell parameters f_NR5GC_Init(NR_1); //if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0;}; //@sic R5s200421 sic@ f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)}; //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

After Change:

function f_TC_7_1_1_4_2_4_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var DRB_Identity v_DRBId; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711424; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms160; // (changed timer poll to 160 from 80 / var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DL_t_Reassembly_Type v_T_Reassembly; //Init Cell parameters f_NR5GC_Init(NR_1); //if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0;}; //@sic R5s200421 sic@ f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None,-,v_T_PollRetransmit)}; // (changed timer poll to 160 from 80 / v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime v_RLC_BearerConfigList, // (changed timer poll to 160 from 80 / -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

## Correction to f_TC_7_1_1_4_2_5_NR5GC

| Function name | f_TC_7_1_1_4_2_5_NR5GC |
| --- | --- |
| Reason for change | Same change as 1.1 |
| Summary of change | v_T_PollRetransmit is updated to 160msec and v_SS_Drb_ConfigList,v_RLC_BearerConfigList is updated and provided to SS and peer message v_RLC_BearerConfigList is passed as a parameter to f_NR5GC_RRC_ConnectedState3N_Def |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |
| MCC 160 Comment R1 | Accepted as improvement based on approved prose CR R5-227416 |

Before Change:

function f_TC_7_1_1_4_2_5_NR5GC() runs on NR5GC_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM var DRB_Identity v_DRBId ; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711425; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_0_And_1_0, omit, omit,enabled); //@sic R5s201144 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)}; //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

After Change:

function f_TC_7_1_1_4_2_5_NR5GC() runs on NR5GC_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM var DRB_Identity v_DRBId ; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711425; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := ms160; //WA#WI=978604 (changed timer poll to 160 from 80 / var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DL_t_Reassembly_Type v_T_Reassembly; //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_0_And_1_0, omit, omit,enabled); //@sic R5s201144 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None,-,v_T_PollRetransmit)}; //(changed timer poll to 160 from 80 / v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime v_RLC_BearerConfigList, // (changed timer poll to 160 from 80 / -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);