3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221154

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4693 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE measurement testcase 8.3.1.3a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 3A, systemInfoValueTag IE in SIB1 is increased to indicate modification of SIB3, but systemInfoValueTagList_r13 IE is not updated to indicate the same for CAT-M1 UEs. The value of the second position in the systemInfoValueTagList_r13 IE needs to be increased, to indicate modification of SIB3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | systemInfoValueTagList_r13 IE is updated to indicate modification of SIB3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant CAT-M1 UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_8_3_1_3a_EUTRA |
| --- | --- |
| Reason for change | At Step 3A, systemInfoValueTag IE in SIB1 is increased to indicate modification of SIB3, but systemInfoValueTagList_r13 IE is not updated to indicate the same for CAT-M1 UEs. The value of the second position in the systemInfoValueTagList_r13 IE needs to be increased, to indicate modification of SIB3. |
| Summary of change | The value of systemInfoValueTagList_r13[1] is increased. |
| TTCN module | LTE\8_3\RRC_Measurement.ttcn |
| MCC160 Comment | Accepted, but Prose CR needed |

Before change:

function f_TC_8_3_1_3a_EUTRA() runs on EUTRA_PTC

{ //Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A3 (intra and inter-frequency measurements) / RSRQ based measurements

var integer v_T0_RS_EPRE_Cell1 := -85;

var integer v_T1_RS_EPRE_Cell1 := -91; //@sic R5-126026 sic@

var integer v_T0_RS_EPRE_Cell2 := -91;

var integer v_T1_RS_EPRE_Cell2 := -85; //@sic R5-126026 sic@

var integer v_T2_RS_EPRE_Cell2 := tsc_NonSuitableOffCellRS_EPRE;

var integer v_T2_RS_EPRE_Cell3 := -85; //@sic R5-134457 sic@

var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def;

var template(value) CellPowerList_Type v_CellPowerList;

var PhysCellId v_PhysCellId_Cell2;

var PhysCellId v_PhysCellId_Cell3;

var Frequency_IE_Type v_Frequency_IE_Cell2;

var Frequency_IE_Type v_Frequency_IE_Cell3;

var ChannelBandwidthDependency_Type v_ChBandDependency_Cell2;

var ChannelBandwidthDependency_Type v_ChBandDependency_Cell3;

var float v_TimerValue1;

var boolean v_FeatrGrp_25; //@sic R5-192720 sic@

var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; //@sic R5-192720 sic@

//<=== SKIPPED ===>

//@siclog "Step 3A" siclog@

//The SS notifies the UE of change of System Information and changes the system information 3. The systemInfoValueTag in the SystemInformationBlockType1 is increased.

f_EUTRA_CellInfo_SetSysInfoSIB3_Q_QualMin_r9(eutra_Cell1, -30); //@sic R5-126026 sic@

f_EUTRA_ModifySysinfo (eutra_Cell1);

//<=== SKIPPED ===>

} //end of testcase 8.3.1.3a

After change:

function f_TC_8_3_1_3a_EUTRA() runs on EUTRA_PTC

{ //Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A3 (intra and inter-frequency measurements) / RSRQ based measurements

var integer v_T0_RS_EPRE_Cell1 := -85;

var integer v_T1_RS_EPRE_Cell1 := -91; //@sic R5-126026 sic@

var integer v_T0_RS_EPRE_Cell2 := -91;

var integer v_T1_RS_EPRE_Cell2 := -85; //@sic R5-126026 sic@

var integer v_T2_RS_EPRE_Cell2 := tsc_NonSuitableOffCellRS_EPRE;

var integer v_T2_RS_EPRE_Cell3 := -85; //@sic R5-134457 sic@

var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def;

var template(value) CellPowerList_Type v_CellPowerList;

var PhysCellId v_PhysCellId_Cell2;

var PhysCellId v_PhysCellId_Cell3;

var Frequency_IE_Type v_Frequency_IE_Cell2;

var Frequency_IE_Type v_Frequency_IE_Cell3;

var ChannelBandwidthDependency_Type v_ChBandDependency_Cell2;

var ChannelBandwidthDependency_Type v_ChBandDependency_Cell3;

var float v_TimerValue1;

var boolean v_FeatrGrp_25; //@sic R5-192720 sic@

var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; //@sic R5-192720 sic@

var template (value) EUTRA_CellInfo_Type v_CellInfo;

//<=== SKIPPED ===>

//@siclog "Step 3A" siclog@

//The SS notifies the UE of change of System Information and changes the system information 3. The systemInfoValueTag in the SystemInformationBlockType1 is increased.

f_EUTRA_CellInfo_SetSysInfoSIB3_Q_QualMin_r9(eutra_Cell1, -30); //@sic R5-126026 sic@

if (pc_ue_CategoryDL_M1) {

v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1);     v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.systemInfoValueTagList_r13[1] := 1;

f_EUTRA_CellInfo_Set(eutra_Cell1, v_CellInfo);

}

f_EUTRA_ModifySysinfo (eutra_Cell1);

//<=== SKIPPED ===>

} //end of testcase 8.3.1.3a