3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221158

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0871 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS Supplementary Services testcase 8.40 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 8.40 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS Supplementary Services testcase 8.40 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.40.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

2 Corrections required	3

3 Branches executed	3

4 Execution Log Files	3

4.1 HiSilicon	3

5 References	3

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.40 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Ting Gao

gaoting@starpointcomm.com

Verification Test Summary

Test Case: 	8.40

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	Starpoint SP9500 Test System

UE used:	Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required

None.

## Additional changes:

| Function | fl_IMS_USSD_RequestURI |
| --- | --- |
| Reason for change | According to 24.390 the USSD string and the phone-context parameter need to be treated as tel-URI in terms of RFC 3261 clause 19.1.6 i.e. the phone-context parameter is part of the SIP-URI's user part. |
| Summary of change | New local function to encode the tel-URI |
| TTCN module | IMS_Procedures_SupplementaryServices |

function fl_IMS_USSD_SipUri_UserPart(charstring p_UssdString, charstring p_HomeDomainName, URL_Encoding_Type p_EncodingType) return charstring { return f_URL_Encoding(p_UssdString, p_EncodingType) & ";phone-context=" & p_HomeDomainName; }

function fl_IMS_USSD_RequestURI(charstring p_UssdString) runs on IMS_PTC return template (present) SipUrl { var charstring v_HomeDomainName := f_IMS_PTC_ImsInfo_GetHomeDomainName(); var template (present) GenericParam v_UserParam := cr_GenericParam_Common("user", cr_GenValueToken("dialstring")); var template (present) SemicolonParam_List v_Params := superset(v_UserParam); var template (present) charstring v_UserName := (fl_IMS_USSD_SipUri_UserPart(p_UssdString, v_HomeDomainName, urlEncodingRFC3986), fl_IMS_USSD_SipUri_UserPart(p_UssdString, v_HomeDomainName, urlEncodingRFC2396)); var template charstring v_Password := omit; var template charstring v_Host := v_HomeDomainName; var template (present) UriComponents v_UriComponents := cr_SipUriComponent(cr_UserInfo(v_UserName, v_Password), v_Host); return cr_SipUrl_Common(c_sipScheme, v_UriComponents, v_Params); }

| Function | f_IMS_USSD_MOCallSetup |
| --- | --- |
| Reason for change | recvInfo and accept assigned to the wrong variable |
| Summary of change | recvInfo and accept assigned to v_MessageHeader_200OK |
| TTCN module | IMS_Procedures_SupplementaryServices |

function f_IMS_USSD_MOCallSetup(charstring p_UssdString) runs on IMS_PTC { var IMS_DATA_REQ v_IMS_DATA_REQ; var INVITE_Request v_InviteRequest; var MIME_Message v_MIME_Message; var MIME_Part v_MIME_Part; var SDP_Message v_SDP_Offer; var template (present) SipUrl v_RequestURI := fl_IMS_USSD_RequestURI(p_UssdString); var template (present) MessageHeader v_MessageHeader_Invite; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeader_200OK; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) SDP_Message v_SDP_MessageTX; var boolean v_SessionTimerNeeded := not f_IMS_PTC_RanTypeIsNR5GC(); // @sic R5-223475 for 5G, no similar CR for 4G sic@ // Receive INVITE v_MessageHeader_Invite := f_IMS_InviteRequest_MessageHeaderRX(v_RequestURI, -, -, -, -, -, v_SessionTimerNeeded); v_MessageHeader_Invite.recvInfo := cr_RecvInfo_Def("g.3gpp.ussd"); v_MessageHeader_Invite.accept := cr_Accept({cr_AcceptBody(tsc_MimeType_Ussd), cr_AcceptBody(c_sdpApplication), cr_AcceptBody(tsc_MimeType_MultipartMixed)}); v_MessageHeader_Invite.contentType := cr_ContentTypeMIME; v_IMS_DATA_REQ := valueof(f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(A_2_1_A4, v_RequestURI, v_MessageHeader_Invite)); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; v_MIME_Message := v_InviteRequest.messageBody.mimeMessageBody; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); // Check SDP offer v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, c_sdpApplication); v_SDP_Offer := f_IMS_SIP_DecodeMatchAndCheckSDP(v_MIME_Part.body.sdpMessageBody, fl_IMS_USSD_BuildSDP_OfferRX()); // Check USSD v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, tsc_MimeType_Ussd); fl_IMS_USSD_Message_DecodeAndMatch(v_MIME_Part.body.xmlBody, cr_UssdData(-, p_UssdString)); // Send 100 Trying IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine100, f_IMS_InviteResponse_100_MessageHeaderTX(v_InviteRequest)))); // Send 200 OK v_MessageHeader_200OK := f_IMS_OtherResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, cs_ContentTypeSDP); v_MessageHeader_200OK.recvInfo := cs_RecvInfo_Def("g.3gpp.ussd"); v_MessageHeader_200OK.accept := cs_Accept({cs_AcceptBody(tsc_MimeType_Ussd), cs_AcceptBody(c_sdpApplication), cs_AcceptBody(tsc_MimeType_MultipartMixed)}); v_SDP_MessageTX := fl_IMS_USSD_BuildSDP_AnswerTX(v_SDP_Offer); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader_200OK, cs_MessageBody_SDP(v_SDP_MessageTX)))); // Receive ACK v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine200); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_RequestURI, v_MessageHeader_Ack))); }

# Branches executed

The test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## HiSilicon

The Hisilicon Balong 5000 UE passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_40_NR5GC_n41_PASS.spm

- PICS/PIXIT parameter file:

PCT_PICS_PIXIT.sppar

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221159：Supporting information for addition of Supplementary Services testcase 8.40 in FR1. |
| --- | --- |