3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221171

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2801 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR5GC UAC test case 11.3.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following looping back the IP PDU at step 2, the UE PDCP SDU buffer will be emptied. Following this, any subsequent PDUs will be looped back immediately with no delay or buffering, according to 36.509 5.4.4.3: NOTE 2: After the PDCP SDU buffer becomes empty the loopback will return any received PDCP SDU in uplink directly as specified in clause 5.4.4.2. In order to reactivate the loopback delay and PDCP SDU buffering the SS shall deactivate UE test loop B function first. This means that following step 5Ab6, the IP PDU may be looped back before the SS can put the UE into RRC_INACTIVE state, failing the test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Open and then close the UE test loop before step 5Ab6, so that the IP PDU delay is re-applied for the subsequent loopback steps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1 Overview	3

2 Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | fl_TC_11_3_1a_TestBody |
| --- | --- |
| Reason for change | Following looping back the IP PDU at step 2, the UE PDCP SDU buffer will be emptied. Following this, any subsequent PDUs will be looped back immediately with no delay or buffering, according to 36.509 5.4.4.3: NOTE 2: After the PDCP SDU buffer becomes empty the loopback will return any received PDCP SDU in uplink directly as specified in clause 5.4.4.2. In order to reactivate the loopback delay and PDCP SDU buffering the SS shall deactivate UE test loop B function first. This means that following step 5Ab6, the IP PDU may be looped back before the SS can put the UE into RRC_INACTIVE state, failing the test case. A prose cr will be submitted in the next RAN5 meeting. |
| Summary of change | Open and then close the UE test loop before step 5Ab6, so that the IP PDU delay is re-applied for the subsequent loopback steps. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#97 |

Before Change

function fl_TC_11_3_1a_TestBody() runs on NR5GC_PTC { var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var octetstring v_EncodedRrcSetup; var template(value) DL_CCCH_Message v_RrcSetup; var float v_TimerValue := 6.0; var float v_TimerValue2 := 30.0; timer t_WaitTime; //@siclog "Step 1" siclog@ //SS changes SIB1 according to Table 11.3.1a.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(7,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 2" siclog@ //The SS transmits one IP PDU. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, v_SDAP_Data)); //@siclog "Step 3" siclog@ //The SS transmits an RRCRelease message with suspendConfig IE and move the UE to RRC_Inactive state. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 4 siclog@ //Check: Does the UE transmit RRCResumeRequest message including mo-Data as resume cause within 6s? t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Data))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } [] t_WaitTime.timeout {} } //@siclog "Step 5" siclog@ //SS changes SIB1 according to Table 11.3.1a.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(4,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // f_NR_WaitModificationPeriods(nr_Cell1); @sic R5s220313 sic@ //@sic R5-222048 sic@ //@siclog "Step 5A - 5Ab8" siclog@ //Start Timer=30 sec. (Note 2) //EXCEPTION: Steps 5Aa1-5Ab8 describe optional behaviour that depends on the UE implementation. t_WaitTime.start(v_TimerValue2); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Data))) { t_WaitTime.stop; } [] t_WaitTime.timeout { //@sic R5-213502 sic@ //@siclog "Step 5Ab2" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 5Ab3" siclog@ //The UE transmits an RRCResumeRequest message including resumeCause as mt-Access. SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mt_Access))); //@siclog "Step 5Ab4" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 5Ab5" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 5Ab6" siclog@ //The SS transmits one IP PDU. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, v_SDAP_Data)); //@siclog "Step 5Ab7" siclog@ //The SS transmits an RRCRelease message with suspendConfig IE and move the UE to RRC_Inactive state. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 5Ab8" siclog@ //The UE transmits RRCResumeRequest message including mo-Data as resume cause. SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Data))); } } ... }

After Change

function fl_TC_11_3_1a_TestBody() runs on NR5GC_PTC { var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var octetstring v_EncodedRrcSetup; var template(value) DL_CCCH_Message v_RrcSetup; var float v_TimerValue := 6.0; var float v_TimerValue2 := 30.0; timer t_WaitTime; //@siclog "Step 1" siclog@ //SS changes SIB1 according to Table 11.3.1a.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(7,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 2" siclog@ //The SS transmits one IP PDU. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, v_SDAP_Data)); //@siclog "Step 3" siclog@ //The SS transmits an RRCRelease message with suspendConfig IE and move the UE to RRC_Inactive state. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 4 siclog@ //Check: Does the UE transmit RRCResumeRequest message including mo-Data as resume cause within 6s? t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Data))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } [] t_WaitTime.timeout {} } //@siclog "Step 5" siclog@ //SS changes SIB1 according to Table 11.3.1a.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(4,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // f_NR_WaitModificationPeriods(nr_Cell1); @sic R5s220313 sic@ //@sic R5-222048 sic@ //@siclog "Step 5A - 5Ab8" siclog@ //Start Timer=30 sec. (Note 2) //EXCEPTION: Steps 5Aa1-5Ab8 describe optional behaviour that depends on the UE implementation. t_WaitTime.start(v_TimerValue2); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Data))) { t_WaitTime.stop; } [] t_WaitTime.timeout { //@sic R5-213502 sic@ //@siclog "Step 5Ab2" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 5Ab3" siclog@ //The UE transmits an RRCResumeRequest message including resumeCause as mt-Access. SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mt_Access))); //@siclog "Step 5Ab4" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 5Ab5" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); f_NR5GC_OpenUE_TestLoopMode(nr_Cell1); f_NR5GC_CloseUE_TestLoopModeB(nr_Cell1, '02'O); //@siclog "Step 5Ab6" siclog@ //The SS transmits one IP PDU. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, v_SDAP_Data)); //@siclog "Step 5Ab7" siclog@ //The SS transmits an RRCRelease message with suspendConfig IE and move the UE to RRC_Inactive state. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 5Ab8" siclog@ //The UE transmits RRCResumeRequest message including mo-Data as resume cause. SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Data))); } } ... }