3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221175

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2804 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 11.4.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, after Step29a1, a FOR loop is implemented in which function f_IP_ChangeDrbMappingNR(IP, nr_Cell1, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); is called. The problem here is that v_PduSessionInfoList[i].QFI is set to OMIT when Eutra sends the PDU Session Info List to NR. This causes run time error. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reworked the for loop, and ensured that proper QFI is assigned in order to fix the run time error. QFIs are assigned according to the PDU Sessions established earlier (PDU Session for IMS uses QFI 2 and PDU Session for emergency uses QFI 1). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1	Correction to fl_TC_11_4_11_NR5GC_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Correction to fl_TC_11_4_11_NR5GC_TestBody

| Function name | fl_TC_11_4_11_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, after Step29a1, a FOR loop is implemented in which function f_IP_ChangeDrbMappingNR(IP, nr_Cell1, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); is called. The problem here is that v_PduSessionInfoList[i].QFI is set to OMIT when Eutra sends the PDU Session Info List to NR. This causes run time error. This needs to be addressed |
| Summary of change | Reworked the for loop, and ensured that proper QFI is assigned in order to fix the run time error. QFIs are assigned according to the PDU Sessions established earlier (PDU Session for IMS uses QFI 2 and PDU Session for emergency uses QFI 1). |
| TTCN module | Emergency_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment | Alternative solution implemented |

Before Change:

function fl_TC_11_4_11_NR5GC_TestBody() runs on NR5GC_PTC { var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var template (present) NG_Request_Type v_RequestType := (cr_NG_Request_Type('010'B), cr_NG_Request_Type('100'B)); var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var O1_Type v_EmergencySessionId; var template (value) NG_PDU_SessionId v_SessionId; var integer i; //@siclog "Step 17" siclog@ f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); // @sic R5s220883 sic@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 18-28" siclog@ f_NR5GC_IRAT_MobilityRegistration (nr_Cell1, v_NAS_SecurityParams, omit, f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams), f_LastVisited_TAI(nr_Cell1), -, -, noRrcConnectionRelease, -, cds_NG_NetworkFeatureSupport_NoN26); // Tell IMS PTCs the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); // always wait for PDU info, even if not 1st time v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); // This function stores all received PDN Connectivity msgs, so just expect one more... //@siclog "Step 29a1" siclog@ f_NR5GC_PDUSessionEstablishment (nr_Cell1, (pc_noOf_PDUsSameConnection+1), tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, -, -, -, v_PduSessionInfoList); // make sure all the PDNs configured on EUTRA are changed in routing table @sic R5s210149, R5s220883 sic@ for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { f_IP_ChangeDrbMappingNR(IP, nr_Cell1, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); if (match (v_PduSessionInfoList[i].PdnIndex, f_GetMultiPdnIndex(Emergency_PDN))) { // Keep note of this for later checks v_EmergencySessionId := int2oct(v_PduSessionInfoList[i].PDU_SessionId, 1); } } // ...then check one of received msgs is for the emergency bearer // @siclog "Step 1 Parallel Behaviour" siclog@ for (i := 1; i <= f_NR5GC_MsgInDefault_GetNumOfNASMessageReq(); i := i+1){ // @sic R5s220174 sic@ v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i); if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('100'B), omit, omit)) { // Emergency bearer v_SessionId := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionId; if (match (v_EmergencySessionId, v_SessionId.sessionId)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Parallel Behaviour Step 1"); } // transfer of existing bearer } else if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -,-, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('010'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Request Type neither existing nor emergency PDU session"); } } // @siclog "Step 30" siclog@ f_Delay(3.0); // tell IMS stop waiting for Re-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Stop Waiting For Reregister")); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Stop Waiting For Reregister")); // @siclog "Step 31-33" siclog@ f_NR5GC_PDUSessionRelease (nr_Cell1, v_EmergencySessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); // @siclog "Step 34" siclog@ f_NR_RRCRelease(nr_Cell1); }

After Change:

function fl_TC_11_4_11_NR5GC_TestBody() runs on NR5GC_PTC { var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var template (present) NG_Request_Type v_RequestType := (cr_NG_Request_Type('010'B), cr_NG_Request_Type('100'B)); var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var O1_Type v_EmergencySessionId; var template (value) NG_PDU_SessionId v_SessionId; var integer i; //@siclog "Step 17" siclog@ f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); // @sic R5s220883 sic@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 18-28" siclog@ f_NR5GC_IRAT_MobilityRegistration (nr_Cell1, v_NAS_SecurityParams, omit, f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams), f_LastVisited_TAI(nr_Cell1), -, -, noRrcConnectionRelease, -, cds_NG_NetworkFeatureSupport_NoN26); // Tell IMS PTCs the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); // always wait for PDU info, even if not 1st time v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); // This function stores all received PDN Connectivity msgs, so just expect one more... //@siclog "Step 29a1" siclog@ f_NR5GC_PDUSessionEstablishment (nr_Cell1, (pc_noOf_PDUsSameConnection+1), tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, -, -, -, v_PduSessionInfoList); // make sure all the PDNs configured on EUTRA are changed in routing table @sic R5s210149, R5s220883 sic@ for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { //WA#11_4_11 MOVED f_IP_ChangeDrbMappingNR(IP, nr_Cell1, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); if (match (v_PduSessionInfoList[i].PdnIndex, f_GetMultiPdnIndex(Emergency_PDN))) { // Keep note of this for later checks v_EmergencySessionId := int2oct(v_PduSessionInfoList[i].PDU_SessionId, 1); v_PduSessionInfoList[i].QFI := 1; //WA#11_4_11 } if (match (v_PduSessionInfoList[i].PdnIndex, f_GetMultiPdnIndex(IMS_DNN))) { //WA#11_4_11 : New IF statement // Keep note of this for later checks v_EmergencySessionId := int2oct(v_PduSessionInfoList[i].PDU_SessionId, 1); v_PduSessionInfoList[i].QFI := 2; } f_IP_ChangeDrbMappingNR(IP, nr_Cell1, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); } // ...then check one of received msgs is for the emergency bearer // @siclog "Step 1 Parallel Behaviour" siclog@ for (i := 1; i <= f_NR5GC_MsgInDefault_GetNumOfNASMessageReq(); i := i+1){ // @sic R5s220174 sic@ v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i); if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('100'B), omit, omit)) { // Emergency bearer v_SessionId := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionId; if (match (v_EmergencySessionId, v_SessionId.sessionId)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Parallel Behaviour Step 1"); } // transfer of existing bearer } else if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -,-, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('010'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Request Type neither existing nor emergency PDU session"); } } // @siclog "Step 30" siclog@ f_Delay(3.0); // tell IMS stop waiting for Re-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Stop Waiting For Reregister")); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Stop Waiting For Reregister")); // @siclog "Step 31-33" siclog@ f_NR5GC_PDUSessionRelease (nr_Cell1, v_EmergencySessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); // @siclog "Step 34" siclog@ f_NR_RRCRelease(nr_Cell1); }

#### MCC160 Implementation

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId,

NR_SRB_COMMON_IND p_ReceivedAsp,

SRBs_AlreadyConfigured p_SRB := SRB1_2,

template (omit) SDAP_Config p_SDAP_Config,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := ?,

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_BuildQosFlow := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit,

boolean p_OmitDNN := false  // @sic R5-223987 sic@

) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

// Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SSConfig;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList;

var SdapConfig_Type v_SdapConfig;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var template (value) SDAP_Config v_SDAP_Config;

var PDN_Index_Type v_PDN_Index;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@

var template (omit) DNN v_DNN;

var PDU_PDN_DNN_Type v_PDUType := None;  // @sic R5s210148 sic@

var template (omit) IMS_TestConfiguration_Type v_MediaType := omit; // @sic R5s211486 sic@

var boolean v_ExistingPDN:= false; // @sic R5s210149 sic@

var template (omit) PDN_AddressInfo_Type v_ExtraPDN_Address := omit;

var integer i;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

p_ReceivedAsp.Signalling.Nas[0].Pdu,

p_PDU_SessionId,

p_PTI,

p_IntegrityProtMaxDataRate,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_AdditionalInfo)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B) or

match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '100'B)) { // @sic R5-202550, R5s210518, R5s20883 sic@

// this is the only way to flag it's an emergency PDU

v_GSM_MobilityInfo.PDUType := Emergency_PDN;  // @sic R5-206296 sic@

v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486 sic@

}

if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ - compiler warning may be ignored due to check

v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@

} else {

select(v_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@

case(IMS_DNN, Emergency_PDN) {   // @sic R5-206296, R5s210518 sic@

v_SDAP_Config := cs_SDAP_Config(-, -, {2});

}

case (URLLC_DNN) {

v_SDAP_Config := cs_SDAP_Config(-, -, {9}); // @sic R5-221467 sic@

}

case else {

v_SDAP_Config := cs_SDAP_Config;

}

}

}

f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig,

v_CellGroupConfig,

v_SSConfig,

v_SDAP_ConfigurationList,

p_NR_CellId,

{v_GSM_MobilityInfo},

p_SRB,

v_SDAP_Config); // @sic R5s201532 sic@

// Only one PDU session input, so only 1st element relevant in output

v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig);

//It is assumed that by default only 1 QoS Flow is specified

v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]);

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo);

v_GSM_MobilityInfo.PdnIndex := v_PDN_Index;  // @sic R5s210480 sic@

// @sic R5-204400 sic@

if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@

if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) {

v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo, v_MediaType));

}

}

// If this is a default DRB, enable IP handling for this DRB

if (valueof(v_SDAP_Config.defaultDRB)) {

if (isvalue(p_PduInfoFromLTE)) { // @sic R5s210149 sic@

// The UE has just moved from EUTRA. If so, data from EUTRA should be used, rather than created anew

for (i := 0; i < lengthof (p_PduInfoFromLTE); i := i + 1) {

if (v_PDN_Index == valueof(p_PduInfoFromLTE[i].PdnIndex)) {

// Data from EUTRA should be used, rather than created anew

v_ExistingPDN := f_NR_ChangeDrbMappingForExistingPDNs(p_NR_CellId, p_PduInfoFromLTE[i], v_QosFlowId.QFI));

v_GSM_MobilityInfo.EPS_Bearer := valueof(p_PduInfoFromLTE[i].EPS_Bearer); // Overwrite with stored value

break;

}

}

}

if (not v_ExistingPDN) { // completely new PDU session @sic R5s210149 sic@

f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s201094 sic@

/* @sic R5-217449, R5s220617 sic@ */

}

}

...

}

function f_NR5GC_IRAT_S1ToN1_Steps14(NR_CellId_Type p_NR_CellId,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

boolean p_FirstTimeS1ToN1 := true,

InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED)  runs on NR5GC_PTC return NR_RadioBearerId_Type

{

var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList;

//@sic R5s201060 sic@

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var boolean v_FailWhenK_LessN := false;

var template NG_Request_Type v_RequestType := cr_NG_Request_Type('001'B);

var boolean v_MsgRxd := false;

var integer i;

var default v_MyDefaultVar := null;

timer  t_Wait := 8.0; // @sic R5-204399, R5s210077 sic@

if (p_IwkN26 == SUPPORTED) { // @sic R5-204401 sic@

v_FailWhenK_LessN := true; // The only difference between 14a1 and 14b1 branch

v_RequestType := cr_NG_Request_Type('010'B); // @sic R5-206290 sic@

}

// always wait for PDU info, even if not 1st time

v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA);

// @sic R5s210520 sic@

// Clear store of msgs received in parallel

f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@

// Then activate store for any new msgs received - this works for Modification Reqs too

v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId));

t_Wait.start;  //  @sic R5-204399 sic@

// make sure all the PDNs configured on EUTRA are changed in routing table @sic R5s210149, R5s210520 sic@

for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) {

f_NR_ChangeDrbMappingForExistingPDNs (p_NR_CellId, v_PduSessionInfoList[i]);

}

...

}

/*

* @desc      To change the DRB mapping for a PDN that's previously been configured in EUTRA

*            If the QFI isn't included in p_PduSessionInfo, then the p_AlternativeQFI is used

*            If both are set to omit, then return false

* @param     p_PduSessionInfo

* @param     p_AlternativeQFI (default: omit)

* @return    boolean

*/

function f_NR_ChangeDrbMappingForExistingPDNs (NR_CellId_Type p_NR_CellId,

EUTRA_NR_PduSessionInfo_Type p_PduSessionInfo,

template (omit) integer p_AlternativeQFI := omit) runs on NR5GC_PTC return boolean

{

var integer v_QFI := 0;

var boolean v_ChangeRouting := true;

if (isvalue(PduSessionInfo.QFI)) {

v_QFI := PduSessionInfo.QFI;

} else if (isvalue(p_AlternativeQFI)){

v_QFI := valueof(p_AlternativeQFI);

} else {

v_ChangeRouting := false;

}

f (v_ChangeRouting) {

f_IP_ChangeDrbMappingNR(IP, p_NR_CellId, cs_IP_QosFlowId(p_PduSessionInfo.PDU_SessionId, v_QFI), p_PduSessionInfo.PdnIndex);

}

return v_ChangeRouting;

}