3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221213

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2823 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for function f_NR5GC_PDUSessionEstablishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TTCN CR R5s220807 MCC introduced new variable p_NormalIMSOperation which will cause that for normal IMS testcase function f_NR_DelayForUserPlaneSignalling wil be not called. This is not correct for pixit combination pc_noOf_PDUsNewConnection = 1 pc_noOf_PDUsSameConnection = 1 If we have an IMS testcase and if pc_noOf_PDUsNewConnection > 1 then RRC connection is released immediately after f_NR5GC_PDUSessionEstablishment in the function f_NR5GC_RRC_Idle_Steps16_20. This will cause that UE have not a time to make an IMS registration and TC fail(TTCN receive unexpected data - IMS REGISTRATION). The TTCN should release the RRCconnection only for normal IMS operation, but when pc_noOf_PDUsNewConnection = 0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update f_NR5GC_PDUSessionEstablishment that f_NR_DelayForUserPlaneSignalling will be not called for IMS testcases but only when pc_noOf_PDUsNewConnection = 0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IMS tests when pc_noOf_PDUsNewConnection > 0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	5

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR5GC_PDUSessionEstablishment |
| --- | --- |
| Reason for change | The TTCN CR R5s220807 MCC introduced new variable p_NormalIMSOperation which will cause that for normal IMS testcase function f_NR_DelayForUserPlaneSignalling wil be not called. This is not correct for pixit combination pc_noOf_PDUsNewConnection = 1 pc_noOf_PDUsSameConnection = 1 If we have an IMS testcase and if pc_noOf_PDUsNewConnection > 1 then RRC connection is released immediately after f_NR5GC_PDUSessionEstablishment in the function f_NR5GC_RRC_Idle_Steps16_20. This will cause that UE have not a time to make an IMS registration and TC fail(TTCN receive unexpected data - IMS REGISTRATION). The TTCN should release the RRCconnection only for normal IMS operation, but when pc_noOf_PDUsNewConnection = 0. |
| Summary of change | Update f_NR5GC_PDUSessionEstablishment that f_NR_DelayForUserPlaneSignalling will be not called for IMS testcases but only when pc_noOf_PDUsNewConnection = 0 |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted in principle, but added to f_NR5GC_RRC_Idle_Steps16_20 in order to reduce impact |

Before Change:

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId, …. { var SRBs_AlreadyConfigured v_SRBs := SRB1_2; var NR_RadioBearerId_Type v_SRB := p_SRB; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_K_NumOfReqsProcessed := 0; var integer v_L_NumOfReqsReceivedInParallel := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var boolean v_IMS_Configured := false; var PDU_PDN_DNN_Type v_PDUType; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var default v_MyDefaultVar := null; timer t_Wait := 8.0; // @sic R5-206288 sic@ 8s is less than T3540 if (p_EstReqAlreadyInDefault == 0){ // @sic R5s210520 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ } // Then activate store for any new msgs received v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); while ((v_K_NumOfReqsProcessed < p_ExpectedNumberOfNewPDUSessions) and not v_EndLoop) { // @sic R5-199075, R5-204329 sic@ v_RxdMsg := false; t_Wait.start; // @sic R5-204329 sic@ if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > v_L_NumOfReqsReceivedInParallel) { // @sic R5s220174 sic@ // If another msg has been recieved in parallel since the last time we looked v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel + 1; if (v_L_NumOfReqsReceivedInParallel > p_ExpectedNumberOfNewPDUSessions){ // @sic R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "More PDU Session Establishment Messages Received Than Expected"); } v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(v_L_NumOfReqsReceivedInParallel); // @sic R5s220174 sic@ v_RxdMsg := true; t_Wait.stop; // @sic R5s210520 sic@ } else { // no outstanding msgs, so must wait for next PDU Establishment alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // Stop timer and continue with while loop t_Wait.stop; v_RxdMsg := true; } [] t_Wait.timeout { if (p_FailWhenK_LessN) { // @sic R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not enough PDU Session Establishment Messages Received"); } else { v_EndLoop := true; } } } } if (v_K_NumOfReqsProcessed > 0) { // @sic R5s200751 sic@ if (isvalue(p_TriggerWithThisAPN)) { // @sic R5-199075 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, valueof(p_TriggerWithThisAPN)); // @sic R5-204439 sic@ } } if (v_RxdMsg) { // @sic R5-204329 sic@ this will be false if either just triggered a new PDN, or about to end the loop // Update K now as latest msg is about to be processed v_K_NumOfReqsProcessed := v_K_NumOfReqsProcessed + 1; if (v_SRB == tsc_NR_RbId_SRB1) { // @sic R5-198923 sic@ v_SRBs := SRB1; } v_PDUType := f_NR5GC_PDUSessionEstablishment_InnerLoop(p_NR_CellId, v_ReceivedAsp, v_SRBs, p_SDAP_Config, p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr, p_PDU_SessionId, p_PTI, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_IntegrityProtMaxDataRate, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_AdditionalInfo, p_Cause, p_RQTimer, p_EAP, p_BackOff, p_PDUSessionAuthenticationRequired, p_BuildQosFlow, p_PduInfoFromLTE, // @sic R5-198184 sic@ p_OmitDNN); // @sic R5-223987 sic@ if (v_SRB == tsc_NR_RbId_SRB1) { // SRB2 will be configured in the 1st instance (if it hasn't already) v_SRB := tsc_NR_RbId_SRB2; v_SRBs := SRB1_2; // @sic R5-198923 sic@ } v_IMS_Configured := v_IMS_Configured or (match (IMS_DNN, v_PDUType)); // @sic R5-206296 sic@ } } if (p_NormalIMSOperation) { // @sic R5s210515, R5s220807 sic@ /* Need to delay for IP/IMS signalling if RRCRelease is about to be sent if set to noRrcConnectionRelease, assume the connection will be up long enough to complete IMS signalling without waiting here */ // Wait for IP signalling @sic R5-204329, R5-206296 sic@ if (v_IMS_Configured) { v_WaitForUserplaneSignalling := waitForIMS; } else { // currently all PDUs require IP signalling v_WaitForUserplaneSignalling := dontWaitForIMS; } f_NR_DelayForUserPlaneSignalling(v_WaitForUserplaneSignalling); // @sic R5-206296 sic@ } // all expected msgs have been processed, so deactivate the extra default deactivate(v_MyDefaultVar); }

After Change:

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId, …. { var SRBs_AlreadyConfigured v_SRBs := SRB1_2; var NR_RadioBearerId_Type v_SRB := p_SRB; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_K_NumOfReqsProcessed := 0; var integer v_L_NumOfReqsReceivedInParallel := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var boolean v_IMS_Configured := false; var PDU_PDN_DNN_Type v_PDUType; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var default v_MyDefaultVar := null; timer t_Wait := 8.0; // @sic R5-206288 sic@ 8s is less than T3540 if (p_EstReqAlreadyInDefault == 0){ // @sic R5s210520 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ } // Then activate store for any new msgs received v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); while ((v_K_NumOfReqsProcessed < p_ExpectedNumberOfNewPDUSessions) and not v_EndLoop) { // @sic R5-199075, R5-204329 sic@ v_RxdMsg := false; t_Wait.start; // @sic R5-204329 sic@ if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > v_L_NumOfReqsReceivedInParallel) { // @sic R5s220174 sic@ // If another msg has been recieved in parallel since the last time we looked v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel + 1; if (v_L_NumOfReqsReceivedInParallel > p_ExpectedNumberOfNewPDUSessions){ // @sic R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "More PDU Session Establishment Messages Received Than Expected"); } v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(v_L_NumOfReqsReceivedInParallel); // @sic R5s220174 sic@ v_RxdMsg := true; t_Wait.stop; // @sic R5s210520 sic@ } else { // no outstanding msgs, so must wait for next PDU Establishment alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // Stop timer and continue with while loop t_Wait.stop; v_RxdMsg := true; } [] t_Wait.timeout { if (p_FailWhenK_LessN) { // @sic R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not enough PDU Session Establishment Messages Received"); } else { v_EndLoop := true; } } } } if (v_K_NumOfReqsProcessed > 0) { // @sic R5s200751 sic@ if (isvalue(p_TriggerWithThisAPN)) { // @sic R5-199075 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, valueof(p_TriggerWithThisAPN)); // @sic R5-204439 sic@ } } if (v_RxdMsg) { // @sic R5-204329 sic@ this will be false if either just triggered a new PDN, or about to end the loop // Update K now as latest msg is about to be processed v_K_NumOfReqsProcessed := v_K_NumOfReqsProcessed + 1; if (v_SRB == tsc_NR_RbId_SRB1) { // @sic R5-198923 sic@ v_SRBs := SRB1; } v_PDUType := f_NR5GC_PDUSessionEstablishment_InnerLoop(p_NR_CellId, v_ReceivedAsp, v_SRBs, p_SDAP_Config, p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr, p_PDU_SessionId, p_PTI, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_IntegrityProtMaxDataRate, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_AdditionalInfo, p_Cause, p_RQTimer, p_EAP, p_BackOff, p_PDUSessionAuthenticationRequired, p_BuildQosFlow, p_PduInfoFromLTE, // @sic R5-198184 sic@ p_OmitDNN); // @sic R5-223987 sic@ if (v_SRB == tsc_NR_RbId_SRB1) { // SRB2 will be configured in the 1st instance (if it hasn't already) v_SRB := tsc_NR_RbId_SRB2; v_SRBs := SRB1_2; // @sic R5-198923 sic@ } v_IMS_Configured := v_IMS_Configured or (match (IMS_DNN, v_PDUType)); // @sic R5-206296 sic@ } } if (p_NormalIMSOperation or pc_noOf_PDUsNewConnection >0) { // @sic R5s210515, R5s220807 sic@ /* Need to delay for IP/IMS signalling if RRCRelease is about to be sent if set to noRrcConnectionRelease, assume the connection will be up long enough to complete IMS signalling without waiting here */ // Wait for IP signalling @sic R5-204329, R5-206296 sic@ if (v_IMS_Configured) { v_WaitForUserplaneSignalling := waitForIMS; } else { // currently all PDUs require IP signalling v_WaitForUserplaneSignalling := dontWaitForIMS; } f_NR_DelayForUserPlaneSignalling(v_WaitForUserplaneSignalling); // @sic R5-206296 sic@ } // all expected msgs have been processed, so deactivate the extra default deactivate(v_MyDefaultVar); }/

#### MCC160 Implementation:

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId,

integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection,  // @sic R5s201358 sic@,

NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease,

template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (omit) SDAP_Config p_SDAP_Config := omit,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) B4_Type p_ServiceType := '0000'B,

boolean  p_NormalIMSOperation := true // @sic R5s220807 sic@

)  runs on NR5GC_PTC

{

var boolean v_WaitIfIMS := p_NormalIMSOperation;

//Steps 16 - 19a1

// No SRB2 so must be on SRB1

if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection  // @sic R5s201358 sic@ < 0");

}

if (not p_NormalIMSOperation and pc_noOf_PDUsNewConnection >0) { // @sic R5s221213 sic@

v_WaitIfIMS := true;

}

f_NR5GC_PDUSessionEstablishment (p_NR_CellId,

p_ExpectedNumberOfNewPDUSessions,

tsc_NR_RbId_SRB1,

p_SDAP_Config, // @sic R5s210134 sic@

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_SSC_Mode,

p_DNN,

p_QoS_Rules,

p_Session_AMBR,

p_S_NSSAI,

p_QoSFlowDescr,

-, -, -, -, -, -, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -,

v_WaitIfIMS);  // @sic R5s220807, R5s221213 sic@

...