3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221215

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0878 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS 5GS supplementary test case 8.36 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Purpose of this test case is to test a Consultative Call Transfer, being the UE tested the Transferor. At the preamble a MO call is established and then an MMI command is set to start the call transfer (call A will be put On Hold and a new call will be established) as it is Consultative an additional action should be taken on UE side to finish the transfer procedure, transfer confirmation. At the moment, test case implementation is missing the transfer call confirmation MMI, translated as AT command with AT+CHLD=4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added MMI command to complete the Call Transfer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.36.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Correction to function f_TC_8_36_NR5GC_IMS1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_8_36_NR5GC_IMS1

| Function name | f_TC_8_36_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Test case implementation is missing the transfer call confirmation MMI, translated as AT command with AT+CHLD=4. |
| Summary of change | Added MMI command to complete the Call Transfer. |
| TTCN module | \IMS\NR5GC_8\IMS_NR5GC_SupplementaryServiceTestcases.ttcn |
| MCC160 Comment | Postponed until RAN5 decision: Originally the test case has had a step 19 "The UE is made to hold the call". This step has been removed by R&S's prose CR R5-213522. When R&S now thinks that the change in R5-213522 is not correct anymore, another prose CR shall be raised and discussed at RAN5. |

Before Change:

<<SKIPPED CODE>> function f_TC_8_36_NR5GC_IMS1() runs on IMS_PTC { // MO Voice Call Explicit Communication Transfer / Consultative Call Transfer / 5GS /* sister test case of 15.25: Explicit Communication Transfer according to 24.629 with consultative transfer (24.629 annex A.2): The UE is the transferror (UE-B in figure A.2), the SS emulates the transferee (UE-A) and the Transfer-Target (UE-C); in the test case there are two dialogs only as the UE (UE-B) is not involved in dialog 3 of figure A.2 */ var IMS_CC_MO_CallInfo_Type v_CallInfo_Dialog1; var IMS_CC_MO_CallInfo_Type v_CallInfo_Dialog2; var charstring v_CalleeUri_Dialog1 := px_IMS_CalleeUri; var charstring v_CalleeUri_Dialog2 := px_IMS_CalleeUri2; var charstring v_CalleeContactUri_Dialog1 := px_IMS_CalleeContactUri; var charstring v_CalleeContactUri_Dialog2 := px_IMS_CalleeContactUri2; var template (value) SDP_Message v_SDP_LatestMessageBySS; var template (value) SipUrl v_ContactSipUrlSS_Dialog1; var SipUrl v_ContactSipUrlUE; var boolean v_RadioBearersEstablished; var boolean v_DontIncrementSessVers; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, v_CalleeUri_Dialog1); v_CallInfo_Dialog1 := f_IMS_MOCallSetup_NR5GC_A_4_1(v_CalleeUri_Dialog1, v_CalleeContactUri_Dialog1); /* Call between the UE (transferror, UE-B) and the emulated transferee (UE-A) */ v_DontIncrementSessVers := true; // @sic R5s211703: session version shall not be incremented sic@ v_SDP_LatestMessageBySS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_CallInfo_Dialog1.LatestSdpOffer, v_DontIncrementSessVers); v_ContactSipUrlSS_Dialog1 := f_MessageHeader_GetContactSipUrl(v_CallInfo_Dialog1.MessageHeader183); /* NOTE: Dialog 2 should have only one transferee's Contact (even though in f_TC_15_25_IMS1 there is v_SessionInfo_Dialog1.ContactUrl and v_ContactUrl_1) */ v_ContactSipUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ Initiate Explicit Communication Transfer (ECT) - 3GPP TS 24.629 f_UT_IMS_ExplicitCallTransfer(MMI, v_CalleeUri_Dialog2); // @siclog "Step 2 - 5" siclog@ f_IMS_MOCallHoldResume(HOLD, v_CallInfo_Dialog1.InviteRequestWithSdp.Invite, v_CallInfo_Dialog1.MessageHeader183, v_CallInfo_Dialog1.LatestSdpOffer, v_CalleeContactUri_Dialog1); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 6" Void siclog@ // @siclog "Step 7 - 18" siclog@ f_IMS_PTC_ImsInfo_ActivateDialog(secondDialog); f_IMS_PTC_ImsInfo_DialogSetSessIdTX("11111112"); // @sic R5-221479 sic@ v_RadioBearersEstablished := true; // @sic R5s211703 sic@ v_CallInfo_Dialog2 := f_IMS_MOCallSetup_NR5GC_A_4_1(v_CalleeUri_Dialog2, v_CalleeContactUri_Dialog2, v_RadioBearersEstablished); /* Call between the UE (transferror, UE-B) and the emulated Transfer-Target (UE-C) */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @siclog "Step 19" Void siclog@ // @siclog "Step 20 - 23" siclog@ f_IMS_MOCallHoldResume(HOLD, v_CallInfo_Dialog2.InviteRequestWithSdp.Invite, v_CallInfo_Dialog2.MessageHeader183, v_CallInfo_Dialog2.LatestSdpOffer, v_CalleeContactUri_Dialog2); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_8_36_NR5GC_IMS1() runs on IMS_PTC { // MO Voice Call Explicit Communication Transfer / Consultative Call Transfer / 5GS /* sister test case of 15.25: Explicit Communication Transfer according to 24.629 with consultative transfer (24.629 annex A.2): The UE is the transferror (UE-B in figure A.2), the SS emulates the transferee (UE-A) and the Transfer-Target (UE-C); in the test case there are two dialogs only as the UE (UE-B) is not involved in dialog 3 of figure A.2 */ var IMS_CC_MO_CallInfo_Type v_CallInfo_Dialog1; var IMS_CC_MO_CallInfo_Type v_CallInfo_Dialog2; var charstring v_CalleeUri_Dialog1 := px_IMS_CalleeUri; var charstring v_CalleeUri_Dialog2 := px_IMS_CalleeUri2; var charstring v_CalleeContactUri_Dialog1 := px_IMS_CalleeContactUri; var charstring v_CalleeContactUri_Dialog2 := px_IMS_CalleeContactUri2; var template (value) SDP_Message v_SDP_LatestMessageBySS; var template (value) SipUrl v_ContactSipUrlSS_Dialog1; var SipUrl v_ContactSipUrlUE; var boolean v_RadioBearersEstablished; var boolean v_DontIncrementSessVers; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, v_CalleeUri_Dialog1); v_CallInfo_Dialog1 := f_IMS_MOCallSetup_NR5GC_A_4_1(v_CalleeUri_Dialog1, v_CalleeContactUri_Dialog1); /* Call between the UE (transferror, UE-B) and the emulated transferee (UE-A) */ v_DontIncrementSessVers := true; // @sic R5s211703: session version shall not be incremented sic@ v_SDP_LatestMessageBySS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_CallInfo_Dialog1.LatestSdpOffer, v_DontIncrementSessVers); v_ContactSipUrlSS_Dialog1 := f_MessageHeader_GetContactSipUrl(v_CallInfo_Dialog1.MessageHeader183); /* NOTE: Dialog 2 should have only one transferee's Contact (even though in f_TC_15_25_IMS1 there is v_SessionInfo_Dialog1.ContactUrl and v_ContactUrl_1) */ v_ContactSipUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ Initiate Explicit Communication Transfer (ECT) - 3GPP TS 24.629 f_UT_IMS_ExplicitCallTransfer(MMI, v_CalleeUri_Dialog2); // @siclog "Step 2 - 5" siclog@ f_IMS_MOCallHoldResume(HOLD, v_CallInfo_Dialog1.InviteRequestWithSdp.Invite, v_CallInfo_Dialog1.MessageHeader183, v_CallInfo_Dialog1.LatestSdpOffer, v_CalleeContactUri_Dialog1); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 6" Void siclog@ // @siclog "Step 7 - 18" siclog@ f_IMS_PTC_ImsInfo_ActivateDialog(secondDialog); f_IMS_PTC_ImsInfo_DialogSetSessIdTX("11111112"); // @sic R5-221479 sic@ v_RadioBearersEstablished := true; // @sic R5s211703 sic@ v_CallInfo_Dialog2 := f_IMS_MOCallSetup_NR5GC_A_4_1(v_CalleeUri_Dialog2, v_CalleeContactUri_Dialog2, v_RadioBearersEstablished); /* Call between the UE (transferror, UE-B) and the emulated Transfer-Target (UE-C) */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @siclog "Step 19" Void siclog@ // @siclog "Step 20 - 23" siclog@ f_UT_ExplicitCallTransfer(MMI); f_IMS_MOCallHoldResume(HOLD, v_CallInfo_Dialog2.InviteRequestWithSdp.Invite, v_CallInfo_Dialog2.MessageHeader183, v_CallInfo_Dialog2.LatestSdpOffer, v_CalleeContactUri_Dialog2); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); <<SKIPPED CODE>>