3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221217

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4697 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_RmvFbdnPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At the end of the test procedure sequence, UE performs TAU procedure through manual PLMN selection of forbidden PLMN. But TTCN doesn’t handle optional IMS reregistration procedure after the TAU complete, as per TS 36.508 subclause 4.5A.30. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow UE to perform optional IMS Reregistration procedure after tracking area update by introducing function f_EUTRA_WaitForIMSReRegister. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.1.7 and 9.2.1.1.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_RmvFbdnPLMN |
| --- | --- |
| Reason for change | At the end of the test procedure sequence, UE performs TAU procedure through manual PLMN selection of forbidden PLMN. But TTCN doesn’t handle optional IMS reregistration procedure after the TAU complete, as per TS 36.508 subclause 4.5A.30. |
| Summary of change | Allow UE to perform optional IMS Reregistration procedure after tracking area update by introducing function f_EUTRA_WaitForIMSReRegister. |
| TTCN module | EUTRA_NASCommonFunctions.ttcn |
| MCC160 Comment | Accepted as an improvement |

Before change:

function f_EUTRA_RmvFbdnPLMN(EUTRA_CellId_Type p_CellId_Detach, // @sic R5s110498, R5s190327 sic@

EUTRA_CellId_Type p_CellId_Removal,

EUTRA_ATTACH_Type p_ForcedAttach,

EUTRA_RRC_STATE_Type p_UE_State := RRC_CONNECTED

) runs on EUTRA_PTC // @sic R5-106675 sic@

{ //@sic R5-162910:  added p_UE_State sic@

var template (value) CellPowerList_Type v_CellPowerList;

var GutiParameters_Type v_Guti_Params;

v_Guti_Params := f_EUTRA_CellInfo_GetGuti(p_CellId_Removal);

//release RRC connection to bring UE to a permanently stable state

if (p_UE_State == RRC_CONNECTED){ //@sic R5-162910:  added p_UE_State sic@

f_EUTRA_RRC_ConnectionRelease(p_CellId_Detach); // @sic R5s120720, R5s190327 sic@

}

//configure cells to make cell of forbidden PLMN serving cell

v_CellPowerList := {

cs_CellPower(p_CellId_Detach, tsc_NonSuitableOffCellRS_EPRE),

cs_CellPower(p_CellId_Removal, tsc_ServingCellRS_EPRE)

};

f_EUTRA_SetCellPowerList(v_CellPowerList); // @sic R5s110498 sic@

f_UT_ManualPLMN_Select(UT, f_Asn2Nas_PlmnId (v_Guti_Params.PLMN_Identity));

f_EUTRA_TrackingAreaUpdate (p_CellId_Removal, -, p_ForcedAttach); // @sic R5s190327 sic@

f_UT_AutomaticPLMN_Select (UT); // @sic R5s190327 sic@

}

After change:

function f_EUTRA_RmvFbdnPLMN(EUTRA_CellId_Type p_CellId_Detach, // @sic R5s110498, R5s190327 sic@

EUTRA_CellId_Type p_CellId_Removal,

EUTRA_ATTACH_Type p_ForcedAttach,

EUTRA_RRC_STATE_Type p_UE_State := RRC_CONNECTED

) runs on EUTRA_PTC // @sic R5-106675 sic@

{ //@sic R5-162910:  added p_UE_State sic@

var template (value) CellPowerList_Type v_CellPowerList;

var GutiParameters_Type v_Guti_Params;

v_Guti_Params := f_EUTRA_CellInfo_GetGuti(p_CellId_Removal);

//release RRC connection to bring UE to a permanently stable state

if (p_UE_State == RRC_CONNECTED){ //@sic R5-162910:  added p_UE_State sic@

f_EUTRA_RRC_ConnectionRelease(p_CellId_Detach); // @sic R5s120720, R5s190327 sic@

}

//configure cells to make cell of forbidden PLMN serving cell

v_CellPowerList := {

cs_CellPower(p_CellId_Detach, tsc_NonSuitableOffCellRS_EPRE),

cs_CellPower(p_CellId_Removal, tsc_ServingCellRS_EPRE)

};

f_EUTRA_SetCellPowerList(v_CellPowerList); // @sic R5s110498 sic@

f_UT_ManualPLMN_Select(UT, f_Asn2Nas_PlmnId (v_Guti_Params.PLMN_Identity));

f_EUTRA_TrackingAreaUpdate (p_CellId_Removal, -, p_ForcedAttach); // @sic R5s190327 sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

f_EUTRA_WaitForIMSReRegister(p_CellId_Removal); //@sic R5-204524 sic@

f_UT_AutomaticPLMN_Select (UT); // @sic R5s190327 sic@

}