3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221246

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2843 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_NR_Calculate_PF_PO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current implementation of the function f_NR_Calculate_PF_PO in 22wk43, Symbol is not initialized which results in run time issue |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_NR_Calculate_PF_PO , the symbol value is initialized |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the SA test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the function f_NR_Calculate_PF_PO	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk43 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_NR_Calculate_PF_PO

| Template name | f_NR_Calculate_PF_PO |
| --- | --- |
| Reason for change | - In the current implementation of the function f_NR_Calculate_PF_PO in wk43, Symbol is not initialized which results in a run time issue |
| Summary of change | - In the function f_NR_Calculate_PF_PO , the symbol value is initialized |
| TTCN module | NR5GC_SysinfoPaging.ttcn |
| MCC160 Comment | Accepted, implemented as follows: v_PagingOccasion.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); because v_PagingOccasion is not declared as a template. Similar change done in f_NR_ModifySysinfo and f_NR_ModifySysinfo_PWS for the intialisation respectively of v_StartNextModPeriod and v_TimeOfShortMessage |

Before Change:

function f_NR_Calculate_PF_PO (SubFrameTiming_Type p_CurrentTiming, PagingCycle p_T, PCCH_Config p_PCCH_Config, B32_Type p_NG_TMSI, integer p_SlotsNumberPerSubframe, SubcarrierSpacing p_SCS, integer p_SlotOffset) return SubFrameTiming_Type { //@sic R5s190291 change 1: change first parameter p_Sfn -> p_CurrentTiming (type SubFrameTiming_Type) sic@ var integer v_UE_ID := bit2int(p_NG_TMSI) mod 1024; // 5G-S-TMSI mod 1024 var integer v_PF_Offset := fl_NR_ConvertPF_Offset(p_PCCH_Config.nAndPagingFrameOffset); // var integer v_Ns := fl_NR_ConvertNs(p_PCCH_Config.ns); //number of paging occasions for a PF var integer v_NextSfn := p_CurrentTiming.SFN.Number + tsc_NR_SfnDelay; //Absolute value - no mod 1024 var integer v_SfnPlusOffsetModT; var integer v_FirstPagingFrame; var integer v_T := fl_NR_ConvertPagingCycle(p_T); var integer v_N := fl_NR_GetN(p_PCCH_Config.nAndPagingFrameOffset, v_T); // N: number of total paging frames in T @sic R5s190150 change 4 sic@ // var integer v_I_s; var SubFrameTiming_Type v_PagingOccasion; //Calculate the first paging Frame //SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) //Absolute value - no mod 1024 @sic R5-211417 sic@ v_SfnPlusOffsetModT := (v_T / v_N) * (v_UE_ID mod v_N); if ((v_NextSfn mod v_T) < v_SfnPlusOffsetModT) { v_FirstPagingFrame := ((v_NextSfn - v_NextSfn mod v_T) + v_SfnPlusOffsetModT) - v_PF_Offset; } else { v_FirstPagingFrame := (v_NextSfn - v_NextSfn mod v_T) + v_SfnPlusOffsetModT + v_T - v_PF_Offset; } //* Calculate i_s = floor (UE_ID/N) mod Ns // v_I_s := (v_UE_ID mod v_N) mod v_Ns; Note: unused so far //PDCCH monitoring occasions according to paging-SearchSpace and firstPDCCH-MonitoringOccasionOfPO //PDCCH monitoring occasions according to the default association: PDCCH monitoring occasions for paging are same as for RMSI as defined in Section 13 in TS 38.213 //For Ns = 1, there is only one PO which starts in the PF. //For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF. v_PagingOccasion.HSFN.Number := (p_CurrentTiming.HSFN.Number + (v_FirstPagingFrame)/1024 ) mod 1024; //@sic R5-211417 sic@ v_PagingOccasion.SFN.Number := v_FirstPagingFrame mod 1024; //@sic R5-211417 sic@ v_PagingOccasion.Subframe.Number := p_SlotOffset / p_SlotsNumberPerSubframe; v_PagingOccasion := f_NR_SubFrameTiming_SetSlot(v_PagingOccasion, p_SlotOffset mod p_SlotsNumberPerSubframe, p_SCS); return v_PagingOccasion; }

After Change:

function f_NR_Calculate_PF_PO (SubFrameTiming_Type p_CurrentTiming, PagingCycle p_T, PCCH_Config p_PCCH_Config, B32_Type p_NG_TMSI, integer p_SlotsNumberPerSubframe, SubcarrierSpacing p_SCS, integer p_SlotOffset) return SubFrameTiming_Type { //@sic R5s190291 change 1: change first parameter p_Sfn -> p_CurrentTiming (type SubFrameTiming_Type) sic@ var integer v_UE_ID := bit2int(p_NG_TMSI) mod 1024; // 5G-S-TMSI mod 1024 var integer v_PF_Offset := fl_NR_ConvertPF_Offset(p_PCCH_Config.nAndPagingFrameOffset); // var integer v_Ns := fl_NR_ConvertNs(p_PCCH_Config.ns); //number of paging occasions for a PF var integer v_NextSfn := p_CurrentTiming.SFN.Number + tsc_NR_SfnDelay; //Absolute value - no mod 1024 var integer v_SfnPlusOffsetModT; var integer v_FirstPagingFrame; var integer v_T := fl_NR_ConvertPagingCycle(p_T); var integer v_N := fl_NR_GetN(p_PCCH_Config.nAndPagingFrameOffset, v_T); // N: number of total paging frames in T @sic R5s190150 change 4 sic@ // var integer v_I_s; var SubFrameTiming_Type v_PagingOccasion; //Calculate the first paging Frame //SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) //Absolute value - no mod 1024 @sic R5-211417 sic@ v_SfnPlusOffsetModT := (v_T / v_N) * (v_UE_ID mod v_N); if ((v_NextSfn mod v_T) < v_SfnPlusOffsetModT) { v_FirstPagingFrame := ((v_NextSfn - v_NextSfn mod v_T) + v_SfnPlusOffsetModT) - v_PF_Offset; } else { v_FirstPagingFrame := (v_NextSfn - v_NextSfn mod v_T) + v_SfnPlusOffsetModT + v_T - v_PF_Offset; } //* Calculate i_s = floor (UE_ID/N) mod Ns // v_I_s := (v_UE_ID mod v_N) mod v_Ns; Note: unused so far //PDCCH monitoring occasions according to paging-SearchSpace and firstPDCCH-MonitoringOccasionOfPO //PDCCH monitoring occasions according to the default association: PDCCH monitoring occasions for paging are same as for RMSI as defined in Section 13 in TS 38.213 //For Ns = 1, there is only one PO which starts in the PF. //For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF. v_PagingOccasion.HSFN.Number := (p_CurrentTiming.HSFN.Number + (v_FirstPagingFrame)/1024 ) mod 1024; //@sic R5-211417 sic@ v_PagingOccasion.SFN.Number := v_FirstPagingFrame mod 1024; //@sic R5-211417 sic@ v_PagingOccasion.Subframe.Number := p_SlotOffset / p_SlotsNumberPerSubframe; v_PagingOccasion.Symbol := cs_SymbolTimingInfo_FirstSlot; //WA#WI=1022866 v_PagingOccasion := f_NR_SubFrameTiming_SetSlot(v_PagingOccasion, p_SlotOffset mod p_SlotsNumberPerSubframe, p_SCS); return v_PagingOccasion; }