3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221253

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2847 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.5.9.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 8.1.5.9.2 to the IWD_22wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.5.9.2 required for approval. See the detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function f_TC_8_1_5_9_2_NR5GC_TestBody()	4

2.2	Function f_NR_DecodeULDcchMsg()	15

3	Branches executed	17

4	Execution Log Files	18

4.1	MediaTek MT6983	18

5	References	18

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.5.9.2 which is part of the NR5GC test suite in iwd-TTCN3-B2020-09_D22wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.5.9.2

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## Function f_TC_8_1_5_9_2_NR5GC_TestBody()

| Function name | f_TC_8_1_5_9_2_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - In the current Implementation of the test case , at step 6 and 11, v_LateNonCriticalExt := valueof(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension) assignment is done without checking if(ispresent(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension) condition holds True ; this is resulting in run time error - When sending 1st DL segment of Reconfiguration using cs_38508_DLDedicatedMessageSegment, NAS message (Service Accept) can not be piggybacked as there is no asn1 place holder to send the NAS message along with dlDedicatedMessageSegment_r16 nonCriticalExtension SEQUENCE {} OPTIONAL. - Before Step 7 , SS configuration functions are called with CNF required flag set to true , To avoid the instability of receiving security mode complete at step 7 ,while handling the CNF for the previous SS configuration messages, the SS configuration messages can be sent with No Cnf Required flag - At step 11, v_ExistingDRBs has to be set with f_NR5GC_MobileInfo_GetDRBInfoList() - At step 21, Segment index used is “3” which is incorrect ; this has to be corrected to have “2”. |
| Summary of change | - At Step 6 and 11 , if(ispresent(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension) is checked before assigning v_LateNonCriticalExt := valueof(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension). if the rrcReconfiguration.lateNonCriticalExtension is not present , then v_LateNonCriticalExt is assigned with ‘00’O so that further ‘00’O can be appended till the lengthof(bit2oct(encvalue(v_RRCReconfiguration)))< (2*tsc_Max_NR_PDCP_SDU_size) condition gets satisfied - Removed the piggybacked NAS message (Service Accept) from the first segment of cs_38508_DLDedicatedMessageSegment in step 6 and the Service accept message is sent separately after the transmission of 1st segment - CNF is set to False in the SS configurations before receiving the security mode complete to avoid the instability of security mode complete comes before the CNF is handled - At step 11, v_ExistingDRBs is set with f_NR5GC_MobileInfo_GetDRBInfoList() - At Step 21, Segment Index is corrected as ‘2’ |
| TTCN module | \ttcn-3\ttcn\develop\NR5GC\8_1_5\RRC_Others_NR5GC.ttcn |
| MCC160 Comment | - Accepted. - Accepted. f_NR_TransmitDLSegment is also adapted to removed the NAS parameter. See below for implementation. - Alternative solution proposed, where the SS is configured before sending the segment. See below for implementation. - Accepted. - Acceoted. |

Before Change:

function f_TC_8_1_5_9_2_NR5GC_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> // Add padding to get size of encoded RRCReconfiguration message < 2*Max PDCP SDU size, so that it will fit in two segments while (lengthof(bit2oct(encvalue(v_RRCReconfiguration)))< (2*tsc_Max_NR_PDCP_SDU_size)) { v_LateNonCriticalExt := valueof(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension); v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension := v_LateNonCriticalExt & '00'O; }; v_RRCReconfigurationEncoded := bit2oct(encvalue(v_RRCReconfiguration)); // Send first segment and NAS message f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 0, notLastSegment, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())}); //Reconfigure SS to add SRB2 and DRB f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config, cs_TimingInfo_Now); // Configure PDCP security f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // @siclog "Step 7" siclog@ // Check: Does the UE transmit a SecurityModeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE // @siclog "Step 8" siclog@ // The SS transmits a DLDedicatedMessageSegment containing the second(last) segment of the RRCReconfiguration message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 1, lastSegment); // @siclog "Step 9" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); } // @siclog "Step 10" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 10"); // @siclog "Step 11" siclog@ // The SS transmits a DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to modify SRB and DRB configuration. // Must release and re-configure the SS for SRB2 and DRB1 to re-establish the RLC f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2)}); f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBId))}); // Store updated DRB information v_ExistingDRBs := {}; f_NR5GC_MobileInfo_SetDRBInfoList(v_ExistingDRBs); // DRB1 reconfigured during reconfiguration f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_SRB2_Config}); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, { cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBId, cs_38508_PDCP_Config)}); // @sic R5s220029 sic@ // Build RLC-BearerConfig for SRB2 and DRB v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB (nr_Cell1, {v_DRBId}, true); // There should be 2 entries and both of them need to update the re-establishRLC flag v_CellGroupConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_CellGroupConfig.rlc_BearerToAddModList[1].reestablishRLC := true_; v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext); // Add padding to get size of encoded RRCReconfiguration message < 3*Max PDCP SDU size, so that it will fit in three segments while (lengthof(bit2oct(encvalue(v_RRCReconfiguration)))< (3*tsc_Max_NR_PDCP_SDU_size)) { v_LateNonCriticalExt := valueof(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension); v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension := v_LateNonCriticalExt & '00'O; }; v_RRCReconfigurationEncoded := bit2oct(encvalue(v_RRCReconfiguration)); // Send first segment - no NAS message to send this time f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 0, notLastSegment); //Re-establish DRB in SS with exactly the same config as earlier f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_DRBInfo.SS_Config}, cs_TimingInfo_Now); // Configure PDCP security f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // @siclog "Step 12" siclog@ // The SS transmits the second segment of the RRCReconfiguration message to modify SRB and DRB configuration. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 1, notLastSegment); // @siclog "Step 13" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); // @siclog "Step 14" siclog@ // Step 1-8 of generic test procedure in TS 38.508-1 [4] subclause 4.5.4 to bring the UE in RRC_CONNECTED mode. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); // @siclog "Step 15" siclog@ // The SS transmits the third(last) segment of the RRCReconfiguration message to modify SRB2 and DRB configuration. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 2, lastSegment); // @siclog "Step 16" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message in the next 3s? t_Watchdog.start; alt { []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16, RRCReconfigurationComplete sent by UE "); } []t_Watchdog.timeout {} }; // @siclog "Step 17" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); // @siclog "Step 18" siclog@ // Step 1-8 of generic test procedure in TS 38.508-1 [4] subclause 4.5.4 to bring the UE in RRC_CONNECTED mode. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); // @siclog "Step 19" siclog@ // The SS transmits a DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to // modify SRB and DRB configuration. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 0, notLastSegment); // @siclog "Step 20" siclog@ // The SS transmits the second segment of the RRCReconfiguration message to modify SRB and DRB configuration. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 1, notLastSegment); // @siclog "Step 21" siclog@ f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 3, lastSegment); // @siclog "Step 22" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message in the next 3s? t_Watchdog.start; alt { []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) { t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16, RRCReconfigurationComplete NOT sent by UE "); } }; // @siclog "Step 23" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable // of exchanging IP data on DRB#1 established in Steps 12-21? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 23"); // @siclog "Step 24-27" siclog@ f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); }//end of f_TC_8_1_5_9_2_NR5GC_TestBody

After Change:

function f_TC_8_1_5_9_2_NR5GC_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> // Add padding to get size of encoded RRCReconfiguration message < 2*Max PDCP SDU size, so that it will fit in two segments while (lengthof(bit2oct(encvalue(v_RRCReconfiguration)))< (2*tsc_Max_NR_PDCP_SDU_size)) { if(ispresent(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension)) //WA#WI=1019654 { v_LateNonCriticalExt := valueof(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension); } else{ v_LateNonCriticalExt := '00'O; } v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension := v_LateNonCriticalExt & '00'O; }; v_RRCReconfigurationEncoded := bit2oct(encvalue(v_RRCReconfiguration)); // Send first segment and NAS message f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 0, notLastSegment, /*{cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())}*/-); //WA#WI=1019654 SRB.send (cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg()))); //WA#WI=1019654 // @sic R5s201387 Baseline Moving sic@ //Reconfigure SS to add SRB2 and DRB f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config, cs_TimingInfo_Now,false); //WA#WI=1019654 // Configure PDCP security /*f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering);*/ //WA#WI=1019654 f_NR_SS_AS_SecurityConfig(nr_Cell1, cs_NR_AS_SecurityStart_REQ(nr_Cell1, cs_NR_AS_IntProt_CiphActivate(v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering), cs_TimingInfo_Now,false)); //WA#WI=1019654 // @siclog "Step 7" siclog@ // Check: Does the UE transmit a SecurityModeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE // @siclog "Step 8" siclog@ // The SS transmits a DLDedicatedMessageSegment containing the second(last) segment of the RRCReconfiguration message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 1, lastSegment); // @siclog "Step 9" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); } // @siclog "Step 10" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 10"); // @siclog "Step 11" siclog@ // The SS transmits a DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to modify SRB and DRB configuration. // Must release and re-configure the SS for SRB2 and DRB1 to re-establish the RLC f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2)}); f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBId))}); // Store updated DRB information v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); //WA#WI=1019654 f_NR5GC_MobileInfo_SetDRBInfoList(v_ExistingDRBs); // DRB1 reconfigured during reconfiguration f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_SRB2_Config}); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, { cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBId, cs_38508_PDCP_Config)}); // @sic R5s220029 sic@ // Build RLC-BearerConfig for SRB2 and DRB v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB (nr_Cell1, {v_DRBId}, true); // There should be 2 entries and both of them need to update the re-establishRLC flag v_CellGroupConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_CellGroupConfig.rlc_BearerToAddModList[1].reestablishRLC := true_; v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext); // Add padding to get size of encoded RRCReconfiguration message < 3*Max PDCP SDU size, so that it will fit in three segments while (lengthof(bit2oct(encvalue(v_RRCReconfiguration)))< (3*tsc_Max_NR_PDCP_SDU_size)) { if(ispresent(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension)) //WA#WI=1019654 { v_LateNonCriticalExt := valueof(v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension); } else{ v_LateNonCriticalExt := '00'O; } //WA#WI=1019654 v_RRCReconfiguration.message_.c1.rrcReconfiguration.criticalExtensions.rrcReconfiguration.lateNonCriticalExtension := v_LateNonCriticalExt & '00'O; }; v_RRCReconfigurationEncoded := bit2oct(encvalue(v_RRCReconfiguration)); // Send first segment - no NAS message to send this time f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 0, notLastSegment); //Re-establish DRB in SS with exactly the same config as earlier f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_DRBInfo.SS_Config}, cs_TimingInfo_Now); // Configure PDCP security f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // @siclog "Step 12" siclog@ // The SS transmits the second segment of the RRCReconfiguration message to modify SRB and DRB configuration. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 1, notLastSegment); // @siclog "Step 13" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); // @siclog "Step 14" siclog@ // Step 1-8 of generic test procedure in TS 38.508-1 [4] subclause 4.5.4 to bring the UE in RRC_CONNECTED mode. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); // @siclog "Step 15" siclog@ // The SS transmits the third(last) segment of the RRCReconfiguration message to modify SRB2 and DRB configuration. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 2, lastSegment); // @siclog "Step 16" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message in the next 3s? t_Watchdog.start; alt { []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16, RRCReconfigurationComplete sent by UE "); } []t_Watchdog.timeout {} }; // @siclog "Step 17" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); // @siclog "Step 18" siclog@ // Step 1-8 of generic test procedure in TS 38.508-1 [4] subclause 4.5.4 to bring the UE in RRC_CONNECTED mode. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); // @siclog "Step 19" siclog@ // The SS transmits a DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to // modify SRB and DRB configuration. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 0, notLastSegment); // @siclog "Step 20" siclog@ // The SS transmits the second segment of the RRCReconfiguration message to modify SRB and DRB configuration. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 1, notLastSegment); // @siclog "Step 21" siclog@ f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 2, //WA#WI=1019654 lastSegment); // @siclog "Step 22" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message in the next 3s? t_Watchdog.start; alt { []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) { t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16, RRCReconfigurationComplete NOT sent by UE "); } }; // @siclog "Step 23" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable // of exchanging IP data on DRB#1 established in Steps 12-21? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 23"); // @siclog "Step 24-27" siclog@ f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); }//end of f_TC_8_1_5_9_2_NR5GC_TestBody

MCC TF160 proposed implementation:

function f_NR_TransmitDLSegment(NR_CellId_Type p_NR_CellId,

octetstring p_RRCReconfigurationEncoded,

integer p_Index,

DLDedicatedMessageSegment_r16_IEs.rrc_MessageSegmentType_r16 p_SegmentType

) runs on NR_BASE_PTC

{

var octetstring v_Segment;

var template (value) NR_RRC_MSG_Request_Type v_RrcMsg;

v_Segment := substr(p_RRCReconfigurationEncoded, (p_Index * tsc_Max_NR_PDCP_SDU_size), tsc_Max_NR_PDCP_SDU_size);

v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_DLDedicatedMessageSegment(1, v_Segment, p_SegmentType));

SRB.send(cas_NR_RrcNasPduList_REQ(p_NR_CellId,

tsc_NR_RbId_SRB1,

cs_TimingInfo_Now,

v_RrcMsg,

-,

-, -,

cs_RlcBearerRouting_NR(nr_Cell1)));

}

//Reconfigure SS to add SRB2 and DRB

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config, cs_TimingInfo_Now);

// Configure PDCP security

f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1,

v_SecurityParams.AS_Integrity,

v_SecurityParams.AS_Ciphering);

// Send first segment and NAS message

f_NR_TransmitDLSegment(nr_Cell1,

v_RRCReconfigurationEncoded,

0,

notLastSegment);

//@sic R5s221253 sic@

SRB.send (cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())));

// @siclog "Step 7" siclog@

// Check: Does the UE transmit a SecurityModeComplete message?

SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE

// @siclog "Step 8" siclog@

// The SS transmits a DLDedicatedMessageSegment containing the second(last) segment of the RRCReconfiguration message.

f_NR_TransmitDLSegment(nr_Cell1,

v_RRCReconfigurationEncoded,

1,

lastSegment);

## Function f_NR_DecodeULDcchMsg()

| Function name | f_NR_DecodeULDcchMsg() |
| --- | --- |
| Reason for change | In the current Implementation of the function f_NR_DecodeULDcchMsg , segment number has to be filled in line with the segment index . currently it is always filled as 1 |
| Summary of change | Segment number is filled in line with the segment index |
| TTCN module | ttcn-3\ttcn\develop\NR5GC\8_1_5\RRC_Others_NR5GC.ttcn |
| MCC160 Comment | This function is not used in TC 8.1.5.9.2, therefore this change is not needed for its verification. Please provide a separate Cat F CR to correct this function. |

Before Change:

function f_NR_DecodeULDcchMsg(octetstring p_Octetstring) { //@sic R5s220330 sic@ var UL_DCCH_Message v_UL_Dcch_Msg; var bitstring v_Bitstring := oct2bit(p_Octetstring); var integer v_Result := decvalue(v_Bitstring, v_UL_Dcch_Msg); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE NR Capability cannot be decoded"); } } //Transmit a segment of the encoded RRCReconfiguration message. The size of the segment is tsc_Max_NR_PDCP_SDU_size function f_NR_TransmitDLSegment(NR_CellId_Type p_NR_CellId, octetstring p_RRCReconfigurationEncoded, integer p_Index, DLDedicatedMessageSegment_r16_IEs.rrc_MessageSegmentType_r16 p_SegmentType, template (omit) NG_NAS_MSG_RequestList_Type p_NasMsgList := omit ) runs on NR_BASE_PTC { var octetstring v_Segment; var template (value) NR_RRC_MSG_Request_Type v_RrcMsg; v_Segment := substr(p_RRCReconfigurationEncoded, (p_Index * tsc_Max_NR_PDCP_SDU_size), tsc_Max_NR_PDCP_SDU_size); v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_DLDedicatedMessageSegment(1, v_Segment, p_SegmentType)); SRB.send(cas_NR_RrcNasPduList_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, v_RrcMsg, p_NasMsgList, -, -, cs_RlcBearerRouting_NR(nr_Cell1))); }

After Change:

function f_NR_DecodeULDcchMsg(octetstring p_Octetstring) { //@sic R5s220330 sic@ var UL_DCCH_Message v_UL_Dcch_Msg; var bitstring v_Bitstring := oct2bit(p_Octetstring); var integer v_Result := decvalue(v_Bitstring, v_UL_Dcch_Msg); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE NR Capability cannot be decoded"); } } //Transmit a segment of the encoded RRCReconfiguration message. The size of the segment is tsc_Max_NR_PDCP_SDU_size function f_NR_TransmitDLSegment(NR_CellId_Type p_NR_CellId, octetstring p_RRCReconfigurationEncoded, integer p_Index, DLDedicatedMessageSegment_r16_IEs.rrc_MessageSegmentType_r16 p_SegmentType, template (omit) NG_NAS_MSG_RequestList_Type p_NasMsgList := omit ) runs on NR_BASE_PTC { var octetstring v_Segment; var template (value) NR_RRC_MSG_Request_Type v_RrcMsg; v_Segment := substr(p_RRCReconfigurationEncoded, (p_Index * tsc_Max_NR_PDCP_SDU_size), tsc_Max_NR_PDCP_SDU_size); v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_DLDedicatedMessageSegment(p_Index, v_Segment, p_SegmentType)); //WA#WI=1019654 SRB.send(cas_NR_RrcNasPduList_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, v_RrcMsg, p_NasMsgList, -, -, cs_RlcBearerRouting_NR(nr_Cell1))); }

# Branches executed

## This NR TC was executed in NR5GC FR1 Band Combination n78 with NEA1 and NIA1.

# Execution Log Files

## MediaTek MT6983

The MediaTek MT6983 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_5_9_2_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221254: Supporting information for addition of NR5GC TC 8.1.5.9.2 |
| --- | --- |