3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221257

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4703 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Highest UL and DL bandwidth for Band 18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 36.508 Table 4.3.1.1.18-1, the highest channel bandwidth for Band 18 is 15 MHz. In the current TTCN implementation, it is set to 10 MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set maximum DL and UL bandwidth of Band 18 to 15 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bandwidth not be set properly for band 18 for cases that run on maximum channel bandwidth |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.5, 7.1.7.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 change 1	2

2.2 change 2	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 change 1

| Function name | f_EUTRA_GetHighestDL_Bandwidth |
| --- | --- |
| Reason for change | According to TS 36.508 Table 4.3.1.1.18-1, the highest channel bandwidth for Band 18 is 15 MHz. In the current TTCN implementation, it is set to 10 MHz. |
| Summary of change | Updated f_EUTRA_GetHighestDL_Bandwidth to set maximum DL bandwidth of Band 18 to 15 MHz |
| TTCN module | Common\EUTRA\EUTRA_CellInfoFrequency.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_EUTRA_GetHighestDL_Bandwidth(integer p_Band) return Dl_Bandwidth_Type { //@sic R5-163085 FrequencyBandindicator type changed to integer sic@ var Dl_Bandwidth_Type v_Dl_Bandwidth; //@sic R5-172672 sic@ select (p_Band){ case (1,2,3,4,7,9,10,20,22,23,28,32,33,35,36,37,38,39,40,41,42,43,44,45,46,47,48,65,66,69,71) { //@sic R5s110709 R5-125778 R5-165521 R5-166128 R5-166132 R5-176557 sic@ v_Dl_Bandwidth := n100; // Max ChBandwidth = 20 } case (5,6,8,11,12,13,14,15,16,17,18,24,25,27,29,30,53,85) { // @sic R5-115722 R5-121738 R5-125832 R5-126021 R5-145606 R5-192336 sic@ v_Dl_Bandwidth := n50; // Max ChBandwidth = 10 } case (19, 21,26,34,67,68,70) { // @sic R5-121905 R5-176216 R5s190782 sic@ v_Dl_Bandwidth := n75; // Max ChBandwidth = 15 } case (31,72, 73, 87, 88) { // @sic R5-140509 R5-176210 R5-196725 R5-205078 sic@ v_Dl_Bandwidth := n25; // Max ChBandwidth = 5 } case else{ FatalError (__FILE__, __LINE__, "invalid Band"); } }; return v_Dl_Bandwidth; }

After Change:

function f_EUTRA_GetHighestDL_Bandwidth(integer p_Band) return Dl_Bandwidth_Type { //@sic R5-163085 FrequencyBandindicator type changed to integer sic@ var Dl_Bandwidth_Type v_Dl_Bandwidth; //@sic R5-172672 sic@ select (p_Band){ case (1,2,3,4,7,9,10,20,22,23,28,32,33,35,36,37,38,39,40,41,42,43,44,45,46,47,48,65,66,69,71) { //@sic R5s110709 R5-125778 R5-165521 R5-166128 R5-166132 R5-176557 sic@ v_Dl_Bandwidth := n100; // Max ChBandwidth = 20 } case (5,6,8,11,12,13,14,15,16,17,24,25,27,29,30,53,85) { // @sic R5-115722 R5-121738 R5-125832 R5-126021 R5-145606 R5-192336 sic@ v_Dl_Bandwidth := n50; // Max ChBandwidth = 10 } case (18,19, 21,26,34,67,68,70) { // @sic R5-121905 R5-176216 R5s190782 sic@ v_Dl_Bandwidth := n75; // Max ChBandwidth = 15 } case (31,72, 73, 87, 88) { // @sic R5-140509 R5-176210 R5-196725 R5-205078 sic@ v_Dl_Bandwidth := n25; // Max ChBandwidth = 5 } case else{ FatalError (__FILE__, __LINE__, "invalid Band"); } }; return v_Dl_Bandwidth; }

## 2.2 change 2

| Function name | f_EUTRA_GetHighestUL_Bandwidth |
| --- | --- |
| Reason for change | According to TS 36.508 Table 4.3.1.1.18-1, the highest channel bandwidth for Band 18 is 15 MHz. In the current TTCN implementation, it is set to 10 MHz. |
| Summary of change | Updated f_EUTRA_GetHighestUL_Bandwidth to set maximum UL bandwidth of Band 18 to 15 MHz |
| TTCN module | Common\EUTRA\EUTRA_CellInfoFrequency.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_EUTRA_GetHighestUL_Bandwidth(integer p_Band) return Ul_Bandwidth_Type { //@sic R5-163085 FrequencyBandindicator type changed to integer sic@ var Ul_Bandwidth_Type v_Ul_Bandwidth; //@sic R5-172672 sic@ select (p_Band){ case (1,2,3,4,7,9,10,20,22,23,28, 33,35,36,37,38,39,40,41,42,43,44, 45, 46, 48, 65, 66, 71) { //@sic R5s110709 R5-125778 R5-165521 R5-166128 R5-166132 R5-176557 sic@ v_Ul_Bandwidth := n100; // Max ChBandwidth = 20 } case (5,6,8,11,12,13,14, 15,16, 17,18, 24, 25, 27, 29,30, 53, 85) { // @sic R5-115722 R5s120822 R5-125832 R5-126021 R5-145606 R5-192336 sic@ v_Ul_Bandwidth := n50; // Max ChBandwidth = 10 } case (19, 21,26,34,67,68,70) { //@sic R5-121905 R5-176216 R5-190782 sic@ v_Ul_Bandwidth := n75; // Max ChBandwidth = 15 } case (31, 72, 73, 87, 88) { // @sic R5-140509 R5-176210 R5-196725 R5-205078 sic@ v_Ul_Bandwidth := n25; // Max ChBandwidth = 5 } case else{ FatalError (__FILE__, __LINE__, "invalid Band"); } }; return v_Ul_Bandwidth; }

After Change:

function f_EUTRA_GetHighestUL_Bandwidth(integer p_Band) return Ul_Bandwidth_Type { //@sic R5-163085 FrequencyBandindicator type changed to integer sic@ var Ul_Bandwidth_Type v_Ul_Bandwidth; //@sic R5-172672 sic@ select (p_Band){ case (1,2,3,4,7,9,10,20,22,23,28, 33,35,36,37,38,39,40,41,42,43,44, 45, 46, 48, 65, 66, 71) { //@sic R5s110709 R5-125778 R5-165521 R5-166128 R5-166132 R5-176557 sic@ v_Ul_Bandwidth := n100; // Max ChBandwidth = 20 } case (5,6,8,11,12,13,14, 15,16, 17, 24, 25, 27, 29,30, 53, 85) { // @sic R5-115722 R5s120822 R5-125832 R5-126021 R5-145606 R5-192336 sic@ v_Ul_Bandwidth := n50; // Max ChBandwidth = 10 } case (18,19, 21,26,34,67,68,70) { //@sic R5-121905 R5-176216 R5-190782 sic@ v_Ul_Bandwidth := n75; // Max ChBandwidth = 15 } case (31, 72, 73, 87, 88) { // @sic R5-140509 R5-176210 R5-196725 R5-205078 sic@ v_Ul_Bandwidth := n25; // Max ChBandwidth = 5 } case else{ FatalError (__FILE__, __LINE__, "invalid Band"); } }; return v_Ul_Bandwidth; }