3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221287

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0881 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS 5GS Call Control test case 7.21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During test case 7.21 execution after a MO audio call has been established, SS tries to modify the call adding video media. Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. In addition, MMI command to modify the video call to an audio call is duplicated: first is sent at Step 8 and then sent again while executing f_IMS_VideoCall_NR5GC_ReleaseVideoMO function. Proposal is to send an MMI command aligned with the action expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed the MMI command sent in Step 3a6 and Step 3b1. Removed duplicated MMI command sent in Step 8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.21.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	New function f_UT_AcceptVideoCallModification	3

2.2	Correction to enumerated Ut_Common_Req_Type	3

2.3	New template cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION	4

2.4	Correction to function fl_UT_HandleCmd	4

2.5	Correction to function f_TC_7_21_NR5GC_IMS1	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## New function f_UT_AcceptVideoCallModification

| Function name | f_UT_AcceptVideoCallModification |
| --- | --- |
| Reason for change | Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. |
| Summary of change | Added new function to send a MMI command to accept video media for current audio call. |
| TTCN module | \Common\Common\UpperTesterFunctions.ttcn |
| MCC160 Comment | Accepted in principle but called it f_UT_IMS_AcceptMediaChange and used AT command +CCMMD |

Before Change:

<<SKIPPED CODE>> function f_UT_AcceptVideoCallModification(UT_PTC_MTC_PORT p_Port, charstring p_CDUIdentifier) { fl_UT_ApplyCommand(p_Port, cas_UT_Req(ACCEPT_VIDEO_CALL_MODIFICATION, NO_CNF_REQUIRED)); } <<SKIPPED CODE>>

## Correction to enumerated Ut_Common_Req_Type

| Enumerated name | Ut_Common_Req_Type |
| --- | --- |
| Reason for change | Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. |
| Summary of change | Added new entry to type enumerated Ut_Common_Req_Type. |
| TTCN module | \Common\Common\UpperTesterFunctions.ttcn |
| MCC160 Comment | Not needed – used existing type IMS_MODIFY_CALL |

Before Change:

<<SKIPPED CODE>> type enumerated Ut_Common_Req_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NBIOT, NR5GC, NR5GC_IRAT, POS, SSNITZ, UTRAN) */ ACCEPT_MTSI_TEXT, ACTIVATE_BEARER, ACTIVATE_MESSAGE_WAIT_INDICATION, ACTIVATE_TEST_MODE_E, APPLAYER_MEASREPORT, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> type enumerated Ut_Common_Req_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NBIOT, NR5GC, NR5GC_IRAT, POS, SSNITZ, UTRAN) */ ACCEPT_MTSI_TEXT, ACCEPT_VIDEO_CALL_MODIFICATION, ACTIVATE_BEARER, ACTIVATE_MESSAGE_WAIT_INDICATION, ACTIVATE_TEST_MODE_E, APPLAYER_MEASREPORT, <<SKIPPED CODE>>

## New template cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION

| Template name | cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION |
| --- | --- |
| Reason for change | Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. |
| Summary of change | Added new template to send MMI "ACCEPT_VIDEO_CALL_MODIFICATION”. |
| TTCN module | \Common\Common\UpperTesterCmds.ttcn |
| MCC160 Comment | Not needed. Modified existing template cs_AT_IMS_MODIFY_CALL |

New template:

<<SKIPPED CODE>> template (value) Ut_System_Cmd_Type cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION := cs_AT_Cmd("ACCEPT_VIDEO_CALL_MODIFICATION"); <<SKIPPED CODE>>

## Correction to function fl_UT_HandleCmd

| Function name | fl_UT_HandleCmd |
| --- | --- |
| Reason for change | Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. |
| Summary of change | Added new case to function fl_UT_HandleCmd to handle video media addition to an audio call. |
| TTCN module | \Common\MTC\MTC_UpperTester.ttcn |
| MCC160 Comment | Not needed |

Before Change:

<<SKIPPED CODE>> case (RELEASE_IMS_CALL) { v_SystemCmd := cs_AT_RELEASE_IMS_CALL (v_ParamListVal[0].Value); } case (INITIATE_VIDEO_CALL) { v_SystemCmd := cs_AT_START_IMS_VIDEO_CALL(v_ParamListVal[0].Value, v_ParamListVal[1].Value); } case (IMS_CONFIGURE_VIDEO_CALL) { v_SystemCmd := cs_AT_IMS_CONFIGURE_VIDEO_CALL; } case (REQUEST_CALL_INFO) { v_SystemCmd := cs_AT_REQUEST_CALL_INFO; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> case (RELEASE_IMS_CALL) { v_SystemCmd := cs_AT_RELEASE_IMS_CALL (v_ParamListVal[0].Value); } case (INITIATE_VIDEO_CALL) { v_SystemCmd := cs_AT_START_IMS_VIDEO_CALL(v_ParamListVal[0].Value, v_ParamListVal[1].Value); } case (IMS_CONFIGURE_VIDEO_CALL) { v_SystemCmd := cs_AT_IMS_CONFIGURE_VIDEO_CALL; } case (ACCEPT_VIDEO_CALL_MODIFICATION) { v_SystemCmd := cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION; } case (REQUEST_CALL_INFO) { v_SystemCmd := cs_AT_REQUEST_CALL_INFO; } <<SKIPPED CODE>>

## Correction to function f_TC_7_21_NR5GC_IMS1

| Function name | f_TC_7_21_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. - In addition, MMI command to modify the video call to an audio call is duplicated: first is sent at Step 8 and then sent again while executing f_IMS_VideoCall_NR5GC_ReleaseVideoMO function. |
| Summary of change | 1. Changed the MMI command sent in Step 3a6 and Step 3b1. 2. Removed duplicated MMI command sent in Step 8. |
| TTCN module | \Common\Common\UpperTesterFunctions.ttcn |
| MCC160 Comment | 1. The AT command used needs to be documented in the prose. Similar changes to be made to both TTCN and prose of sister test case 17.2. Accepted in principle subject to prose CR approval. 2. Accepted |

Before Change:

<<SKIPPED CODE>> if (isvalue(v_IMS_DATA_RSP)) { v_Response := valueof(v_IMS_DATA_RSP.Response); v_SdpAnswer := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_Response.messageBody, v_ExpectedSdpAnswer); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a1"); // @siclog "Step 3a2" siclog@ Stop timer t_Timer.stop; // @siclog "Step 3a3 - 3a4" siclog@ PRACK, 200 OK f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequestWithSdpVideo.Invite, v_Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a4"); // @siclog "Step 3a5" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 3a6 - 3a8" siclog@ MMI (Accept Call), 200 OK, ACK f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdpVideo); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a7"); } else { // @siclog "Step 3b1" siclog@ MMI (Accept Call) f_UT_AnswerCall(MMI); // @siclog "Step 3b2" siclog@ 200 OK v_ResponseWithSdp := f_IMS_MTCallSetup_Receive200OKforINVITE(v_InviteRequestWithSdpVideo, v_ExpectedSdpAnswer); v_SdpAnswer := v_ResponseWithSdp.SdpMessage; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3b2"); // @siclog "Step 3b3" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 3b4" siclog@ ACK f_IMS_InviteMT_SendACK(v_InviteRequestWithSdpVideo); } // @siclog "Step 4 - 7" Void siclog@ // @siclog "Step 8" siclog@ f_UT_IMS_RemoveVideoMedia(MMI, v_CallIdentification); // @siclog "Step 8 - 13" siclog@ MMI (Remove Video), INVITE, 100 Trying, 200 OK, ACK f_IMS_VideoCall_NR5GC_ReleaseVideoMO(v_SdpAnswer, v_CallIdentification); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9, 13"); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> if (isvalue(v_IMS_DATA_RSP)) { v_Response := valueof(v_IMS_DATA_RSP.Response); v_SdpAnswer := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_Response.messageBody, v_ExpectedSdpAnswer); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a1"); // @siclog "Step 3a2" siclog@ Stop timer t_Timer.stop; // @siclog "Step 3a3 - 3a4" siclog@ PRACK, 200 OK f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequestWithSdpVideo.Invite, v_Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a4"); // @siclog "Step 3a5" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 3a6 - 3a8" siclog@ MMI (Accept Call), 200 OK, ACK f_UT_AcceptVideoCallModification(MMI); // @sic R5-210056 sic@ f_IMS_MTCallSetup_Receive200OK_SendACK(v_InviteRequestWithSdpVideo); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a7"); } else { // @siclog "Step 3b1" siclog@ MMI (Accept Call) f_UT_AcceptVideoCallModification(MMI); // @siclog "Step 3b2" siclog@ 200 OK v_ResponseWithSdp := f_IMS_MTCallSetup_Receive200OKforINVITE(v_InviteRequestWithSdpVideo, v_ExpectedSdpAnswer); v_SdpAnswer := v_ResponseWithSdp.SdpMessage; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3b2"); // @siclog "Step 3b3" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 3b4" siclog@ ACK f_IMS_InviteMT_SendACK(v_InviteRequestWithSdpVideo); } // @siclog "Step 4 - 7" Void siclog@ // @siclog "Step 8" siclog@ //f_UT_IMS_RemoveVideoMedia(MMI, v_CallIdentification); // @siclog "Step 8 - 13" siclog@ MMI (Remove Video), INVITE, 100 Trying, 200 OK, ACK f_IMS_VideoCall_NR5GC_ReleaseVideoMO(v_SdpAnswer, v_CallIdentification); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9, 13"); // @siclog "Step 14 - 16" siclog@ MMI (Release Call), BYE, 200 OK f_IMS_CallReleaseMO(v_InviteRequestWithSdpAudio.Invite); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_SpeechCall); } <<SKIPPED CODE>>

#### MCC160 Implementation:

template (value) Ut_System_Cmd_Type cs_AT_IMS_MODIFY_CALL  (Ut_ParameterList_Type p_ParamList) :=

cs_AT_Cmd("AT+CCMMD="& p_ParamList[0].Value & "," & p_ParamList[1].Value & "," & p_ParamList[2].Value & "<CR>");                             /* @status    APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NBIOT, NR5GC, NR5GC_IRAT, POS, SSNITZ, UTRAN) */

function f_UT_AddVideoToIMSCall(UT_PTC_MTC_PORT p_Port,

charstring p_CDUIdentifier)

{

fl_UT_ApplyCommand(p_Port, cas_UT_Req(IMS_MODIFY_CALL, NO_CNF_REQUIRED, {cs_Ut_Parameter_Common("CallId", p_CDUIdentifier),

cs_Ut_Parameter_Common("Modify Parameter", "1"),                                                                               cs_Ut_Parameter_Common("Media", "m=audio\0D\0Am=video")}));

}

function f_UT_IMS_RemoveVideoMedia(UT_PTC_MTC_PORT p_Port,

charstring p_CDUIdentifier)

{

fl_UT_ApplyCommand(p_Port, cas_UT_Req(IMS_MODIFY_CALL, NO_CNF_REQUIRED, {cs_Ut_Parameter_Common("CallId", p_CDUIdentifier),

cs_Ut_Parameter_Common("Modify Parameter", "1"),                                                                            cs_Ut_Parameter_Common("Media", "m=audio")}));

}

function f_UT_IMS_AcceptMediaChange(UT_PTC_MTC_PORT p_Port,

charstring p_CDUIdentifier,

charstring p_ProposedMedia := "m=audio\0D\0Am=video")

{

fl_UT_ApplyCommand(p_Port, cas_UT_Req(IMS_MODIFY_CALL, NO_CNF_REQUIRED, {cs_Ut_Parameter_Common("CallId", p_CDUIdentifier),

cs_Ut_Parameter_Common("Modify Parameter", "3"),

cs_Ut_Parameter_Common("Media", p_ProposedMedia)}));

}

function f_TC_7_21_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MO Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS

var template (omit) IMS_DATA_RSP v_IMS_DATA_RSP;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdpAudio;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdpVideo;

var IMS_ResponseWithSdp_Type v_ResponseWithSdpAudio;

var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpListAudio;

var IMS_ResponseWithSdp_Type v_ResponseWithSdpVideo;

var template (value) SDP_Message v_SdpOffer;

var SDP_media_desc v_MediaDescrAudio;

var SDP_attribute v_FmtpAttributeAudio;

var charstring v_FmtAudio;

var charstring v_BrVal;

var charstring v_BwVal;

var template (present) SDP_attribute_fmtp_format_specific_params v_FmtpParamsAudio;

var template (present) SDP_Message v_ExpectedSdpAnswer;

var template (present) IMS_Response_Type v_Response183;

var IMS_Response_Type v_Response;

var SDP_Message v_SdpAnswer;

var charstring v_CallIdentification;

timer t_Timer := 2.0;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, -, normalCall_ReturnId);

v_InviteRequestWithSdpAudio := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1();

v_ResponseWithSdpAudio := f_IMS_MOCallSetup_NR5GC_A_4_2_Step2_8(v_InviteRequestWithSdpAudio);

v_CallIdentification := f_UT_ReceivedIMSCallId(MMI);

f_IMS_TestBody_Set(true);

v_MediaDescrAudio := valueof(f_IMS_SDP_MediaList_GetMediaStream(v_ResponseWithSdpAudio.SdpMessage.media_list, c_audio));            /* @sic R5s221127: Get media description from 183 session progress sic@ */

v_MediaDescriptionRtpListAudio := cs_IMS_SDP_MediaDescriptionRtpList(v_MediaDescrAudio.media_field.fmts, v_MediaDescrAudio.attributes);

v_SdpOffer := f_IMS_BuildSDP_InitialOfferAddVideo_TX(v_MediaDescriptionRtpListAudio, tsc_IMS_SDP_VideoCodecList_H265_only, tsc_SDP_WithoutPreconditions);

v_FmtAudio := f_SDP_MediaDescr_GetAudioCodec(v_MediaDescrAudio, EVS_16000);

v_FmtpAttributeAudio := valueof(f_SDP_AttributeList_GetAttribute(v_MediaDescrAudio.attributes, cr_SDP_Attribute_fmtp(v_FmtAudio)));

v_BrVal := f_SIP_SemicolonParamList_GetParamValue(v_FmtpAttributeAudio.fmtp.params.paramList, "br");

v_BwVal := f_SIP_SemicolonParamList_GetParamValue(v_FmtpAttributeAudio.fmtp.params.paramList, "bw");

v_FmtpParamsAudio := cr_Fmtp_ParamList(cr_EVS_Fmtp_AudioParamsBrBwMaxRed(v_BrVal, v_BwVal));

v_ExpectedSdpAnswer := f_IMS_BuildSDP_MTCallAudioVideo_RX(AddVideo, InitialAnswer, EVS_16000, v_FmtAudio, v_FmtpParamsAudio, H265_90000, tsc_SDP_WithoutPreconditions); // @sic R5-225423: audio description for 183 Session Progress (step 3a1) or 200 OK (step 3b2sic@

f_IMS_IPCAN_Query(IPCAN);           // @sic R5s220939 change 1.1: IPCAN will not send the response before f_NR5GC_IPCAN_MO_SpeechCall (calling f_NR5GC_ModifyPDUSession_AddVoice) has finished sic@

// @siclog "Step 1" siclog@                 INVITE

v_InviteRequestWithSdpVideo := f_IMS_MTCallSetup_Video_SendInvite(AddVideo, v_SdpOffer);

// @siclog "Step 2A" siclog@                Start timer

t_Timer.start;

// @siclog "Step 2 - 3a1" siclog@           100 Trying, 183 Session Progress (optional)

v_Response183 := cr_Response(c_statusLine183,

f_IMS_InviteResponse_183_MessageHeaderRX(v_InviteRequestWithSdpVideo.Invite, -, tsc_OptionTagList_100rel),

cr_MessageBody_SDP);

v_IMS_DATA_RSP := f_IMS_ReceiveResponse_OptionalTrying_Timeout(t_Timer,

v_InviteRequestWithSdpVideo.Invite.msgHeader,

v_InviteRequestWithSdpVideo.RoutingInfo.Protocol,

v_Response183);

if (isvalue(v_IMS_DATA_RSP)) {

v_Response := valueof(v_IMS_DATA_RSP.Response);

v_SdpAnswer := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_Response.messageBody, v_ExpectedSdpAnswer);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a1");

// @siclog "Step 3a2" siclog@             Stop timer

t_Timer.stop;

// @siclog "Step 3a3 - 3a4" siclog@       PRACK, 200 OK

f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequestWithSdpVideo.Invite, v_Response.msgHeader);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a4");

// @siclog "Step 3a5" siclog@

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo);

f_IMS_IPCAN_WaitForTrigger(IPCAN);

// @siclog "Step 3a6 - 3a8" siclog@       MMI (Accept Call), 200 OK, ACK

f_UT_IMS_AcceptMediaChange(MMI, v_CallIdentification);

f_IMS_MTCallSetup_Receive200OK_SendACK (v_InviteRequestWithSdpVideo);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a7");

}

else {

// @siclog "Step 3b1" siclog@             MMI (Accept Call)

f_UT_IMS_AcceptMediaChange(MMI, v_CallIdentification);

// @siclog "Step 3b2" siclog@             200 OK

v_ResponseWithSdpVideo := f_IMS_MTCallSetup_Receive200OKforINVITE(v_InviteRequestWithSdpVideo, v_ExpectedSdpAnswer);

v_SdpAnswer := v_ResponseWithSdpVideo.SdpMessage;

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3b2");

// @siclog "Step 3b3" siclog@

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo);

f_IMS_IPCAN_WaitForTrigger(IPCAN);

// @siclog "Step 3b4" siclog@             ACK

f_IMS_InviteMT_SendACK(v_InviteRequestWithSdpVideo);

}

// @siclog "Step 4 - 7" Void siclog@

// @siclog "Step 8" siclog@

f_UT_IMS_RemoveVideoMedia(MMI, v_CallIdentification);

// @siclog "Step 8 - 13" siclog@            MMI (Remove Video), INVITE, 100 Trying, 200 OK, ACK

f_IMS_VideoCall_NR5GC_ReleaseVideoMO(v_SdpAnswer, v_CallIdentification);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9, 13");

// @siclog "Step 14 - 16" siclog@           MMI (Release Call), BYE, 200 OK

f_IMS_CallReleaseMO(v_InviteRequestWithSdpAudio.Invite);

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MO_SpeechCall);

}

function f_IMS_MTSpeechAddVideoRemoveVideo() runs on IMS_PTC

{

var IMS_CC_MT_SpeechCallInfo_Type v_SpeechCallInfo;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var IMS_ResponseWithSdp_Type v_ResponseWithSdp;

var IMS_Response_Type v_SessionProgress183;

var template (present) IMS_Response_Type v_ExpectedSessionProgress183;

var template (value) INVITE_Request v_InviteRequest;

var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpListAudio;

var template (value)   SDP_Message v_SdpOffer;

var template (present) SDP_Message v_SdpAnswer;

var template SDP_Message v_SdpAnswer200OK;

var template (value) MessageHeader v_MessageHeader_Invite;

var template (value) MessageHeader v_MessageHeader_Update;

var template (value) MessageHeader v_MessageHeader_Ack;

var template (present) MessageHeader v_MessageHeader_Response;

var template (omit) IMS_DATA_RSP v_IMS_DATA_RSP_Optional;

var SipUrl v_ContactUrl;

var InternetProtocol_Type v_Protocol;

var charstring v_CallIdentification;

var IMS_SDP_AudioCodecList_Type v_AudioCodecList;

var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN();

timer t_Timer := 5.0;

f_IMS_CC_StartCall(IPCAN_MT_SpeechCall);    // @sic R5s190003, R5s190006 sic@

if (v_IsWLAN) {

v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR;

v_SpeechCallInfo := f_IMS_MTCallSetup_AnnexC11a(v_AudioCodecList);

}

else {

v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR;

v_SpeechCallInfo := f_IMS_MTCallSetup_AnnexC11(v_AudioCodecList);

}

v_ContactUrl := v_SpeechCallInfo.ContactUrl;

v_CallIdentification := f_UT_GetCallId(MMI);

f_IMS_TestBody_Set(true);

// @siclog "Step 1" siclog@

// Step 1 SS sends re-INVITE with second SDP offer to add video.

v_MediaDescriptionRtpListAudio := f_SDP_MediaDescriptionSubsequentOfferAudioTX(AMR_16000);          /* @sic R5s221127 sic@ */

v_SdpOffer := f_IMS_BuildSDP_InitialOfferAddVideo_TX(v_MediaDescriptionRtpListAudio);               /* @sic R5-213524, R5-213538, R5-213539, R5-213541: f_IMS_BuildSDP_MTCallAudioVideo_TX replaced by f_IMS_BuildSDP_InitialOfferAddVideo_TX sic@ */

v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(AddVideo, v_SdpOffer, v_ContactUrl);   /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */

v_InviteRequest := v_InviteRequestWithSdp.Invite;

if (v_IsWLAN) {

// @siclog "Step 2" siclog@

v_Protocol := v_InviteRequestWithSdp.RoutingInfo.Protocol;

v_SdpAnswer200OK := f_IMS_BuildSDP_MTCallAudioVideo_RX(AddVideo);

//@siclog "Step 2A" siclog@

v_ExpectedSessionProgress183 := cr_Response(c_statusLine183,

f_IMS_InviteResponse_183_MessageHeaderRX(v_InviteRequest),

cr_MessageBody_SDP);

//Start timer

t_Timer.start; //not in Prose!!

v_IMS_DATA_RSP_Optional := f_IMS_ReceiveResponse_OptionalTrying_Timeout(t_Timer, v_InviteRequest.msgHeader, v_Protocol, v_ExpectedSessionProgress183);

t_Timer.stop;

//@siclog "Step 2B-2C" siclog@

if (isvalue(v_IMS_DATA_RSP_Optional)) { // @sic R5s190003 sic@

v_SessionProgress183 := valueof(v_IMS_DATA_RSP_Optional.Response);

f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_SessionProgress183.messageBody, v_SdpAnswer200OK);

f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequest, v_SessionProgress183.msgHeader);

v_SdpAnswer200OK := omit;

}

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo);

f_IMS_IPCAN_WaitForTrigger(IPCAN);

}

else {

// @siclog "Step 2: Void" siclog@

// @siclog "Step 3" siclog@

v_ResponseWithSdp := f_IMS_MTCallSetup_Video_Receive183(AddVideo, v_InviteRequestWithSdp);    /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */

v_SessionProgress183 := v_ResponseWithSdp.Response;

// @siclog "Step 4 - 5" siclog@

f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequest, v_SessionProgress183.msgHeader);

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo);            /* @sic R5-181141: trigger moved after step 4/5 sic@ */

f_IMS_IPCAN_WaitForTrigger(IPCAN);

// @siclog "Step 6" siclog@

v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudioVideo_TX(AddVideo, -, -, v_ResponseWithSdp.SdpMessage);     /* @sic R5-213524, R5-213538, R5-213539, R5-213541: f_IMS_BuildSDP_MTCallAudioVideo_TX replaced by f_IMS_BuildSDP_UpdatedOfferAudioVideo_TX sic@ */

v_MessageHeader_Update := f_IMS_UpdateRequest_MessageHeaderTX(v_InviteRequest, PreconditionSupported);    /* @sic R5-160795: PreconditionSupported sic@ */

v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol();                                                   /* @sic R5s160540 change 1.5 sic@ */

IMS_Client.send(cas_IMS_Update_Request(cs_IMS_RoutingInfo(v_Protocol), cs_UPDATE_Request(v_ContactUrl, v_MessageHeader_Update, cs_MessageBody_SDP(v_SdpOffer))));

// @siclog "Step 7" siclog@

v_SdpAnswer := f_IMS_BuildSDP_MTCallAudioVideo_RX(AddVideo, FinalAnswer);  /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */

v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Update, cr_ContentTypeSDP);   /* @sic R5-160795, R5-162930: precondition required sic@ */

v_IMS_DATA_RSP_Optional := f_IMS_ReceiveResponse(v_MessageHeader_Update, v_Protocol, cr_Response(c_statusLine200, v_MessageHeader_Response, cr_MessageBody_SDP));

f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_IMS_DATA_RSP_Optional.Response.messageBody, v_SdpAnswer);

v_SdpAnswer200OK := omit;

}

// @siclog "Step 7A (17.2)" siclog@

// @siclog "Step 2D (G.17.2)" siclog@

// @siclog "Step 6 (17.2)" siclog@

f_UT_IMS_AcceptMediaChange(MMI, v_CallIdentification);

// @siclog "Step 8 - 9 (17.2)" siclog@

// @siclog "Step 3 - 4 (G.17.2)" siclog@

// The UE responds to the re-INVITE with a 200 OK final response.

// The SS acknowledges the receipt of 200 OK for the re-INVITE.

f_IMS_MTCallSetup_Receive200OK_SendACK(v_InviteRequestWithSdp, v_SdpAnswer200OK, tsc_SDP_WithPreconditions);

// @siclog "Step 10 (17.2)" siclog@

// @siclog "Step 5 (G.17.2)" siclog@

// SS sends a re-INVITE with a SDP offer indicating that the video component is removed from the call

v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudioVideo_TX(RemoveVideo);   /* @sic R5-213524, R5-213538, R5-213539, R5-213541: f_IMS_BuildSDP_MTCallAudioVideo_TX replaced by f_IMS_BuildSDP_UpdatedOfferAudioVideo_TX sic@ */

v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(RemoveVideo, v_SdpOffer, v_ContactUrl);  /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */

// @siclog "Step 10A - 11 (17.2)" siclog@

// @siclog "Step 6 - 7 (G.17.2)" siclog@

v_MessageHeader_Invite := v_InviteRequestWithSdp.Invite.msgHeader;

v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Invite, cr_ContentTypeSDP);    /* @sic R5-160795, R5-162930: cr_ContentTypeSDP, precondition required sic@ */

f_IMS_ReceiveResponse_OptionalTrying(v_InviteRequestWithSdp.Invite.msgHeader,

v_InviteRequestWithSdp.RoutingInfo.Protocol,

cr_Response(c_statusLine200, v_MessageHeader_Response, cr_MessageBody_SDP));  /* @sic R5s160842, R5s170013 sic@ */

// @siclog "Step 11A (17.2)" siclog@

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_ReleaseVideo);

// @siclog "Step 12 (17.2)" siclog@

// @siclog "Step 8  (G.17.2)" siclog@

v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite);

IMS_Client.send(cas_IMS_Ack_Request(cs_IMS_RoutingInfo(v_InviteRequestWithSdp.RoutingInfo.Protocol), cs_ACK_Request(v_ContactUrl, v_MessageHeader_Ack)));

f_IMS_IPCAN_WaitForTrigger(IPCAN);    // Wait for trigger from IPCAN to ensure that radio resources have been released

// @siclog "Step 13: Void (17.2)" siclog@

// @siclog "Step 14 - 16 (17.2)" siclog@

// @siclog "Step 9 - 10  (G.17.2)" siclog@

f_IMS_CallReleaseMT(v_ContactUrl);

f_IMS_TestBody_Set(false);

}