3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221326

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2860 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN-DC FR1 : Addition of Rel-16 RRC test case 8.2.4.1.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 8.2.4.1.1.6 to the 5G/ENDC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.2.4.1.1.6 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.1.1.6.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	5

4	Execution Log Files	5

4.1	MediaTek MT6879	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.2.4.1.1.6 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.2.4.1.1.6

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	MediaTek MT6879

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_GetTestcaseAttrib_Eutra_Release() |
| --- | --- |
| Reason for change | This test case is testing Rel-16 conformance requirements for NR not for EUTRA. For EUTRA RAT the UE is only required to comply to Rel-15 for EN-DC configuration. |
| Summary of change | Change required EUTRA release to Rel-15 for this test case. |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Not needed, already implemented in 22wk37 |

Before Change

function f_GetTestcaseAttrib_Eutra_Release(charstring p_Testcase) return charstring { select (p_Testcase) { …………………………….. case ("TC_8_2_4_1_1_6_ENDC") { return "rel_16"; } …………………….. } return ""; }

After Change

function f_GetTestcaseAttrib_Eutra_Release(charstring p_Testcase) return charstring { select (p_Testcase) { …………………………….. case ("TC_8_2_4_1_1_6_ENDC") { return "rel_16"; } …………………….. } return ""; }

## Change 2

| Function name | f_GetTestcaseAttrib_NR_Release() |
| --- | --- |
| Reason for change | This test case is testing Rel-16 conformance requirements for NR not for EUTRA. |
| Summary of change | Return “rel_16” for this test case (instead of in f_GetTestcaseAttrib_Eutra_Release(), see change 1) |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_GetTestcaseAttrib_NR_Release(charstring p_Testcase) return charstring { select (p_Testcase) { case ("TC_7_1_1_12_3_ENDC") { return "rel_16"; } case ("TC_7_1_1_12_4_1_ENDC") { return "rel_16"; } case ("TC_7_1_1_12_4_2_ENDC") { return "rel_16"; } case ("TC_7_1_1_12_4_3_ENDC") { return "rel_16"; } case ("TC_7_1_1_4_1_5_ENDC") { return "rel_16"; } case ("TC_7_1_1_4_2_6_ENDC") { return "rel_16"; } case ("TC_8_2_3_18_1_ENDC") { return "rel_16"; } case ("TC_8_2_3_18_2_ENDC") { return "rel_16"; } case ("TC_8_2_3_18_3_ENDC") { return "rel_16"; } } return ""; }

After Change

function f_GetTestcaseAttrib_NR_Release(charstring p_Testcase) return charstring { select (p_Testcase) { case ("TC_7_1_1_12_3_ENDC") { return "rel_16"; } case ("TC_7_1_1_12_4_1_ENDC") { return "rel_16"; } case ("TC_7_1_1_12_4_2_ENDC") { return "rel_16"; } case ("TC_7_1_1_12_4_3_ENDC") { return "rel_16"; } case ("TC_7_1_1_4_1_5_ENDC") { return "rel_16"; } case ("TC_7_1_1_4_2_6_ENDC") { return "rel_16"; } case ("TC_8_2_3_18_1_ENDC") { return "rel_16"; } case ("TC_8_2_3_18_2_ENDC") { return "rel_16"; } case ("TC_8_2_3_18_3_ENDC") { return "rel_16"; } case ("TC_8_2_4_1_1_6_ENDC") { return "rel_16"; } } return ""; }

# Branches executed

The test case was executed on EN-DC band combination 1A_n78C using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## MediaTek MT6879

The MediaTek MT6879UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_2_4_1_1_6_LOG.html

- PICS/PIXIT parameter file:

TC_8_2_4_1_1_6_PIXIT.xml

# References

| [1] | R5s221327 : EN-DC FR1 : Supporting information for addition of EN-DC RRC Rel-16 test case 8.2.4.1.1.6 |
| --- | --- |