3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221335

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0885 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_IPCAN_ProcedureEnd |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | While running XCAP testcase 8.1, it is observed that when the UE is switched OFF in the postamble, it releases both the PDU Sessions, the one setup for XCAP and the IMS. This causes testcase failure as the postamble is not able to handle two PDU Session Releases. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that after the need for the XCAP PDU session is over, the XCAP PDU Session should be released before the UE is switched OFF. Basically updated the function f_NR5GC_IPCAN_ProcedureEnd to handle the case when procedureType is IPCAN_XCAP_Signalling. Called a new function f_NR5GC_PDUSessionReleaseXCAP_MT which releases the XCAP PDU Session. Please see below for detailed change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to f_NR5GC_IPCAN_ProcedureEnd	2

2.2	New function f_NR5GC_PDUSessionReleaseXCAP_MT	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to f_NR5GC_IPCAN_ProcedureEnd

| Template name | f_NR5GC_IPCAN_ProcedureEnd |
| --- | --- |
| Reason for change | While running XCAP testcase 8.1, it is observed that when the UE is switched OFF in the postamble, it releases both the PDU Sessions, the one setup for XCAP and the IMS. This causes testcase failure as the postamble is not able to handle two PDU Session Releases. This needs to be addressed. |
| Summary of change | Updated the function f_NR5GC_IPCAN_ProcedureEnd to handle the case when procedureType is IPCAN_XCAP_Signalling. Called a new function f_NR5GC_PDUSessionReleaseXCAP_MT which releases the XCAP PDU Session. Please see below for detailed change. |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment | The real problem is in the postamble procedure where only one PDU session is expected to be released – this is not correct in a multi-PDN environment. Draft prose CR is required to allow more than one PDU session to be released in 38.508-1 clause 4.9.6. Decision postponed until outcome of prose CR. |

Before Change:

function f_NR5GC_IPCAN_ProcedureEnd(IMS_IPCAN_CO_ORD_PORT p_Port, //parameter left for future use NR_CellId_Type p_NR_CellId, IMS_TestProcedure_Type p_ProcedureType, IMS_TestConfiguration_Type p_TestConfiguration, TriggerResult_Type p_CallEstablishmentResult) runs on NR5GC_PTC { select (p_ProcedureType) { case (IPCAN_InitialRegistration) { // just release RRC connection } case (IPCAN_MO_IMS_Signalling, IPCAN_MT_IMS_Signalling) { // just release RRC connection } case (IPCAN_EmergencyCall_NormalService) { if (p_TestConfiguration == IPCAN_SpeechAndEmergencyCall) { // @sic R5-225426 sic@ f_NR5GC_ModifyPDUSession_RemoveEmergency(p_NR_CellId); } else { f_NR5GC_EmergencyCallReleaseMT(p_NR_CellId, false); // @sic R5s220531, R5s220789, R5-225362 sic@ If MO, UT & IMS msgs will already have been sent } } case (IPCAN_EmergencyCall_OptionalPDNRelease) { // @sic R5-225362 sic@ f_NR5GC_EmergencyCallRelease(p_NR_CellId, false, false); // @sic R5s220531, R5s220789 sic@ If MO, UT & IMS msgs will already have been sent } case (IPCAN_EmergencyCall_LimitedService) { } case (IPCAN_MO_SpeechCall, IPCAN_MT_SpeechCall) { if (p_CallEstablishmentResult == NORMAL) { // NOTE: in test cases for unsuccessful behaviour no speech flow is established */ f_NR5GC_ModifyPDUSession_RemoveIMSVoice(p_NR_CellId); // @sic R5s210096: f_NR5GC_ModifyPDUSession_RemoveIMSVoice instead of f_NR5GC_CallRelease sic@ } } case (IPCAN_MO_VideoCall, IPCAN_MT_VideoCall) { if (p_CallEstablishmentResult == NORMAL) { // @sic R5s210877 sic@ f_NR5GC_ModifyPDUSession_RemoveIMSVideo(p_NR_CellId); } } case (IPCAN_XCAP_Signalling) { // just release RRC connection } } }

After Change:

function f_NR5GC_IPCAN_ProcedureEnd(IMS_IPCAN_CO_ORD_PORT p_Port, //parameter left for future use NR_CellId_Type p_NR_CellId, IMS_TestProcedure_Type p_ProcedureType, IMS_TestConfiguration_Type p_TestConfiguration, TriggerResult_Type p_CallEstablishmentResult) runs on NR5GC_PTC { select (p_ProcedureType) { case (IPCAN_InitialRegistration) { // just release RRC connection } case (IPCAN_MO_IMS_Signalling, IPCAN_MT_IMS_Signalling) { // just release RRC connection } case (IPCAN_EmergencyCall_NormalService) { if (p_TestConfiguration == IPCAN_SpeechAndEmergencyCall) { // @sic R5-225426 sic@ f_NR5GC_ModifyPDUSession_RemoveEmergency(p_NR_CellId); } else { f_NR5GC_EmergencyCallReleaseMT(p_NR_CellId, false); // @sic R5s220531, R5s220789, R5-225362 sic@ If MO, UT & IMS msgs will already have been sent } } case (IPCAN_EmergencyCall_OptionalPDNRelease) { // @sic R5-225362 sic@ f_NR5GC_EmergencyCallRelease(p_NR_CellId, false, false); // @sic R5s220531, R5s220789 sic@ If MO, UT & IMS msgs will already have been sent } case (IPCAN_EmergencyCall_LimitedService) { } case (IPCAN_MO_SpeechCall, IPCAN_MT_SpeechCall) { if (p_CallEstablishmentResult == NORMAL) { // NOTE: in test cases for unsuccessful behaviour no speech flow is established */ f_NR5GC_ModifyPDUSession_RemoveIMSVoice(p_NR_CellId); // @sic R5s210096: f_NR5GC_ModifyPDUSession_RemoveIMSVoice instead of f_NR5GC_CallRelease sic@ } } case (IPCAN_MO_VideoCall, IPCAN_MT_VideoCall) { if (p_CallEstablishmentResult == NORMAL) { // @sic R5s210877 sic@ f_NR5GC_ModifyPDUSession_RemoveIMSVideo(p_NR_CellId); } } case (IPCAN_XCAP_Signalling) { //Now release XCAP PDU Session before postamble f_NR5GC_PDUSessionReleaseXCAP_MT(p_NR_CellId); //WA#8_1 R&S change for XCAP } } }

## New function f_NR5GC_PDUSessionReleaseXCAP_MT

| Template name | f_NR5GC_PDUSessionReleaseXCAP_MT |
| --- | --- |
| Reason for change | Please see change 2.1. |
| Summary of change | New function |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

function f_NR5GC_PDUSessionReleaseXCAP_MT(NR_CellId_Type p_NR_CellId, //boolean p_IMSMsgsNotYetSent:= true, EmergencyCallRelease_PDUSession p_Release_Keep := Release_PDUSession) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var PDN_Index_Type v_PdnIndex := v_PDUSession.PdnIndex; // @sic R5s220829 sic@ f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PdnIndex, tsc_IMS_MediaPort_M1, omit); }