3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240644

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3571 | rev | - | Current version: | 18.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_EUTRA_ModifySysinfo_ValueTag |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The length of LTE SFN is 10ms, we should use (10ms*SFN number)/1000 to translate it in the unit of second. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use 100 instead of 1000 in calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - Change 1

| Functionname | f_EUTRA_ModifySysinfo_ValueTag |
| --- | --- |
| Reason for change | The length of LTE SFN is 10ms, we should use (10ms*SFN number)/1000 to translate it in the unit of second. |
| Summary of change | Use 100 instead of 1000 in calculation. |
| TTCN module | EUTRA_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA_ModifySysinfo_ValueTag(EUTRA_CellId_Type p_CellId, boolean p_PagingFlag := true, EUTRA_RRC_STATE_Type p_UE_State := RRC_IDLE, template (omit) Null_Type p_StopSib1Transmission := omit //@sic R5-142409 sic@ ) // @sic R5-110803 change 4: add new parameter sic@ runs on EUTRA_PTC return SubFrameTiming_Type { //@sic R5-104157 return SystemFrameNumber_Type sic@ //@sic R5s160738 return SubFrameTiming_Type sic@ var SubFrameTiming_Type v_Timing; //@sic R5w160204r1 sic@ var SubFrameTiming_Type v_TimeOfPaging; //@sic R5w160204r1 sic@ var SubFrameTiming_Type v_Timing_SFN; //@sic R5w160204r1 sic@ var SubFrameTiming_Type v_StartNextModPeriod; //@sic R5w160204r1 sic@ var SystemFrameNumber_Type v_StartNextPagingCycle; //@sic R5s100475 cl. 4.3 Additional changes sic@ var EUTRA_CellSysInfo_Type v_Sysinfo:= f_EUTRA_CellInfo_GetSYSINFO (p_CellId); // Get SYSINFO to be sent var integer v_ModificationPeriodFrames := f_CalculateModificationPeriod (p_CellId); var PCCH_Config v_PcchConfig := f_EUTRA_CellInfo_GetPCCH_ConfigInSYSINFO(p_CellId); //@sic R5s100475 cl. 4.3 Additional changes sic@ var integer v_PagingCycle := f_EUTRA_GetPagingCycleValue (v_PcchConfig.defaultPagingCycle); //@sic R5s100475 cl. 4.3 Additional changes sic@ select (v_Sysinfo.SysinfoType) { //@sic R5-157569 sic@ case(sysinfoOnly){ // Increment the VALUE TAG in f_EUTRA_SS_ConfigureSysinfo @sic R5s130513 sic@ //Get the current SFN and subframe v_Timing := f_EUTRA_GetCurrentTiming(p_CellId); //Add processing delay to current SFN v_Timing_SFN := v_Timing; v_Timing_SFN.HSFN.Number := (v_Timing.HSFN.Number + (v_Timing.SFN.Number + tsc_SfnDelay)/1024) mod 1024; //@sic R5w160204r1 sic@ v_Timing_SFN.SFN.Number := (v_Timing.SFN.Number + tsc_SfnDelay) mod 1024; //@sic R5s100475 cl. 4.3 Additional changes sic@ // Calculate the next Modification Period // @sic R5s100475 cl. 4.3 Additional changes sic@ v_StartNextModPeriod := v_Timing_SFN; v_StartNextModPeriod.HSFN.Number := ( v_StartNextModPeriod.HSFN.Number + (v_Timing_SFN.SFN.Number + (v_ModificationPeriodFrames -(v_Timing_SFN.SFN.Number mod v_ModificationPeriodFrames)))/1024)mod 1024; //@sic R5w160204r1 sic@ v_StartNextModPeriod.SFN.Number := (v_Timing_SFN.SFN.Number + (v_ModificationPeriodFrames -(v_Timing_SFN.SFN.Number mod v_ModificationPeriodFrames))) mod 1024; //@sic R5s100475 cl. 4.3 Additional changes sic@ if (p_PagingFlag) { v_TimeOfPaging := v_StartNextModPeriod; v_StartNextModPeriod.HSFN.Number := (v_StartNextModPeriod.HSFN.Number + (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames)/1024) mod 1024; //@sic R5w160204r1 sic@ v_StartNextModPeriod.SFN.Number := (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames) mod 1024; if ((p_UE_State == RRC_CONNECTED) and not(pc_ETWS_message or pc_CMAS_Message)) { // When UE is in connected mode, paging message will be sent during a complete modification period // Add a longer processing delay to current SFN v_Timing_SFN.HSFN.Number := ( v_Timing_SFN.HSFN.Number + (v_Timing.SFN.Number + tsc_SfnDelayConnected) / 1024) mod 1024; v_Timing_SFN.SFN.Number := (v_Timing.SFN.Number + tsc_SfnDelayConnected) mod 1024; // @sic R5s100475 cl. 4.3 Additional changes sic@ v_StartNextModPeriod.HSFN.Number := (v_Timing_SFN.HSFN.Number +(v_Timing_SFN.SFN.Number + (v_ModificationPeriodFrames -(v_Timing_SFN.SFN.Number mod v_ModificationPeriodFrames)))/ 1024) mod 1024; //@sic R5s170917 sic@ v_StartNextModPeriod.SFN.Number := (v_Timing_SFN.SFN.Number + (v_ModificationPeriodFrames -(v_Timing_SFN.SFN.Number mod v_ModificationPeriodFrames)))mod 1024; v_TimeOfPaging := v_StartNextModPeriod; v_StartNextModPeriod.HSFN.Number := (v_StartNextModPeriod.HSFN.Number + (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames)/1024) mod 1024; v_StartNextModPeriod.SFN.Number := (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames) mod 1024; } if ((p_UE_State == RRC_CONNECTED) and (pc_ETWS_message or pc_CMAS_Message)) { // When UE is in connected mode and UE suppports ETWS/CMAS, the paging is sent during a pagingCycle // Send Paging messages at the next PagingCycle // Send SYSINFO at the next modificationPeriod v_StartNextPagingCycle := (v_Timing_SFN.SFN.Number + (v_PagingCycle -(v_Timing_SFN.SFN.Number mod v_PagingCycle)))mod 1024; v_TimeOfPaging.HSFN.Number := (v_Timing_SFN.HSFN.Number + (v_Timing_SFN.SFN.Number + (v_PagingCycle -(v_Timing_SFN.SFN.Number mod v_PagingCycle)))/ 1024 )mod 1024; //@sic R5s170425 sic@ v_TimeOfPaging.SFN.Number := v_StartNextPagingCycle; //@sic R5s170070 sic@ v_StartNextModPeriod.HSFN.Number := (v_TimeOfPaging.HSFN.Number + (v_StartNextPagingCycle + (v_ModificationPeriodFrames -(v_StartNextPagingCycle mod v_ModificationPeriodFrames)))/ 1024) mod 1024; v_StartNextModPeriod.SFN.Number := (v_StartNextPagingCycle + (v_ModificationPeriodFrames -(v_StartNextPagingCycle mod v_ModificationPeriodFrames)))mod 1024; } // Delay to ensure the sequencing of Scheduling of SIB modification and Paging @sic R5s130949 sic@ f_Delay(int2float((v_TimeOfPaging.SFN.Number-v_Timing.SFN.Number+1024) mod 1024)/1000.0); //Page UE from the start of v_TimeOfPaging //based on above calculation f_EUTRA_UE_PageSysinfoMod(p_CellId, v_TimeOfPaging, p_UE_State); // @sic R5-110803 change 4: add new parameter sic@ } f_EUTRA_SS_ConfigureSysinfo (p_CellId, cs_TimingInfo_SFN(v_StartNextModPeriod.SFN.Number, v_StartNextModPeriod.HSFN.Number), v_Sysinfo.SB1_ValueTagChanged, p_StopSib1Transmission); // @sic R5s120130 sic@ } case(sysinfoAndBR){ v_StartNextModPeriod := f_EUTRA_ModifySysinfo_ValueTagBR(p_CellId, p_PagingFlag, p_StopSib1Transmission); } case else {FatalError (__FILE__, __LINE__, "Invalid SysinfoType");} } return v_StartNextModPeriod; //@sic R5s160738 sic@ }

After Change:

function f_EUTRA_ModifySysinfo_ValueTag(EUTRA_CellId_Type p_CellId, boolean p_PagingFlag := true, EUTRA_RRC_STATE_Type p_UE_State := RRC_IDLE, template (omit) Null_Type p_StopSib1Transmission := omit //@sic R5-142409 sic@ ) // @sic R5-110803 change 4: add new parameter sic@ runs on EUTRA_PTC return SubFrameTiming_Type { //@sic R5-104157 return SystemFrameNumber_Type sic@ //@sic R5s160738 return SubFrameTiming_Type sic@ var SubFrameTiming_Type v_Timing; //@sic R5w160204r1 sic@ var SubFrameTiming_Type v_TimeOfPaging; //@sic R5w160204r1 sic@ var SubFrameTiming_Type v_Timing_SFN; //@sic R5w160204r1 sic@ var SubFrameTiming_Type v_StartNextModPeriod; //@sic R5w160204r1 sic@ var SystemFrameNumber_Type v_StartNextPagingCycle; //@sic R5s100475 cl. 4.3 Additional changes sic@ var EUTRA_CellSysInfo_Type v_Sysinfo:= f_EUTRA_CellInfo_GetSYSINFO (p_CellId); // Get SYSINFO to be sent var integer v_ModificationPeriodFrames := f_CalculateModificationPeriod (p_CellId); var PCCH_Config v_PcchConfig := f_EUTRA_CellInfo_GetPCCH_ConfigInSYSINFO(p_CellId); //@sic R5s100475 cl. 4.3 Additional changes sic@ var integer v_PagingCycle := f_EUTRA_GetPagingCycleValue (v_PcchConfig.defaultPagingCycle); //@sic R5s100475 cl. 4.3 Additional changes sic@ select (v_Sysinfo.SysinfoType) { //@sic R5-157569 sic@ case(sysinfoOnly){ // Increment the VALUE TAG in f_EUTRA_SS_ConfigureSysinfo @sic R5s130513 sic@ //Get the current SFN and subframe v_Timing := f_EUTRA_GetCurrentTiming(p_CellId); //Add processing delay to current SFN v_Timing_SFN := v_Timing; v_Timing_SFN.HSFN.Number := (v_Timing.HSFN.Number + (v_Timing.SFN.Number + tsc_SfnDelay)/1024) mod 1024; //@sic R5w160204r1 sic@ v_Timing_SFN.SFN.Number := (v_Timing.SFN.Number + tsc_SfnDelay) mod 1024; //@sic R5s100475 cl. 4.3 Additional changes sic@ // Calculate the next Modification Period // @sic R5s100475 cl. 4.3 Additional changes sic@ v_StartNextModPeriod := v_Timing_SFN; v_StartNextModPeriod.HSFN.Number := ( v_StartNextModPeriod.HSFN.Number + (v_Timing_SFN.SFN.Number + (v_ModificationPeriodFrames -(v_Timing_SFN.SFN.Number mod v_ModificationPeriodFrames)))/1024)mod 1024; //@sic R5w160204r1 sic@ v_StartNextModPeriod.SFN.Number := (v_Timing_SFN.SFN.Number + (v_ModificationPeriodFrames -(v_Timing_SFN.SFN.Number mod v_ModificationPeriodFrames))) mod 1024; //@sic R5s100475 cl. 4.3 Additional changes sic@ if (p_PagingFlag) { v_TimeOfPaging := v_StartNextModPeriod; v_StartNextModPeriod.HSFN.Number := (v_StartNextModPeriod.HSFN.Number + (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames)/1024) mod 1024; //@sic R5w160204r1 sic@ v_StartNextModPeriod.SFN.Number := (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames) mod 1024; if ((p_UE_State == RRC_CONNECTED) and not(pc_ETWS_message or pc_CMAS_Message)) { // When UE is in connected mode, paging message will be sent during a complete modification period // Add a longer processing delay to current SFN v_Timing_SFN.HSFN.Number := ( v_Timing_SFN.HSFN.Number + (v_Timing.SFN.Number + tsc_SfnDelayConnected) / 1024) mod 1024; v_Timing_SFN.SFN.Number := (v_Timing.SFN.Number + tsc_SfnDelayConnected) mod 1024; // @sic R5s100475 cl. 4.3 Additional changes sic@ v_StartNextModPeriod.HSFN.Number := (v_Timing_SFN.HSFN.Number +(v_Timing_SFN.SFN.Number + (v_ModificationPeriodFrames -(v_Timing_SFN.SFN.Number mod v_ModificationPeriodFrames)))/ 1024) mod 1024; //@sic R5s170917 sic@ v_StartNextModPeriod.SFN.Number := (v_Timing_SFN.SFN.Number + (v_ModificationPeriodFrames -(v_Timing_SFN.SFN.Number mod v_ModificationPeriodFrames)))mod 1024; v_TimeOfPaging := v_StartNextModPeriod; v_StartNextModPeriod.HSFN.Number := (v_StartNextModPeriod.HSFN.Number + (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames)/1024) mod 1024; v_StartNextModPeriod.SFN.Number := (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames) mod 1024; } if ((p_UE_State == RRC_CONNECTED) and (pc_ETWS_message or pc_CMAS_Message)) { // When UE is in connected mode and UE suppports ETWS/CMAS, the paging is sent during a pagingCycle // Send Paging messages at the next PagingCycle // Send SYSINFO at the next modificationPeriod v_StartNextPagingCycle := (v_Timing_SFN.SFN.Number + (v_PagingCycle -(v_Timing_SFN.SFN.Number mod v_PagingCycle)))mod 1024; v_TimeOfPaging.HSFN.Number := (v_Timing_SFN.HSFN.Number + (v_Timing_SFN.SFN.Number + (v_PagingCycle -(v_Timing_SFN.SFN.Number mod v_PagingCycle)))/ 1024 )mod 1024; //@sic R5s170425 sic@ v_TimeOfPaging.SFN.Number := v_StartNextPagingCycle; //@sic R5s170070 sic@ v_StartNextModPeriod.HSFN.Number := (v_TimeOfPaging.HSFN.Number + (v_StartNextPagingCycle + (v_ModificationPeriodFrames -(v_StartNextPagingCycle mod v_ModificationPeriodFrames)))/ 1024) mod 1024; v_StartNextModPeriod.SFN.Number := (v_StartNextPagingCycle + (v_ModificationPeriodFrames -(v_StartNextPagingCycle mod v_ModificationPeriodFrames)))mod 1024; } // Delay to ensure the sequencing of Scheduling of SIB modification and Paging @sic R5s130949 sic@ f_Delay(int2float((v_TimeOfPaging.SFN.Number-v_Timing.SFN.Number+1024) mod 1024)/100.0); //Page UE from the start of v_TimeOfPaging //based on above calculation f_EUTRA_UE_PageSysinfoMod(p_CellId, v_TimeOfPaging, p_UE_State); // @sic R5-110803 change 4: add new parameter sic@ } f_EUTRA_SS_ConfigureSysinfo (p_CellId, cs_TimingInfo_SFN(v_StartNextModPeriod.SFN.Number, v_StartNextModPeriod.HSFN.Number), v_Sysinfo.SB1_ValueTagChanged, p_StopSib1Transmission); // @sic R5s120130 sic@ } case(sysinfoAndBR){ v_StartNextModPeriod := f_EUTRA_ModifySysinfo_ValueTagBR(p_CellId, p_PagingFlag, p_StopSib1Transmission); } case else {FatalError (__FILE__, __LINE__, "Invalid SysinfoType");} } return v_StartNextModPeriod; //@sic R5s160738 sic@ }