3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240653

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4864 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 8.5.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 24wk37 Implementation of the function f_TC_8_5_5_1_EUTRA : - When extracting the bands to be requested from all the supported bands , at step 1 : v_BandToBeAdded := {v_Index}; assignment is used . As v_index is used as an iterator in the bit string of v_SuppEutraBandsAccToPICS , v_index = 0 actually represents the band support 1 . So it is proposed to use v_BandToBeAdded := {v_Index+1}; Also it is proposed to make below condition as “and” instead of “or” (lengthof(v_RequestedFrequencyBands) < 16) and (v_Index < lengthof(v_SuppEutraBandsAccToPICS)) - At step 10 , UE is expected to do reestablishment request on Eutra Cell 1 . to be prepared for the reeestablishment procedure , SRBs and DRBs to be reset on this cell , so that SN is in line with reestablishment procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | - v_BandToBeAdded := {v_Index+1}; assignment to be made to have the right band supported information - SRB and DRB reset to be done before the reestablishment procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confirmant UE may fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change to the function f_TC_8_5_5_1_EUTRA	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the function f_TC_8_5_5_1_EUTRA

| Function name | f_TC_8_5_5_1_EUTRA |
| --- | --- |
| Reason for change | In 24wk37 Implementation of the function f_TC_8_5_5_1_EUTRA : - When extracting the bands to be requested from all the supported bands , at step 1 : v_BandToBeAdded := {v_Index}; assignment is used . As v_index is used as an iterator in the bit string of v_SuppEutraBandsAccToPICS , v_index = 0 actually represents the band support 1 . So it is proposed to use v_BandToBeAdded := {v_Index+1}; Also it is proposed to make below condition as “and” instead of “or” (lengthof(v_RequestedFrequencyBands) < 16) and (v_Index < lengthof(v_SuppEutraBandsAccToPICS)) - At step 10 , UE is expected to do reestablishment request on Eutra Cell 1 . to be prepared for the reeestablishment procedure , SRBs and DRBs to be reset on this cell , so that SN is in line with reestablishment procedure |
| Summary of change | - v_BandToBeAdded := {v_Index+1}; assignment to be made to have the right band supported information - SRB and DRB reset to be done before the reestablishment procedure |
| TTCN module | ttcn/develop/LTE_A_PRO/8_5/RRC_Others_PRO.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_5_5_1_EUTRA() runs on EUTRA_PTC { /* RACS / UL Message Segment transfer / UECapabilityInformation / Success */ var PhysCellId v_PhysicalCellIdentity_Cell1; var ShortMAC_I v_ShortMAC_I; var EUTRA_SecurityParams_Type v_AuthParams; var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15) var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var SRB_COMMON_IND v_Received_DedicatedMessageSegment; var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); // The position on this list represents the band number var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand}; // Initialise with the primary frequency band var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded; var integer v_Index := 0; var SubFrameTiming_Type v_TimingInfoStep4; var SubFrameTiming_Type v_TimingInfoStep5; var SubFrameTiming_Type v_TimingInfoStep6; var integer v_GrantPeriod; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; timer t_Watchdog := 5.0; f_EUTRA_Init(c1); v_PhysicalCellIdentity_Cell1 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1); //@sic R5s240175 sic@ //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1); /* Preamble to enter UE in state Generic RB Established (state 3) */ f_EUTRA_Preamble (eutra_Cell1, STATE2_IDLEUPDATE); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); // Get bands to be requested, from all the supported bands do { if(v_SuppEutraBandsAccToPICS[v_Index]== '1'B and v_Index != px_ePrimaryFrequencyBand) { // if the UE supposts that band and it is not the primary band (which is already added in the list) v_BandToBeAdded := {v_Index}; v_RequestedFrequencyBands := v_RequestedFrequencyBands & v_BandToBeAdded; // add supported band to the list } v_Index := v_Index + 1; } while ((lengthof(v_RequestedFrequencyBands) < 16) or (v_Index < lengthof(v_SuppEutraBandsAccToPICS))); //@siclog "Step 1" siclog@ // The SS transmits a UECapabilityEnquiry message including eutra to enquire max capability and with rrc-segAllowed-r16 set to enabled. v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily. } else { v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily. } //@sic R5-231397 sic@ // The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(0)))); f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now); //@siclog "Step 7" siclog@ // The SS changes Cell 1 parameters according to the row "T1" in table 8.5.5.1.3.2-1 in order that the radio link quality of Cell 1 is degraded. f_EUTRA_SetCellPower (eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE); ……. }

After Change:

function f_TC_8_5_5_1_EUTRA() runs on EUTRA_PTC { /* RACS / UL Message Segment transfer / UECapabilityInformation / Success */ var PhysCellId v_PhysicalCellIdentity_Cell1; var ShortMAC_I v_ShortMAC_I; var EUTRA_SecurityParams_Type v_AuthParams; var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15) var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var SRB_COMMON_IND v_Received_DedicatedMessageSegment; var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); // The position on this list represents the band number var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand}; // Initialise with the primary frequency band var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded; var integer v_Index := 0; var SubFrameTiming_Type v_TimingInfoStep4; var SubFrameTiming_Type v_TimingInfoStep5; var SubFrameTiming_Type v_TimingInfoStep6; var integer v_GrantPeriod; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; timer t_Watchdog := 5.0; f_EUTRA_Init(c1); v_PhysicalCellIdentity_Cell1 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1); //@sic R5s240175 sic@ //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1); /* Preamble to enter UE in state Generic RB Established (state 3) */ f_EUTRA_Preamble (eutra_Cell1, STATE2_IDLEUPDATE); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); // Get bands to be requested, from all the supported bands do { if(v_SuppEutraBandsAccToPICS[v_Index]== '1'B and (v_Index+1) != px_ePrimaryFrequencyBand) { // if the UE supposts that band and it is not the primary band (which is already added in the list) //WA#WI=1323866 v_BandToBeAdded := {v_Index+1}; //WA#WI=1323866 v_RequestedFrequencyBands := v_RequestedFrequencyBands & v_BandToBeAdded; // add supported band to the list } v_Index := v_Index + 1; //WA#WI=1323866 } while ((lengthof(v_RequestedFrequencyBands) < 16) and (v_Index < lengthof(v_SuppEutraBandsAccToPICS))); //@siclog "Step 1" siclog@ // The SS transmits a UECapabilityEnquiry message including eutra to enquire max capability and with rrc-segAllowed-r16 set to enabled. v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily. } else { v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily. } //@sic R5-231397 sic@ // The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(0)))); f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now); f_EUTRA_SS_SRBs_DRBs_Reset (eutra_Cell1); //WA#WI=1323866 //@siclog "Step 7" siclog@ // The SS changes Cell 1 parameters according to the row "T1" in table 8.5.5.1.3.2-1 in order that the radio link quality of Cell 1 is degraded. f_EUTRA_SetCellPower (eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE); …… }