3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240655

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4866 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to current TTCN implementation, At Step 17a2 the SS release the rrc connection. After that, at Step 17b2A a timer of 30s is started. At Step 17b2Bb1 the timer expires, and then at Step 17b2Bb2 there is another rrc connection release. This is not needed. It is observed that the guard timer of 300s is sometimes not enough for the testcase when run wth a NTN device. This also needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the rrc connection rlease at Step 17b2Bb2. A Prose CR will be raised for this. Guard timer is increased to 600s. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | These changes are needed in addition to those mentioned in R5s240572 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | fl_TC_22_5_20_Body |
| --- | --- |
| Reason for change | According to current TTCN implementation, At Step 17a2 the SS release the rrc connection. After that, at Step 17b2A a timer of 30s is started. At Step 17b2Bb1 the timer expires, and then at Step 17b2Bb2 there is another rrc connection release. This is not needed. |
| Summary of change | Removed the rrc connection rlease at Step 17b2Bb2. A Prose CR will be raised for this. |
| TTCN module | ENBIOT_NAS.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> t_Wait.start(5.0); alt { //@siclog "Step 17a1" siclog@ //@sic R5-195313 sic@ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, // @sic R5s190574 sic@ tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer('0003'O, v_UserData&v_UserData&v_UserData))))) { t_Wait.stop; //@siclog "Step 17a2" siclog@ f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1); //@sic R5-245482 sic@ } //@siclog "Step 17b1" siclog@ //@sic R5-195313 sic@ [] t_Wait.timeout { //The SS transmits an RRCConnectionRelease-NB message //@siclog "Step 17b2" siclog@ //@sic R5-195313 sic@ f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1); // @sic R5s190574 sic@ //@siclog "Step 17b2A" siclog@ //@sic R5-245482 sic@ t_Timer_2.start(30.0); alt { [] SRB.check(receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(cr_EstablishmentCause_NB_Any, ?)))) { t_Timer_2.stop; //@siclog "Step 17b2Ba1 - 17b2Ba3" siclog@ //@sic R5-195313 sic@ v_NAS_Ind := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell1, // @sic R5s190574 sic@ ?, cr_NAS_IndicationWithPiggybacking (tsc_SHT_IntegrityProtected_PartiallyCiphered, cr_CONTROL_PLANE_SERVICE_REQUEST(?), cr_ESM_DATA_TRANSPORT (?, cr_UserDataContainer('0003'O, v_UserData&v_UserData&v_UserData)))); f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 19"); //@siclog "Step 17b2Ba4" siclog@ //@sic R5-195313 sic@ SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, // @sic R5s190574 sic@ tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); //@siclog "Step 17b2Ba5" siclog@ f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1); //@sic R5-245482 sic@ } //@siclog "Step 17b2Bb1" siclog@ [] t_Timer_2.timeout { f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1); //@sic R5-245482 sic@ } } } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> t_Wait.start(5.0); alt { //@siclog "Step 17a1" siclog@ //@sic R5-195313 sic@ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, // @sic R5s190574 sic@ tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer('0003'O, v_UserData&v_UserData&v_UserData))))) { t_Wait.stop; //@siclog "Step 17a2" siclog@ f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1); //@sic R5-245482 sic@ } //@siclog "Step 17b1" siclog@ //@sic R5-195313 sic@ [] t_Wait.timeout { //The SS transmits an RRCConnectionRelease-NB message //@siclog "Step 17b2" siclog@ //@sic R5-195313 sic@ f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1); // @sic R5s190574 sic@ //@siclog "Step 17b2A" siclog@ //@sic R5-245482 sic@ t_Timer_2.start(30.0); alt { [] SRB.check(receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(cr_EstablishmentCause_NB_Any, ?)))) { t_Timer_2.stop; //@siclog "Step 17b2Ba1 - 17b2Ba3" siclog@ //@sic R5-195313 sic@ v_NAS_Ind := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell1, // @sic R5s190574 sic@ ?, cr_NAS_IndicationWithPiggybacking (tsc_SHT_IntegrityProtected_PartiallyCiphered, cr_CONTROL_PLANE_SERVICE_REQUEST(?), cr_ESM_DATA_TRANSPORT (?, cr_UserDataContainer('0003'O, v_UserData&v_UserData&v_UserData)))); f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 19"); //@siclog "Step 17b2Ba4" siclog@ //@sic R5-195313 sic@ SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, // @sic R5s190574 sic@ tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); //@siclog "Step 17b2Ba5" siclog@ f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1); //@sic R5-245482 sic@ } //@siclog "Step 17b2Bb1" siclog@ [] t_Timer_2.timeout { //WA#WI1348961 WA#22_5_20 COMMENTED OUT f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1); //@sic R5-245482 sic@ } } } } <<SKIPPED CODE>>

## Change 1

| Template name | fl_TC_22_5_20_Body |
| --- | --- |
| Reason for change | It is observed that the guard timer of 300s is sometimes not enough for the testcase when run wth a NTN device. This also needs to be addressed. |
| Summary of change | Guard timer is increased to 600s. |
| TTCN module | NBIOT_Testsuite.ttcn |
| MCC160 Comment |  |

Before Change:

testcase TC_22_5_20() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT/ UE in NB-S1 mode supporting control plane data back-off timer/ Service reject with extended wait time CP data/ Release with extended wait time CP data/ Attach accept with extended wait time CP data var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(300); v_NBIOT := NBIOT_PTC.create("NBIOT") alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_5_20_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

After Change:

testcase TC_22_5_20() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT/ UE in NB-S1 mode supporting control plane data back-off timer/ Service reject with extended wait time CP data/ Release with extended wait time CP data/ Attach accept with extended wait time CP data var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(600); //WA#WI1348961 WA#22_5_20 300-->600 v_NBIOT := NBIOT_PTC.create("NBIOT") alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_5_20_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }