3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240669

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3529 | rev | 1 | Current version: | 18.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC testcase 8.1.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint,TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS38.523-1 Table 8.1.1.2.1.3.2-1 and Table 8.1.1.2.1.3.2-2, the initial conditions for T0 are not set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set downlink power levels for T0 before step1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The prose and the ttcn implementation are inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1	Corrections required	3

1.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: Ting Gao

gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_2_1_TestBody |
| --- | --- |
| Reason for change | According to TS38.523-1 Table 8.1.1.2.1.3.2-1 and Table 8.1.1.2.1.3.2-2, the initial conditions for T0 are not set. |
| Summary of change | Set downlink power levels for T0 before step1. |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC { //@sic R5-197054 sic@ var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList; var float v_Wait := 5.0; //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); //@siclog "Step 3" siclog@ //The SS waits for 2s (T300 expire). f_Delay(2.0); //@siclog "Step 4" siclog@ //Check: Does UE send RRCSetupRequest in 5 second? if (f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, -, omit)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Step 5" siclog@ //Check: Does the test result of generic test procedure step 1-8 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE? f_NR5GC_CheckRRCIdle(nr_Cell1, noRrcConnectionRelease); //@siclog "Step 6" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE f_NR_RRCRelease(nr_Cell1); //@siclog "Step 7" siclog@ //The SS remove connEstFailOffset in SIB1. f_NR_CellInfo_SetSIB1_ConnEstFailureControl(nr_Cell1, cs_ConnEstFailureControl_TC81121); //@siclog "Step 8" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS //@siclog "Step 8A" siclog@ //@sic R5-201216 sic@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 9" siclog@ //void //@siclog "Step 10" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 11" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); //@siclog "Step 12" siclog@ //The SS waits for 2s to let T300 expire. f_Delay(2.0); //@siclog "Step 12A" siclog@ //SS re-adjusts the SS levels according to row "T1" in table 8.1.1.2.1.3.2-1/2. f_NR_SetCellPower (nr_Cell1, -78, -82); f_NR_SetCellPower (nr_Cell3, -88, -91); f_Delay(35.0); … }

After Change

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC { //@sic R5-197054 sic@ var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList; var float v_Wait := 5.0; //SS re-adjusts the SS levels according to row "T0" in table 8.1.1.2.1.3.2-1/2. f_NR_SetCellPower (nr_Cell1, -78, -82); //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); //@siclog "Step 3" siclog@ //The SS waits for 2s (T300 expire). f_Delay(2.0); //@siclog "Step 4" siclog@ //Check: Does UE send RRCSetupRequest in 5 second? if (f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, -, omit)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Step 5" siclog@ //Check: Does the test result of generic test procedure step 1-8 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE? f_NR5GC_CheckRRCIdle(nr_Cell1, noRrcConnectionRelease); //@siclog "Step 6" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE f_NR_RRCRelease(nr_Cell1); //@siclog "Step 7" siclog@ //The SS remove connEstFailOffset in SIB1. f_NR_CellInfo_SetSIB1_ConnEstFailureControl(nr_Cell1, cs_ConnEstFailureControl_TC81121); //@siclog "Step 8" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS //@siclog "Step 8A" siclog@ //@sic R5-201216 sic@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 9" siclog@ //void //@siclog "Step 10" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 11" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); //@siclog "Step 12" siclog@ //The SS waits for 2s to let T300 expire. f_Delay(2.0); //@siclog "Step 12A" siclog@ //SS re-adjusts the SS levels according to row "T1" in table 8.1.1.2.1.3.2-1/2. //f_NR_SetCellPower (nr_Cell1, -78, -82); f_NR_SetCellPower (nr_Cell3, -88, -91); f_Delay(35.0); … }