3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240696

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3596 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT test case 8.1.1.3.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The cell power level for conducted test environment is not set correctly in the current implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the correct power level for conducted test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.7a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | fl_TC_8_1_1_3_7a_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | The cell power level for conducted test environment is not set correctly in the current implementation. |
| Summary of change | Set the correct power level for conducted test. |
| TTCN module | RRC_ConnRel_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_1_3_7a_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var integer v_EUTRA_CellPower_T1 := -96;// @sic R5-234558 sic@ var octetstring v_T325_InMillisec; var boolean v_FlagEutra := false; var float v_T325; var float v_ReleaseMilliSecondsDelay := 300.0; // @sic R5s211680 sic@ timer t_WaitTime; //@siclog "Step 1" siclog@ //The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7a.3.2-1/2 f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA38_SetCellPower(eutra_Cell1, v_EUTRA_CellPower_T1, v_EUTRA_CellPower_T1); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 3" siclog@ //SS starts timer T325. v_T325_InMillisec := f_EUTRA_NR_WaitForCoOrd_Data(NR, "T325"); v_T325 := int2float(oct2int(v_T325_InMillisec)/1000); f_Delay(v_ReleaseMilliSecondsDelay/1000.0); // Wait for sending NR RRCRelease @sic R5s211680 sic@ t_WaitTime.start(v_T325); <<< Skipped >>> }

After Change:

function fl_TC_8_1_1_3_7a_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var integer v_EUTRA_CellPower_T1_Conducted := -75; var integer v_EUTRA_CellPower_T1 := -96;// @sic R5-234558 sic@ var octetstring v_T325_InMillisec; var boolean v_FlagEutra := false; var float v_T325; var float v_ReleaseMilliSecondsDelay := 300.0; // @sic R5s211680 sic@ timer t_WaitTime; //@siclog "Step 1" siclog@ //The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7a.3.2-1/2 f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA38_SetCellPower(eutra_Cell1, v_EUTRA_CellPower_T1_Conducted, v_EUTRA_CellPower_T1); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 3" siclog@ //SS starts timer T325. v_T325_InMillisec := f_EUTRA_NR_WaitForCoOrd_Data(NR, "T325"); v_T325 := int2float(oct2int(v_T325_InMillisec)/1000); f_Delay(v_ReleaseMilliSecondsDelay/1000.0); // Wait for sending NR RRCRelease @sic R5s211680 sic@ t_WaitTime.start(v_T325); <<< Skipped >>> }