3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240440

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3473 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - After receiving the RRCResume message, there may be time difference between UE receiving RRCResume and UE applying the new RRC config. carried in RRCResume. Some UE implementations transmit the RLC Ack (for the rrcResume) using RACH as soon as the rrcResume is received without applying the configuration received within rrcResume and before transmitting rrcResumeComplete. Proposal is to make the rrcResume cases flexible to receive the resume complete either using UL DCI Grant or RAR grant. So, SS is required to configure a short UL periodic to consider this Random Access procedure successfully completed by UE if it gets into below clause of TS 38.321, section 5.1.5 Contention Resolution. i.e., 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers: 2> if the C-RNTI MAC CE was included in Msg3: 3> if the Random Access procedure was initiated for SpCell beam failure recovery or for beam failure recovery of both BFD-RS sets of SpCell (as specified in clause 5.17) and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 4> consider this Contention Resolution successful; 4> stop ra-ContentionResolutionTimer; 4> discard the TEMPORARY_C-RNTI; 4> consider this Random Access procedure successfully completed. 2> else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its TEMPORARY_C-RNTI: 2. RRCResumeComplete has been received so SS can switch back to SR based UL Grant configurations now. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added a flag p_StartGrantPeriodic to enable Periodic grant configuration before sending rrcResume in function f_NR_SendRRCResume_Def. 2. Added a flag p_SRGrantEnable to control the UL Grant configuration as SR based in function f_NR_RRCResumeComplete_Def. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | .6.4.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Other NR5GC rrcResume test cases like 8.1.1.4.1, 8.1.1.4.2, 8.1.5.9.3, 8.1.6.1.4.2, 11.3.3 etc are impacted as well. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 TTCN-3 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_NR_SendRRCResume_Def |
| --- | --- |
| Reason for change | After receiving the RRCResume message, there may be time difference between UE receiving RRCResume and UE applying the new RRC configuraition received in in RRCResume. Some UE implementations transmit the RLC Ack (for the rrcResume) using RACH as soon as the rrcResume is received without applying the configuration received within rrcResume and before transmitting rrcResumeComplete. Proposal is to make the rrcResume cases flexible to receive the resume complete either using UL DCI Grant or RAR grant. So, SS is required to configure a short UL periodic grant to consider this Random Access procedure successfully completed by UE if it gets into below clause of TS 38.321, section 5.1.5 Contention Resolution. i.e., 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers: 2> if the C-RNTI MAC CE was included in Msg3: 3> if the Random Access procedure was initiated for SpCell beam failure recovery or for beam failure recovery of both BFD-RS sets of SpCell (as specified in clause 5.17) and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 4> consider this Contention Resolution successful; 4> stop ra-ContentionResolutionTimer; 4> discard the TEMPORARY_C-RNTI; 4> consider this Random Access procedure successfully completed. 2> else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its TEMPORARY_C-RNTI: |
| Summary of change | Added a flag p_StartGrantPeriodic to enable Periodic grant configuration before sending rrcResume in function f_NR_SendRRCResume_Def. |
| TTCN module | NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 | Accepted in principle. As the issue is related to the random access procedure, it is cleaner to modify the random access in the SS: when bringing UE to RRC_INACTIVE state the SS RACH procedure is reconfigured to a list of 2 RACH procedures: RACH CBRA with UE contention resolution Id MAC CE, RACH CBRA with C-RNTI. When the Resume procedure is completes, SS RACH procedure is reconfigured to the default one. |

Before Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, template (omit) MeasConfig p_MeasConfig := omit, template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit ) runs on NR5GC_PTC { //@sic R5-201646: added p_RadioBearerConfig sic@ //@sic R5-204441: added p_MeasConfig sic@ //@sic R5-241544: for SDT added v_RrcResume sic@ var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); var template (value) DL_DCCH_Message v_RrcResume := f_NR_38508_Get_RRCResume_Def(p_NR_CellId, p_RRC_TI, p_CellGroupConfig, p_RadioBearerConfig, p_MeasConfig, p_RRCResume_v1560_IEs); // Switch on UL grant assignments f_NR_ULGrantConfiguration_Start(p_NR_CellId); // Refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters //@sic R5s210578 sic@ //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo); //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo); //@sic R5s210578 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume)); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }

After Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, template (omit) MeasConfig p_MeasConfig := omit, template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit, boolean p_StartGrantPeriodic := true //WA#WI=1271929 ) runs on NR5GC_PTC { //@sic R5-201646: added p_RadioBearerConfig sic@ //@sic R5-204441: added p_MeasConfig sic@ //@sic R5-241544: for SDT added v_RrcResume sic@ var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); var template (value) DL_DCCH_Message v_RrcResume := f_NR_38508_Get_RRCResume_Def(p_NR_CellId, p_RRC_TI, p_CellGroupConfig, p_RadioBearerConfig, p_MeasConfig, p_RRCResume_v1560_IEs); //WA#WI=1271929 - Periodic grant var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); if(p_StartGrantPeriodic) { if (v_SCS == kHz30) { f_NR_ULGrantConfiguration_StartPeriodicGrant(p_NR_CellId, cs_TimingInfo_SubFrame(2), valueof (cs_NR_ResourceAllocationDef), 10); // periodic grant every 5ms (80% of 12ms the Inactivity Timer) } else { f_NR_ULGrantConfiguration_StartPeriodicGrant(p_NR_CellId, cs_TimingInfo_SubFrame(5), valueof (cs_NR_ResourceAllocationDef), 10); // periodic grant every 5ms (80% of 12ms the Inactivity Timer) } }else { // Switch on UL grant assignments f_NR_ULGrantConfiguration_Start(p_NR_CellId); } // Refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters //@sic R5s210578 sic@ //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo); //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo); //@sic R5s210578 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume)); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }

MCC160 proposed implementation

Create 2 constants:

const boolean tsc_RRC_InactiveFollowedByRRCResume := true;      /* UE will leave RRC INACTIVE state with an RRC Resume procedure and SS sending RRCReseume (eventually RRCResume) */

const boolean tsc_RRC_InactiveNotFollowedByRRCResume := false;  /* UE will NOT leave RRC INACTIVE state with an RRC Resume message */

Create a new template cs_NR_RachProcedureConfig_RRCResume

template (value) NR_RachProcedureConfig_Type cs_NR_RachProcedureConfig_RRCResume(NR_PhysicalParameters_Type p_PhysicalParameters,

RNTI_Value_Type p_TempC_RNTI_Value,

integer p_TBS_Msg2 := tsc_NR_ENDC_Msg3Grant_Def

) :=

{ /* contention based random access procedure containing a list of two  RACH procedures:

- RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI,

- RACH procedure with contention-based with C-RNTI based contention resolution with DCI 0_1.

*/

//@sic R5s240440 sic@

RachProcedureList := {

cs_NR_RachProcedure(p_PhysicalParameters, cs_NR_ContentionResolutionCtrl_Msg4Based(cs_NR_RachProcedureMsg4(p_PhysicalParameters, -, cs_NR_RachProcedureMsg4WithNoRrcMsg))),

cs_NR_RachProcedure(p_PhysicalParameters,

cs_NR_ContentionResolutionCtrl_CrntiBased(p_PhysicalParameters, f_NR_ResourceAllocation_UplinkGrant(tsc_NR_Msg4GrantHO_Def, p_PhysicalParameters.UL_BWPs[0]), -, -, cs_NR_DciFormat_0_1_Params ),

cs_NR_TempC_RNTI_Explicit(p_TempC_RNTI_Value),

p_TBS_Msg2)

}

};

Add a new parameter in f_NR_RRC_InactiveDef: when set to true (tsc_RRC_InactiveFollowedByRRCResume) SS RACH is reconfigured to a list with 2 RACH CBRA MAC CE CNRTI and CBRA CRNTI.

function f_NR_RRC_InactiveDef(NR_CellId_Type p_NR_CellId,

boolean p_FollowedByRRCResume := tsc_RRC_InactiveFollowedByRRCResume,

template (omit) PLMN_Identity p_PLMN_Identity := omit,

boolean p_ActivateSecurity := true,

template (omit) RRCRelease_v1540_IEs p_RRCRelease_v1540 := omit,

template (omit) SuspendConfig.sdt_Config_r17 p_SDT_Config := omit,

boolean p_ReestablishSDT_DRB := false,

template (omit) NR_RachProcedureConfig_Type p_NR_RachProcedureConfig := omit,

template (omit) SuspendConfig.ran_ExtendedPagingCycleConfig_r18  p_RAN_ExtendedPagingCycleConfig_r18 := omit

) runs on NR5GC_PTC return SubFrameTiming_Type

{ //@sic R5-195325 sic@

//@sic R5-243537 added return v_TimingRRCRelease => time at which RRCRelease is sent sic@

//@sic Added p_ReestablishSDT_DRB and p_NR_RachProcedureConfig for SDT tc sic@

//@sic R5s240440: added p_FollowedByRRCResume sic@

var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(p_NR_CellId);

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

var template (omit) NR_RadioBearerList_Type v_RadioBearerList := omit;

var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList();

var SubFrameTiming_Type v_TimingRRCRelease;

var RNTI_Value_Type v_RNTI_Value_Type := f_NR_CellInfo_GetRNTI(p_NR_CellId);

if (p_ReestablishSDT_DRB) {

v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList);   //For SDT: reconfigure all DRBs -> to ensure UE uses only the SDT-DRB

}

if (isvalue(p_NR_RachProcedureConfig)) {

f_NR_SS_RachProcedureConfig(p_NR_CellId, p_NR_RachProcedureConfig);

} else if (p_FollowedByRRCResume) {

//@sic R5s240440 sic@

/* Configure RACH procedure to a list with 2 RACH:

- RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI => To send RRCReseumeRequest

- RACH procedure with contention-based with C-RNTI based contention resolution => In case UE use RACH procedure to send RLC ACK for RRCResume

*/

f_NR_SS_RachProcedureConfig(p_NR_CellId, cs_NR_RachProcedureConfig_RRCResume(v_NR_PhysicalParameters, v_RNTI_Value_Type, tsc_NR_Msg4GrantHO_Def));

}

v_TimingRRCRelease := f_NR_RRCRelease_Common (p_NR_CellId, cs_38508_RRCRelease ( tsc_NR_RRC_TI_Def,

cs_38508_SuspendConfig(cs_RAN_NotificationAreaInfo_1Cell ( v_CellIdentity, p_PLMN_Identity), p_SDT_Config, p_RAN_ExtendedPagingCycleConfig_r18),

-, -, -,

p_RRCRelease_v1540

),

-,

v_RadioBearerList);

if (p_ActivateSecurity){

//Activate security at SS side in advance so that SS is ready to send RRCResume message

// Only SRBs are configured - so security is activated on SRBs

// When DRBs are configured then security is also started on DRBs

//@sic R5s190505 change 2 sic@

f_NR_SS_AS_ActivateSecurity_Current(p_NR_CellId,tsc_NR_38508_NextHopChainingCount);

}

return v_TimingRRCRelease;

}

Pass the new parameter as tsc_RRC_InactiveFollowedByRRCResume in functions and test cases in which SS needs a reconfiguration.

Same change done in :

f_TC_6_4_1_1_NR5GC_TestBody

f_TC_6_4_2_1_NR5GC_TestBody

f_TC_6_4_2_3_NR5GC_TestBody

f_TC_6_4_2_4_NR5GC_TestBody

f_TC_6_4_2_5_NR5GC_TestBody

fl_TC_8_1_1_4_1_TestBody

f_TC_8_1_5_11_3_NR5GC_TestBody

f_TC_8_1_5_11_3_NR5GC_TestBody

f_TC_8_1_5_11_4_NR5GC_TestBody

f_TC_8_1_5_11_4_NR5GC_TestBody

f_TC_8_1_5_13_1_NR5GC_TestBody

f_TC_8_1_5_13_2_NR5GC_TestBody

f_TC_8_1_5_13_3_NR_TestBody

fl_TC_11_7_4_NR5GC_TestBody

fl_TC_8_2_6_3_3_TestBody

fl_TC_8_2_6_3_4_TestBody

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId,

NG_STATE_Type p_State,

NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF,

boolean p_ActivateSecurityInactiveState := true,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (present) B4_Type p_ServiceType := '0000'B,

template (omit) PreamblePreconfigureDataList p_PreconfigureUe := omit)  runs on NR5GC_PTC

{ //@sic R5s190505: added p_ActivateSecurityInactiveState sic@

//@sic R5-199036: added p_IpPduDelayTime sic@

var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@

if (p_State == STATE_INACTIVE_2A) {

v_ReleaseConnection := noRrcConnectionRelease;

}

// send UT functions to UE before cell switched on

fl_NR5GC_PreconfigUE(p_PreconfigureUe); // @sic R5-225300, R5-237389 sic@

// turn on NR cell

f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session

// Switch on UE

f_NR5GC_SwitchOnAndResetIMS();

// Go to RRC_IDLE

f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType); // @sic R5s210189 sic@

}

select (p_State) {

case (STATE_OFF_0B) {

// Now turn off again

f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A);

}

case (STATE_INACTIVE_2A) {

f_NR_RRC_InactiveDef(p_NR_CellId, tsc_RRC_InactiveFollowedByRRCResume, -,  p_ActivateSecurityInactiveState);

}

case (STATE_CONNECTED_3A) {

// Go to RRC_CONNECTED

f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode);

}

}

}

Pass the new parameter as tsc_RRC_InactiveNotFollowedByRRCResume  in functions and test cases in which SS needs no reconfiguration (except EarlyContentionResolution).

Same change done in :

- fl_TC_14_1_2_1_NR5GC_TestBody

f_TC_6_4_1_1_NR5GC_TestBody

f_TC_6_4_1_2_NR5GC

f_TC_6_4_1_2_NR5GC_TestBody

fl_TC_8_1_1_4_2_TestBody

fl_TC_11_7_4_NR5GC_TestBody

fl_TC_11_7_4_NR5GC_TestBody

fl_TC_6_4_3_1_TestBody

f_TC_6_4_2_2_NR5GC_TestBody

function f_TC_11_7_3_NR5GC_TestBody() runs on NR5GC_PTC

{ // @sic R5s230900 R5-241377 sic@

var template (value) ExtdDRXParams v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0000'B, '0001'B, '00000001'B);// Step 14 - Table 11.7.3.3.3-3: REGISTRATION ACCEPT

var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1);

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template (value) SuspendConfig v_SuspendConfig;

var NG_NAS_GutiParameters_Type v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1);

var NG_5G_S_TMSI v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters);

var PCCH_Config v_PcchConfig := f_NR_GetSIB1_PcchConfig(nr_Cell1);

var PagingCycle v_PagingCycle;

//@siclog "Step 1" siclog@

//The UE is switched on

f_NR5GC_SwitchOnAndResetIMS();

//@siclog "Step 2-15" siclog@

//Steps 19B-19C of the generic procedure for NR RRC_INACTIVE as specified in TS 38.508-1 Table 4.5.3.2-1 are performed.

//The UE transmits a REGISTRATION REQUEST message including  the Requested extended DRX parameters IE

//Steps 19E-19M of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 are performed.

//The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE?

// Note 1:  In NR connected to 5GCN, the Paging Time Window field is ignored and the PTW value is included in the Extended Paging Time Window field.

// The UE transmits a REGISTRATION COMPLETE message

f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams);

//@siclog "Steps 16-17" siclog@

// Steps 19TAa1-20 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 are performed.

f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1);

f_NR_RRC_InactiveDef(nr_Cell1, tsc_RRC_InactiveNotFollowedByRRCResume, -, false);

//@siclog "Steps 18" siclog@

// The SS transmits a Paging message within a paging occasion.

f_NR5GC_UE_Page_eDRX(nr_Cell1,

cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI)), /* @sic R5-236166 sic@ */

1, 1);

//@siclog "Step 19" siclog@

// Check: Does the UE transmits an RRCSetupRequest message indicating that UE has moved to RRC_IDLE state?

f_NR_RRC_SetupRequest_Def(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.3 Step 19");

## Change 2

| Template name | f_NR_RRCResumeComplete_Def |
| --- | --- |
| Reason for change | 1. RRCResumeComplete has been received so SS can switch back to SR based UL Grant configurations now. |
| Summary of change | 1. Added a flag p_SRGrantEnable to control the UL Grant configuration as SR based in function f_NR_RRCResumeComplete_Def. |
| TTCN module | NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 | See change 1 - reconfigure RACH procedure as for initial access Same change done in fl_TC_8_1_1_1_1_TestBody f_TC_8_1_5_8_2_x_NR5GC_TestBody f_TC_8_1_5_8_1_NR5GC_TestBody f_TC_8_1_5_9_3_NR5GC_TestBody f_TC_8_2_6_2_4_NR5GC_TestBody fl_TC_8_2_7_2_1_TestBody fl_TC_11_3_1a_TestBody fl_TC_11_3_3_TestBody fl_TC_11_3_6and6a_TestBody fl_TC_11_3_11_TestBody |

Before Change:

function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId, template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete) runs on NR_BASE_PTC return template (omit) NG_NAS_MSG_Indication_Type { var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; //The UE transmits an RRCResumeComplete message. alt { // @sic R5s200403 sic@ [] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{} [] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{ v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0]; } } return v_DedicatedNAS_Message; }

After Change:

* p_SRGrantEnable = Set to false if periodic Grant was off in f_NR_SendRRCResume_Def. //WA#WI=1271929 */ function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId, template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete, boolean p_SRGrantEnable := true) //WA#WI=1271929 runs on NR_BASE_PTC return template (omit) NG_NAS_MSG_Indication_Type { var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; //The UE transmits an RRCResumeComplete message. alt { // @sic R5s200403 sic@ [] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{} [] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{ v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0]; } } if(p_SRGrantEnable) //WA#WI=1271929 { // The SS starts the UL default(SR Based) grant transmission. f_NR_ULGrantConfiguration_Start(p_NR_CellId); } return v_DedicatedNAS_Message; }

MCC160 proposed implementation

function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId,

template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete)

runs on NR_BASE_PTC return  template (omit) NG_NAS_MSG_Indication_Type

{

var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@

var NR_SRB_COMMON_IND v_ReceivedAsp;

//The UE transmits an RRCResumeComplete message.

alt { // @sic R5s200403 sic@

[] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{}

[] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{

v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0];

}

}

// Reconfigure SS - set DcchDtch on USS with DCI format 1_1

//@sic R5s240461 sic@

f_NR_SS_ReconfigSStoUSS_DCI1_1(p_NR_CellId);

// Reconfigure RACH procedure as for initial access.  @sic R5s240440 sic@

f_NR_SS_RachProcedureConfig_Def(p_NR_CellId);

return v_DedicatedNAS_Message;

}