3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240573

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3532 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_EUTRA_RRC_Procedure_Latency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of function f_EUTRA_RRC_Procedure_Latency, t_Timer is started with buffer time of 50 ms . To improve the stability of this case and also take care of Harq retransmissions it is proposed to increase the buffer time from 50ms to 150ms . Increasing this timer does not affect the test purpose , as the latency check is in place to make sure the correct delay is used between Harq ACK reception and SRB COMMON IND Uplink message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to increase the buffer time of the t_Watchdog from 50ms to 150ms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will remain unstable and may fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_EUTRA_RRC_Procedure_Latency |
| --- | --- |
| Reason for change | In the current TTCN implementation of function f_EUTRA_RRC_Procedure_Latency, t_Timer is started with buffer time of 50 ms . To improve the stability of this case and also take care of Harq retransmissions it is proposed to increase the buffer time from 50ms to 150ms . Increasing this timer does not affect the test purpose , as the latency check is in place to make sure the correct delay is used between Harq ACK reception and SRB COMMON IND Uplink message. |
| Summary of change | It is proposed to increase the buffer time of the t_Watchdog from 50ms to 150ms. |
| TTCN module | ttcn/develop/Common/EUTRA/EUTRA_CommonProcedures.ttcn |
| MCC160 Comment | Accepted in principle. Further improvements: - Handling of additional UL HARCK acks after the first ack - Activation of HARQ error reporting where not done yet |

Before Change:

function f_EUTRA_RRC_Procedure_Latency( EUTRA_CellId_Type p_CellId, template (value) SRB_COMMON_REQ p_SRB_COMMON_REQ_1, template (omit) SRB_COMMON_REQ p_SRB_COMMON_REQ_2 := omit, template (present) SRB_COMMON_IND p_SRB_COMMON_IND_1, template SRB_COMMON_IND p_SRB_COMMON_IND_2 := omit, boolean p_IsFDD, integer p_N, charstring p_TestcaseStep, charstring p_UE_Cat, //@sic R5-153973 to be set to true for UE capability sic@ boolean p_BigULGrant := false, integer p_DelayGrantToDataAddition := 0) // @sic R5s230223 sic@ runs on EUTRA_PTC return SRB_COMMON_IND { ……………. v_T5 := f_EUTRA_TimingInfoAdd(v_T0, ((v_GrantCycle * (v_NoOfGrants - 1)) + p_N)); // @sic R5s200666 sic@ /* T5: last UL grant is sent out; @sic R5s130951 sic@ */ v_Tnow := f_EUTRA_GetCurrentTiming(p_CellId); t_Timer.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T5) + 50) / 1000.0); /* 50ms added to be at the safe side */ …… }

After Change:

function f_EUTRA_RRC_Procedure_Latency( EUTRA_CellId_Type p_CellId, template (value) SRB_COMMON_REQ p_SRB_COMMON_REQ_1, template (omit) SRB_COMMON_REQ p_SRB_COMMON_REQ_2 := omit, template (present) SRB_COMMON_IND p_SRB_COMMON_IND_1, template SRB_COMMON_IND p_SRB_COMMON_IND_2 := omit, boolean p_IsFDD, integer p_N, charstring p_TestcaseStep, charstring p_UE_Cat, //@sic R5-153973 to be set to true for UE capability sic@ boolean p_BigULGrant := false, integer p_DelayGrantToDataAddition := 0) // @sic R5s230223 sic@ runs on EUTRA_PTC return SRB_COMMON_IND { ……………. v_T5 := f_EUTRA_TimingInfoAdd(v_T0, ((v_GrantCycle * (v_NoOfGrants - 1)) + p_N)); // @sic R5s200666 sic@ /* T5: last UL grant is sent out; @sic R5s130951 sic@ */ v_Tnow := f_EUTRA_GetCurrentTiming(p_CellId); t_Timer.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T5) + 150) / 1000.0); /* 150ms added to be at the safe side */ //WA#WI=1318859 …… }

## MCC160 Implementation

function f_EUTRA_RRC_Procedure_Latency( EUTRA_CellId_Type p_CellId, template (value) SRB_COMMON_REQ p_SRB_COMMON_REQ_1, template (omit) SRB_COMMON_REQ p_SRB_COMMON_REQ_2 := omit, template (present) SRB_COMMON_IND p_SRB_COMMON_IND_1, template SRB_COMMON_IND p_SRB_COMMON_IND_2 := omit, boolean p_IsFDD, integer p_N, charstring p_TestcaseStep, charstring p_UE_Cat, //@sic R5-153973 to be set to true for UE capability sic@ boolean p_BigULGrant := false, integer p_DelayGrantToDataAddition := 0) // @sic R5s230223 sic@ runs on EUTRA_PTC return SRB_COMMON_IND { //@sic R5-152105 sic@ @sic R5s150734 R5-153973 R5-167569: p_UE_Cat->charstring sic@ var SYSTEM_IND v_SYSTEM_IND; var SRB_COMMON_IND v_SRB_COMMON_IND; var boolean v_NasFlag := ispresent(p_SRB_COMMON_REQ_1.Signalling.Nas);// @sic R5s200666 sic@ var boolean v_T1Flag := false; var boolean v_T3Flag := false; var boolean v_T4Flag := (not isvalue(p_SRB_COMMON_REQ_2)); // we only check for a second UL PDU when there is a second DL PDU var integer v_NoOfGrants := 5; var integer v_GrantCycle; var integer v_DelayGrantToData; var SubFrameTiming_Type v_T0; // Sending of DL RRC PDU(s) var SubFrameTiming_Type v_T1; // HARQ ACK for DL RRC PDU(s) var SubFrameTiming_Type v_T2; // First UL grant var SubFrameTiming_Type v_T3; // First UL RRC PDU received var SubFrameTiming_Type v_T4; // Second UL RRC PDU received var SubFrameTiming_Type v_T5; // Last UL grant var SubFrameTiming_Type v_Tnow; var TimingInfo_Type v_TimingInfo; var integer v_UlGrantOffset; //@sic R5-144678 R5s141213 sic@ var integer v_I_MCS:=0; // Modulation and Coding Scheme Index @sic R5s150734 sic@ var integer v_N_PRB:=1; // PRBS @sic R5s150734 sic@ timer t_Timer; if (p_IsFDD) { v_GrantCycle := 8; // HARQ RTT = 8 if (pc_FDD_TypeB_HalfDuplex) //@sic R5-182302 R5s180242 sic@ { v_GrantCycle := 10; } v_DelayGrantToData := 5 + p_DelayGrantToDataAddition; // UL data is always sent 4 + 1 TTIs after the corresponding grant @sic R5-144678, R5s230223 sic@ v_UlGrantOffset := 1; //@sic R5-144678 1 TTI added due toR5-144678 sic@ } else { v_GrantCycle := 10; // HARQ RTT = 10 for SN = 6 v_DelayGrantToData := 8; // UL data is sent 4 + 3 + 1 TTIs after the corresponding grant @sic R5-144678 sic@ v_UlGrantOffset := 2; /* @sic R5-144678 R5s141213 1 TTI added due toR5-144678, but that results in sub frame =5 not suitable for grant in TDD config 1, hence 1 more sub frame added to result in SN 6 sic@ */ } //Get timing reference for calculating expected HARQ ACK i.e. when the performance measurement start v_TimingInfo := valueof(p_SRB_COMMON_REQ_1.Common.TimingInfo); v_T0 := v_TimingInfo.SubFrame; /* T0: DL PDU is sent the first time; subframe number shall be 4 * => possible retransmissions are at * T0 + 8, T0 + 16, T0 + 24, T0 + 32 for FDD * T0 + 10, T0 + 20, T0 + 30, T0 + 40 for TDD */ if (pc_ue_CategoryDL_M1) //@sic R5-182302 R5s180242 sic@ { p_SRB_COMMON_REQ_1.Common.TimingInfo.SubFrame.Subframe.Number := 9; if (isvalue(p_SRB_COMMON_REQ_2)) { //@sic R5s180409 sic@ p_SRB_COMMON_REQ_2.Common.TimingInfo.SubFrame.Subframe.Number := 9; } v_T0.Subframe.Number := 1; v_T0.SFN.Number := (v_T0.SFN.Number + 1) mod 1024; //@sic R5s180439 sic@ } v_T2 := f_EUTRA_TimingInfoAdd(v_T0, p_N + v_UlGrantOffset); /* T2: first UL grant is assigned; sub-sequent UL grants are to cope with DL retransmissions @sic R5-144678 R5s141213 sic@ */ if(pc_ue_CategoryDL_M1){ /* If calculation of the activation time results in SF7 for the configuration of the UL grant the UE would schedule PUSCH in SF1 of the next SFN. resulting in a collision with PRACH occupying lowest physical resource blocks 4-9 in SF1. @sic R5s190054 sic@ */ if(v_T2.Subframe.Number == 7){ v_T2.Subframe.Number := v_T2.Subframe.Number + 1; } } //@sic R5s170303 int2char -> int2str sic@ //@sic R5s180538 function rearrangement to make savings sic@ select (p_UE_Cat) { // Table 7.1.7.1.1.3.2-1: Maximum TBsize for different UE categories @sic R5s150734 sic@ //@sic R5-151413 sic@ case (int2str(0), "m1") { // MAX TB size 1000 64QAM Not supported @sic R5s170220 change 1sic@ v_I_MCS := 14 ; // Itbs = 13 v_N_PRB := 4; // results in TB size of 1000 } case (int2str(1), "oneBis") { // MAX TB size 5160 64QAM Not supported @sic R5s190996 sic@ v_I_MCS := 15; // Itbs = 14 v_N_PRB := 18; // results in TB size of 5160 } case else { if (p_BigULGrant) { //@sic R5-153973 sic@ v_I_MCS := 20; //ITbs=19, 16 QAM modulation v_N_PRB := f_ConvertUL_BandwidthToInteger(f_EUTRA_CellInfo_GetUL_ChBandwidth(p_CellId)); if (str2int(p_UE_Cat) == 2 and v_N_PRB > 50) { // the MAX UL TBsize supported by Cat 2 is 25456 v_N_PRB := 58; //truncate N_PRB allocated to result in TBsize of 25456 } } else { //Normal grant v_I_MCS := 20; v_N_PRB := 25; } } } f_EUTRA_ULGrantConfiguration_Common(p_CellId, cs_TimingInfo_EUTRA_NB(v_T2), cs_DciInfo_CcchDcchDtchUL_Explicit(v_I_MCS, v_N_PRB,-, f_EUTRA_FirstRB_UL (p_CellId)), //@sic R5s120099r1 R5s130903 R5s140497 R5s150734 R5s170438 sic@ cs_UplinkTimeAlignment_Synch_Auto_Def,//@sic R5s201232 sic@ cs_UL_GrantConfig_NoOfGrants(v_GrantCycle, v_NoOfGrants)); f_SS_ConfigUL_HARQ_IndMode(p_CellId, tsc_L1Mac_IndicationMode_Enable, v_TimingInfo); /* enable indication of HARQ ack/nack @sic R5s120024: scheduled configuration sic@ */ //@sic R5s120769 sic@ SRB.send(p_SRB_COMMON_REQ_1); /* send (first) DL PDU at T0 */ if (isvalue(p_SRB_COMMON_REQ_2)) { SRB.send(p_SRB_COMMON_REQ_2); /* send second DL PDU at T0 */ } v_T5 := f_EUTRA_TimingInfoAdd(v_T0, ((v_GrantCycle * (v_NoOfGrants - 1)) + p_N)); // @sic R5s200666 sic@ /* T5: last UL grant is sent out; @sic R5s130951 sic@ */ v_Tnow := f_EUTRA_GetCurrentTiming(p_CellId); t_Timer.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T5) + 150) / 1000.0); /* 150ms added to be at the safe side */ alt { [] SYSIND.receive (car_HarqError_IND(p_CellId)) -> value v_SYSTEM_IND { /* HARQ error indication */ if (ischosen(v_SYSTEM_IND.Indication.HarqError.UL)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in latency test"); } else { repeat; } } [] SYSIND.receive (car_UL_HARQ_IND(p_CellId, ?, ?)) -> value v_SYSTEM_IND { /* HARQ ACK => T1 */ if (not v_T1Flag and (v_SYSTEM_IND.Indication.UL_HARQ == ack)) { /* (first) UL HARQ ACK => T1 */ v_T1 := v_SYSTEM_IND.Common.TimingInfo.SubFrame; f_SS_ConfigUL_HARQ_IndMode(p_CellId, tsc_L1Mac_IndicationMode_Disable, -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s120201: no confirmation requested sic@ */ v_T1Flag := true; } repeat; } [not v_T3Flag] SRB.receive(p_SRB_COMMON_IND_1) -> value v_SRB_COMMON_IND { /* First UL RRC PDU => T3 */ v_T3 := v_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T3Flag := true; repeat; // => wait for timeout to be sure that there are no UL grants pending anymore } [not v_T4Flag] SRB.receive(p_SRB_COMMON_IND_2) -> value v_SRB_COMMON_IND { /* Second UL RRC PDU => T4 */ if (not v_T3Flag) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "Incorrect order of UL PDUs"); } v_T4 := v_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; repeat; // => wait for timeout to be sure that there are no UL grants pending anymore } [v_T3Flag and v_NasFlag] SRB.receive(car_SRB2_NasPdu_IND(p_CellId, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, ?))) { repeat; }// @sic R5s200666 sic@ [] t_Timer.timeout { /* timeout => after T5 */ // exit the alt } } if (not v_T1Flag) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "no HARQ ACK for DL PDU"); } if (not v_T3Flag) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "no UL PDU"); } if (not v_T4Flag) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "no second UL PDU"); } // Latency check: if ((f_SubFrameTiming_Duration(v_T1, v_T3) > p_N + v_DelayGrantToData) or /* check first RRC procedure regarding latency requirenments */ (isvalue(p_SRB_COMMON_REQ_2) and (f_SubFrameTiming_Duration(v_T1, v_T4) > p_N + v_DelayGrantToData))) { /* check second RRC procedure regarding latency requirenments */ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } return v_SRB_COMMON_IND; }

## Further Improvements

function fl_TC_8_2_6_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { // Processing delay / PSCell addition / SCG DRB / Success / Latency check / EN-DC var template (omit) octetstring v_NR_SCGConfig := omit; var template (omit) octetstring v_NR_RBConfig := omit; var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; var template (omit) NAS_MSG_RequestList_Type v_ActivateBearer := omit; var template (omit) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs := omit; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1); var charstring v_UE_CategoryExtn; var SubFrameTiming_Type v_Timing; var integer v_N; var boolean v_IsFDD; var SRB_COMMON_IND v_ReceivedMsg; var RRCConnectionReconfigurationComplete_v8a0_IEs v_RRCReconfigCompleteExtn; var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var octetstring v_NR_MRDC_Freqs;// @sic R5s200666 sic@ // enable reporting of HARQ errors f_EUTRA_SS_CommonL1MacIndCtrlConfig(eutra_Cell1, cas_HarqErrorIndication_REQ(eutra_Cell1)); v_IsFDD := (v_EUTRA_FDD_TDD_Mode == FDD); //Get extended UE category v_UE_CategoryExtn := f_EUTRA_MobileInfo_GetUECapability_UECategoryExtn(); // @siclog "Step 1" siclog@ //The SS transmits an RRCConnectionReconfiguration message to add PSCell with SCG DRB // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCConnectionReconfigurationComplete message within 25 (FDD)/28 (TDD) subframes after successful completion of step 1? (Note 2) // Initiate SgNB Keys: v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(eutra_Cell1, MCG_SCG, RRC_CONNECTED); v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, v_Security_Params.SK_Counter); v_Timing := f_EUTRA_GetNextSendOccasion ( eutra_Cell1 ); v_N := 20; v_ReceivedMsg := f_EUTRA_RRC_Procedure_Latency(eutra_Cell1, cas_SRB1_RrcNasPduList_REQ(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_Timing), cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def, omit, omit, v_RadioResourceConfigDedicated, omit, v_RRCConnectionReconfiguration_v890_IEs), v_ActivateBearer), -, car_SRB1_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionReconfigurationComplete_r15), -, v_IsFDD, v_N, "Test Case 8.2.6.2.1 Step 2", v_UE_CategoryExtn); // If the 38.331 RRC Configuration Complete message is present, send it to NR for checking if (ispresent (v_ReceivedMsg.Signalling.Rrc.Dcch.message_.c1.rrcConnectionReconfigurationComplete.criticalExtensions.rrcConnectionReconfigurationComplete_r8.nonCriticalExtension)) { v_RRCReconfigCompleteExtn := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.c1.rrcConnectionReconfigurationComplete.criticalExtensions.rrcConnectionReconfigurationComplete_r8.nonCriticalExtension; if (ispresent (v_RRCReconfigCompleteExtn.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.scg_ConfigResponseNR_r15)) { f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_RRCReconfigCompleteExtn.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.scg_ConfigResponseNR_r15, "Complete")); } } // @siclog "Step 3" siclog@ // The SS sends E-UTRA RRC UECapabilityEnquiry message including RAT-Type eutra-nr // @siclog "Step 4" siclog@ //Check: Does the UE transmit an E-UTRA RRC UECapabilityInformation message within 85 (FDD)/88 (TDD) subframes after successful completion of step 3? (Note 2) f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (int2oct(f_EUTRA_CellInfo_GetBand (eutra_Cell1), 1), "EUTRA Band")); // @sic R5s200666 sic@ v_NR_MRDC_Freqs := f_EUTRA_NR_WaitForCoOrd_Data(NR, "MRDC_Freq"); // @sic R5s200666 sic@ v_Timing := f_EUTRA_GetNextSendOccasion ( eutra_Cell1 ); v_N := 80; f_EUTRA_RRC_Procedure_Latency(eutra_Cell1, cas_SRB1_RrcPdu_REQ(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_Timing), cs_508_UeCapabilityEnquiry_MRDC_r15(tsc_RRC_TI_Def, cs_UE_CapabilityRequest_EUTRA_NR, v_NR_MRDC_Freqs)),// @sic R5s200666 sic@ -, car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_UeCapabilityInformation(tsc_RRC_TI_Def, cr_EUTRA_NRCapInfo)), -, v_IsFDD, v_N, "Test Case 8.2.6.2.1 Step 4", v_UE_CategoryExtn, true ); }

function f_TC_8_2_2_3_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_SCellId, CA_Tested_Type p_CA_Type) runs on EUTRA_PTC { //@sic R5-153722 sic@ var PhysCellId v_PhysicalCellIdentity_Scell; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; var Frequency_IE_Type v_Frequency_IE_Scell; var ChannelBandwidthDependency_Type v_ChBandDependency_SCell; var boolean v_UL_CA; //@sic R5s170752 sic@ var template (value) DL_DCCH_Message v_RRCConnectionReconfiguration; var integer v_Arfcn_SCell; var SCell_DL_Bandwidth_Type v_SCell_DL_Bandwidth; var template (omit) DedicatedSCell_UL_Configuration_Type v_DedicatedSCell_UL_Configuration := omit; var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDDInfo; var template (omit) TDD_Config v_TDD_Config := omit; // omitted in FDD //var template (value) CarrierFreqEUTRA v_CarrierFreq_Scell; var template (value) PUCCH_ConfigDedicated_v1020 v_PUCCH_ConfigDedicated; var RadioResourceConfigCommonSIB v_RadioResourceConfigCommonSIB; var EUTRA_CellSysInfo_Type v_EUTRA_CellSysInfo_Type; var template (value) Scell_Capability_Type v_Scell_Capability := DlOnly; var integer v_N; var boolean v_IsFDD; var template (value) RachProcedureConfig_Type v_RachProcedureConfig; var template (omit) CommonSCell_UL_Configuration_Type v_CommonSCell_UL_Configuration := omit; var template (value) SCellToAddMod_r10 v_SCellToAddMod; var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_TimingInfo2; var charstring v_UE_CategoryExtn; // @sic R5s150734 sic@ var boolean v_FeatrGrp_113 ; //@sic R5-150557 sic@ var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDDInfo_PCell ; //@sic R5-171366 sic@ var template (omit) LAA_SCellConfiguration_r13 v_LAA_SCellConfiguration := omit; var boolean v_LAA_Scell ; f_EUTRA_Init(c3); f_EUTRA_CA_InitCells({p_PCellId, p_SCellId}, p_CA_Type, c3); v_UL_CA :=f_EUTRA_MobileInfo_GetUL_CA(); //@sic R5s170752 sic@ //Get cell dependant parameters v_LAA_Scell := f_EUTRA_CellInfo_IsLAA (p_SCellId); if (v_LAA_Scell) { v_LAA_SCellConfiguration := cs_508_LAA_SCellConfiguration; } v_PhysicalCellIdentity_Scell := f_EUTRA_CellInfo_GetPhyCellId(p_SCellId); v_Frequency_IE_Scell := f_EUTRA_CellInfo_GetFrequencyIEs(p_SCellId); v_ChBandDependency_SCell := f_EUTRA_BandDependentParam(v_Frequency_IE_Scell.DL_ChBandwidth, v_Frequency_IE_Scell.UL_ChBandwidth); //@sic R5-165892 sic@ //v_CarrierFreq_Scell := cs_CarrierFreqEUTRA(v_Frequency_IE_Scell.UL_DL_Earfcn.dl_CarrierFreq, v_Frequency_IE_Scell.UL_DL_Earfcn.ul_CarrierFreq); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (p_PCellId); v_IsFDD := (v_EUTRA_FDD_TDD_Mode == FDD); //Set C-RNTI to be used in s-cell creation f_EUTRA_CellInfo_SetC_RNTI(p_SCellId, tsc_C_RNTI_Def); //Create and configure all cells f_EUTRA_CellConfig_Pcell_Def(p_PCellId); f_EUTRA_CellConfig_Scell(p_SCellId); //No DRB's created // ring UE to initial state f_EUTRA_Preamble(p_PCellId, STATE2_IDLEUPDATE); f_EUTRA_RbEst_Def(p_PCellId); //Get extended UE category v_UE_CategoryExtn := f_EUTRA_MobileInfo_GetUECapability_UECategoryExtn();// @sic R5s150734 sic@ // @sic R5s150852 sic@ f_EUTRA_SetCellPower(p_SCellId, -91);//Make S Cell on f_EUTRA_TestBody_Set(true); // enable reporting of HARQ errors f_EUTRA_SS_CommonL1MacIndCtrlConfig(p_PCellId, cas_HarqErrorIndication_REQ(p_PCellId)); //@siclog "Step 1-4 Void" siclog@ //@siclog "Step 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message containing an sCellToAddModList with SCell Cell 3 addition. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message within 25 (FDD)/28 (TDD) subframes ? //@sic r5-165892 sic@ v_Arfcn_SCell := f_EUTRA_CellInfo_GetEARFCN_DL_r9(p_SCellId); v_SCell_DL_Bandwidth := f_EUTRA_CellInfo_GetSCell_DL_ChBandwidth(p_SCellId); v_EUTRA_CellSysInfo_Type := f_EUTRA_CellInfo_GetSYSINFO(p_SCellId); v_RadioResourceConfigCommonSIB := v_EUTRA_CellSysInfo_Type.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.radioResourceConfigCommon; v_FDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_SCellId); v_FDD_TDDInfo_PCell := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_PCellId); if ((v_FDD_TDDInfo.FDD_TDD == TDD) and not v_LAA_Scell) { //@sic R5-169501 R5-171732 sic@ v_TDD_Config := cs_TDD_ConfigDef(v_FDD_TDDInfo.TDD_SubframeAssignment); // Value omitted in FDD } //@sic R5-169501 R5s170064 sic@ if ( (v_FDD_TDDInfo_PCell.FDD_TDD == TDD) and (v_FDD_TDDInfo.FDD_TDD == FDD) ) { //TDD FDD v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_TDD_FDD_CA; } else if (v_FDD_TDDInfo.FDD_TDD == TDD) { // initialise tdd-Configuration to be used in v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_2TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX, cs_N1PUCCH_AN_CS_r10_FDD_2TX); } else { //FDD v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_1TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX); } //@sic R5s170064 R5s170602 sic@ /*f_EUTRA_SS_CommonCellConfig(p_PCellId, cas_CellConfig_PUCCH_Dedicated(p_PCellId, cs_TimingInfo_Now, v_FDD_TDDInfo_PCell.pucch_Config, v_PUCCH_ConfigDedicated)); */ if (v_UL_CA) { //@sic R5-160646 sic@ v_Scell_Capability := UL_DL; v_CommonSCell_UL_Configuration := cs_RadioResourceConfigCommonSCell_UL_Configuration_DEFAULT(v_ChBandDependency_SCell.Pusch_HoppingOffset, v_RadioResourceConfigCommonSIB.soundingRS_UL_ConfigCommon); v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT(); } v_SCellToAddMod := cs_SCellToAddMod_r10(1, v_PhysicalCellIdentity_Scell, v_Arfcn_SCell, cs_508_RadioResourceConfigCommonSCell_r10_DEFAULT(v_SCell_DL_Bandwidth, tsc_P_b_1Tx, v_CommonSCell_UL_Configuration, v_TDD_Config), cs_RadioResourceConfigDedicatedSCell_r10(cs_PhysicalConfigDedicatedSCell_r10_DEFAULT(v_DedicatedSCell_UL_Configuration,-, v_LAA_SCellConfiguration))); //@sic R5-1171366 sic@ v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_SCell_AddModRelease(tsc_RRC_TI_Def, v_SCellToAddMod, v_PUCCH_ConfigDedicated, -, infinity_); v_RachProcedureConfig := cs_RachProcedureConfig_CRNTI(f_EUTRA_CellInfo_GetRAR_TA(p_SCellId), f_EUTRA_CellInfo_GetDL_ChBandwidth(p_SCellId)); v_Timing := f_EUTRA_GetNextSendOccasion(p_PCellId, 500); // 500 ms later to send RRC message v_TimingInfo2 := f_EUTRA_TimingInfoAdd(v_Timing, 15); // 15 ms later for configuration f_EUTRA_Configure_Before_AddScell(p_PCellId, {p_SCellId}, v_RRCConnectionReconfiguration, v_Scell_Capability, v_RachProcedureConfig, -, -, //@sic R5s160035 sic@ cs_DciInfo_CcchDcchDtchUL_Def, cs_UL_GrantConfig_OnSR, cs_TimingInfo_EUTRA_NB(v_TimingInfo2)); v_N := 20; f_EUTRA_RRC_Procedure_Latency(p_PCellId, cas_SRB1_RrcPdu_REQ(p_PCellId, cs_TimingInfo_EUTRA_NB(v_Timing), v_RRCConnectionReconfiguration), -, car_SRB1_RrcPdu_IND(p_PCellId, cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def)), -, v_IsFDD, v_N, "Test Case 8.2.2.3.x Step 6", v_UE_CategoryExtn);// @sic R5s150734 sic@ //Start Default grant for rest of test case //@sic R5s150680 sic@ f_EUTRA_StartDefULGrantTransmission(p_PCellId); v_Arfcn_SCell := f_EUTRA_CellInfo_GetEARFCN_DL_r9(p_SCellId); v_PhysicalCellIdentity_Scell := f_EUTRA_CellInfo_GetPhyCellId(p_SCellId); v_SCell_DL_Bandwidth := f_EUTRA_CellInfo_GetSCell_DL_ChBandwidth(p_SCellId); v_FDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_SCellId); if (v_FDD_TDDInfo.FDD_TDD == TDD) { //@sic R5-169501 sic@ if ( not v_LAA_Scell) {v_TDD_Config := cs_TDD_ConfigDef(v_FDD_TDDInfo.TDD_SubframeAssignment);} // Value omitted in FDD and LAA v_FeatrGrp_113 := pc_FeatrGrp_113_T; //@sic R5-150557 sic@ } else { v_FeatrGrp_113 := pc_FeatrGrp_113_F; //@sic R5-150557 sic@ } //@sic R5-169501 sic@ if (v_FDD_TDDInfo.FDD_TDD != v_FDD_TDDInfo_PCell.FDD_TDD) { //FDD TDD v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_TDD_FDD_CA; } else if (v_FDD_TDDInfo.FDD_TDD == TDD) { // initialise tdd-Configuration to be used in v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_2TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX, cs_N1PUCCH_AN_CS_r10_FDD_2TX); //@sic R5-133275 sic@ } else { //FDD v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_1TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX); } if (v_UL_CA) { //@sic R5-160646 sic@ if (v_FDD_TDDInfo.FDD_TDD == TDD) { if (v_FeatrGrp_113) { //@sic R5-150557 sic@ v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT(-,//@sic R5s150070 sic@ cs_AntennaInfoUL_r10, cds_SoundingRsUl_ConfigDedicated_8223_TDD, cs_SoundingRS_UL_ConfigDedicated_v1020_8223); } else { v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT(-,//@sic R5s150070 sic@ cs_AntennaInfoUL_r10); } } else { if (v_FeatrGrp_113) { //@sic R5-150557 sic@ v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT(-,//@sic R5s150070 sic@ cs_AntennaInfoUL_r10, cs_SoundingRsUl_ConfigDedicated_8223_FDD, cs_SoundingRS_UL_ConfigDedicated_v1020_8223); } else { v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT(-,//@sic R5s150070 sic@ cs_AntennaInfoUL_r10); } } } v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_SCell_AddModRelease(tsc_RRC_TI_Def, cs_SCellToMod_r10(1, cs_RadioResourceConfigDedicatedSCell_r10(cs_PhysicalConfigDedicatedSCell_r10_DEFAULT(v_DedicatedSCell_UL_Configuration,-,v_LAA_SCellConfiguration))), //@sic R5-171732 sic@ v_PUCCH_ConfigDedicated, -, -, -, cs_MeasConfig(omit,omit,omit,omit,omit,omit,omit,cs_MeasGapConfig_release)//release GAP @sic R5s140010 R5s140205 sic@ ); if (v_UL_CA) { if (v_FeatrGrp_113) {//@sic R5-150557 sic@ f_EUTRA_SS_CommonCellConfig(p_SCellId, cas_CellConfig_ScellModSRS_AntennaInfo_REQ(p_SCellId, cs_TimingInfo_Now, f_EUTRA_CellInfo_GetSounding_ConfigCommon(p_SCellId))); } else { f_EUTRA_SS_CommonCellConfig(p_SCellId, cas_CellConfig_ScellModSRS_AntennaInfo_REQ(p_SCellId, cs_TimingInfo_Now, f_EUTRA_CellInfo_GetSounding_ConfigCommon(p_SCellId))); } } //@siclog "Step 7 - 8" siclog@ //Transmit RRCConnectionReconfiguration //Receive RRCConnectionReconfigurationComplete f_EUTRA_508RRC_AddMod_1Scell(p_PCellId, p_SCellId, 1, // SCellIndex_r10 // p_CA_Type v_UL_CA , // UL CA desired, -, // C_RNTI default := tsc_C_RNTI_Def, cs_MeasGapConfig_release, //release GAP @sic R5s140010 sic@ v_RRCConnectionReconfiguration); // RRCConnectionReconfigurationComplete received on Cell 3 f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.2.3 Step 8"); //@siclog "Step 9-10 Void" siclog@ //@sic R5-153722 sic@ v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_SCell_Release(tsc_RRC_TI_Def, {1}); //Scell Rel v_Timing := f_EUTRA_GetNextSendOccasion(p_PCellId, 200); // 200 ms later to send RRC message v_TimingInfo2 := f_EUTRA_TimingInfoAdd(v_Timing, 5); // 5 ms later for configuration // Pre configure Pcell as a normal serving cell f_EUTRA_ConfigPcellAsNormalServingCell (p_PCellId, cs_TimingInfo_EUTRA_NB(v_TimingInfo2), v_FDD_TDDInfo_PCell.pucch_Config, tsc_NoCnfReq); // Pre configure Scell as a normal non serving cell f_EUTRA_SS_CommonCellConfig(p_SCellId, cas_ServingCellRelease_REQ(p_SCellId, cs_TimingInfo_EUTRA_NB(v_TimingInfo2),tsc_NoCnfReq)); //@siclog "Step 11 - 12" siclog@ //Transmit RRCConnectionReconfiguration //Receive RRCConnectionReconfigurationComplete v_N := 20; f_EUTRA_RRC_Procedure_Latency(p_PCellId, cas_SRB1_RrcPdu_REQ(p_PCellId, cs_TimingInfo_EUTRA_NB(v_Timing), v_RRCConnectionReconfiguration), -, car_SRB1_RrcPdu_IND(p_PCellId, cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def)), -, v_IsFDD, v_N, "Test Case 8.2.2.3.x Step 12", v_UE_CategoryExtn);// // Release DRX and Measurement config on Scell //Configure for default grant f_EUTRA_ConfigScellAsNormalCell(p_SCellId,tsc_NoCnfReq); //@sic R5s150852 sic@ f_EUTRA_StartDefULGrantTransmission(p_PCellId); //@siclog "Step 13" siclog@ //Check if UE is in state RRC_Connected f_EUTRA_508Check_ConnectedState(p_PCellId); /////////////////////// // Postamble /////////////////////// f_EUTRA_TestBody_Set(false); //Switch/power off UE f_EUTRA_Postamble(p_PCellId, E2_CONNECTED); }