3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240624

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3557 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT test case 9.3.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The IMS component has not been reset to allow for new IMS registration on EUTRA side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reseted IMS at EUTRA before the attach procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | fl_TC_9_3_1_3_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | The IMS component has not been reset to allow for new IMS registration on EUTRA side. |
| Summary of change | Reseted IMS at EUTRA before the attach procedure. |
| TTCN module | Mobility_Registration_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted. Also removed the reset command from the NR test body as this is no longer needed. |

Before Change:

function fl_TC_9_3_1_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; f_EUTRA_MobileInfo_SetNetworkFeatureSupport (cs_EPS_NwkFtSup_NR); f_EUTRA_MobileInfo_SetAssignedPdnType(PDN_1, f_GetPdnType()); // @siclog "Step 5" siclog@ f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Test Body"); f_EUTRA38_SetCellPower(eutra_CellA, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE_FR2); //@sic R5-241629 sic@ v_NAS_Ind := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA, tsc_RRC_TI_Def, cr_EstablishmentCause_Any, cr_NAS_IndicationWithPiggybacking(tsc_SHT_NoSecurityProtection, cdr_ATTACH_REQUEST_ImsiNoOldGUTI(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)), cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, f_GetPdnType()))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @sic R5s194877 do extra checks now on Attach Req message sic@ if (not ispresent (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present for ENDC"); } if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for NR"); } f_EUTRA38_MultiPDN_InitialRegistration_Step5_18b1 (eutra_CellA, TESTMode_OFF, v_NAS_Ind); // @sic R5-204399 sic@ }

After Change:

function fl_TC_9_3_1_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; f_EUTRA_MobileInfo_SetNetworkFeatureSupport (cs_EPS_NwkFtSup_NR); f_EUTRA_MobileInfo_SetAssignedPdnType(PDN_1, f_GetPdnType()); // @siclog "Step 5" siclog@ f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Test Body"); f_EUTRA38_SetCellPower(eutra_CellA, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE_FR2); //@sic R5-241629 sic@ f_EUTRA_ResetIMS(PDN_1); v_NAS_Ind := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA, tsc_RRC_TI_Def, cr_EstablishmentCause_Any, cr_NAS_IndicationWithPiggybacking(tsc_SHT_NoSecurityProtection, cdr_ATTACH_REQUEST_ImsiNoOldGUTI(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)), cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, f_GetPdnType()))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @sic R5s194877 do extra checks now on Attach Req message sic@ if (not ispresent (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present for ENDC"); } if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for NR"); } f_EUTRA38_MultiPDN_InitialRegistration_Step5_18b1 (eutra_CellA, TESTMode_OFF, v_NAS_Ind); // @sic R5-204399 sic@ }