3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240625

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3558 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT test case 6.2.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | IMS component is getting reset before Step 20 which may cause failure in the IMS registration on the EUTRA side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the IMS reset after receiving the co-ordination message from EUTRA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

- Corrections required.

  - Change 1

| Function name | function fl_TC_6_2_1_5_TestBody |
| --- | --- |
| Reason for change | IMS component is getting reset before Step 20 which may cause failure in the IMS registration on the EUTRA side. |
| Summary of change | Moved the IMS reset after receiving the co-ordination message from EUTRA. |
| TTCN module | Idle_PLMNSelection_NR5GC_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_6_2_1_5_TestBody() runs on NR5GC_PTC { <<< Skipped >>> // @siclog "Step 19" siclog@ // The SS adjusts cell levels according to row T1 of table 6.2.1.5.3.2-1. if (not f_NR_CellInfo_GetIsFR1(nr_Cell1)) { // SS change NR Cell 1 SIB1 with modification of Qrxlevmin. v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell1, -91.0, v_DeltaPrimaryBand.DeltaNRf1); f_NR_ModifySysinfo(nr_Cell1, false); //Modify Sysinfo in SS without Paging } f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_ResetIMS(PDN_1);// @sic R5s230147 sic@ // @siclog "Step 20" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 after tmin but before tmax expires? alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { // verdict fail RRCSetupRequest received on NR Cell 1 within tmin f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.2.1.5 Step 20"); } [] t_Tmin.timeout { alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_Tmax.stop; // verdict pass RRCSetupRequest received on NR Cell 1 after t_Tmin time but before tmax f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.2.1.5 Step 20"); } [] t_Tmax.timeout { // verdict fail No RRCSetupRequest received f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.2.1.5 Step 20"); } } } } // @siclog "Step 20A-20B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq );// @sic R5s210015 sic@ f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(EUTRA); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, f_NR_Convert_CoOrdTo5G_GUTI(v_CoOrd_Guti, omit))) { // @siclog "Step 21b1-21b8" siclog@ // IF 5GS registration type is set as Mobility Registration in step 20B, THEN Steps 4 to 11 of the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 are performed on NR Cell 1 <<< Skipped >>> }

After Change:

function fl_TC_6_2_1_5_TestBody() runs on NR5GC_PTC { <<< Skipped >>> // @siclog "Step 19" siclog@ // The SS adjusts cell levels according to row T1 of table 6.2.1.5.3.2-1. if (not f_NR_CellInfo_GetIsFR1(nr_Cell1)) { // SS change NR Cell 1 SIB1 with modification of Qrxlevmin. v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell1, -91.0, v_DeltaPrimaryBand.DeltaNRf1); f_NR_ModifySysinfo(nr_Cell1, false); //Modify Sysinfo in SS without Paging } f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 20" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 after tmin but before tmax expires? alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { // verdict fail RRCSetupRequest received on NR Cell 1 within tmin f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.2.1.5 Step 20"); } [] t_Tmin.timeout { alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_Tmax.stop; // verdict pass RRCSetupRequest received on NR Cell 1 after t_Tmin time but before tmax f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.2.1.5 Step 20"); } [] t_Tmax.timeout { // verdict fail No RRCSetupRequest received f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.2.1.5 Step 20"); } } } } // @siclog "Step 20A-20B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq );// @sic R5s210015 sic@ f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(EUTRA); f_NR5GC_ResetIMS(PDN_1); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, f_NR_Convert_CoOrdTo5G_GUTI(v_CoOrd_Guti, omit))) { // @siclog "Step 21b1-21b8" siclog@ // IF 5GS registration type is set as Mobility Registration in step 20B, THEN Steps 4 to 11 of the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 are performed on NR Cell 1 <<< Skipped >>> }