3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240627

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4860 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE 7.1.9.1.x testcases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the test procedure the test case configures sCellDeactivationTimer-r10 as rf32 and is tested at Step 13. The same value is also provided to the SS, although the SS mac is in transparent mode. In order to avoid Scell being deactived by the SS after timer expiry, it is proposed to provide a higher value to the SS than that is provided to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When providing sCellDeactivationTimer to local configuration it is proposed to use rf64. Modified the functions to add additional parameter p_updateDeactTimer and set the value to rf64 when set to true |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.9.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required to test case 7.1.9.1.x	2

2.1	f_TC_7_1_9_1_Common	2

2.2	f_EUTRA_508RRC_AddMod_1Scell	4

2.3	f_EUTRA_508RRC_AddModRel_Scell_Common	13

2.4	f_EUTRA_ConfigPcell_ConfigScells	16

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 7.1.9.1.x

## f_TC_7_1_9_1_Common

| Function name | f_TC_7_1_9_1_Common |
| --- | --- |
| Reason for change | As part of the test procedure the test case configures sCellDeactivationTimer-r10 as rf32 and is tested at Step 13. The same value is also provided to the SS, although the SS mac is in transparent mode. In order to avoid Scell being deactived by the SS after timer expiry, it is proposed to provide a higher value to the SS than that is provided to the UE. |
| Summary of change | When providing sCellDeactivationTimer to local configuration it is proposed to use rf64. Modified the functions to add additional parameter p_updateDeactTimer and set to true |
| TTCN module | MAC_CA |
| MCC160 Comment | The default configuration is SS and UE are configured with same. Requires Prose CR to 36.523-3 or 38.523-1. Postponed till Prose CR approval. |

Before Change:

function f_TC_7_1_9_1_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_SCellId, CA_Tested_Type p_CA_Type) runs on EUTRA_PTC { … … … //@siclog "Step 1-2" siclog@ //Transmit RRCConnectionReconfiguration //Receive RRCConnectionReconfigurationComplete f_EUTRA_508RRC_AddMod_1Scell(p_PCellId, p_SCellId, 1, //SCellIndex_r10 //p_CA_Type v_UL_CA, // UL CA desired, -, // C_RNTI default := tsc_C_RNTI_Def, omit, // MeasGapConfig omit, no gap, v_RRCConnectionReconfiguration); // Reconfig message omit f_SS_Config1TM_DRB_NoMACHeader(p_PCellId, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); // to configure SS to report reception of SR

After Change:

| function f_TC_7_1_9_1_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_SCellId, CA_Tested_Type p_CA_Type) runs on EUTRA_PTC { … … … //@siclog "Step 1-2" siclog@ //Transmit RRCConnectionReconfiguration //Receive RRCConnectionReconfigurationComplete f_EUTRA_508RRC_AddMod_1Scell(p_PCellId, p_SCellId, 1, //SCellIndex_r10 //p_CA_Type v_UL_CA, // UL CA desired, -, // C_RNTI default := tsc_C_RNTI_Def, omit, // MeasGapConfig omit, no gap, v_RRCConnectionReconfiguration, -,-,-,-,-,-,-,-,-,-,-,-,-,-,-,true); // Reconfig message omit update scell deactivation timer f_SS_Config1TM_DRB_NoMACHeader(p_PCellId, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); // to configure SS to report reception of SR |  |
| --- | --- |

## f_EUTRA_508RRC_AddMod_1Scell

| Function name | f_EUTRA_508RRC_AddMod_1Scell |
| --- | --- |
| Reason for change | As part of the test procedure the test case configures sCellDeactivationTimer-r10 as rf32 and is tested at Step 13. The same value is also provided to the SS, although the SS mac is in transparent mode. In order to avoid Scell being deactived by the SS after timer expiry, it is proposed to provide a higher value to the SS than that is provided to the UE. |
| Summary of change | When providing sCellDeactivationTimer to local configuration it is proposed to use rf64. Modified the functions to add additional parameter p_updateDeactTimer and provided as an input to to f_EUTRA_508RRC_AddModRel_Scell_Common |
| TTCN module | MAC_CA |
| MCC160 Comment | The default configuration is SS and UE are configured with same. Requires Prose CR to 36.523-3 or 38.523-1. Postponed till Prose CR approval. |

Before Change:

function f_EUTRA_508RRC_AddMod_1Scell(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_SCellId, SCellIndex_r10 p_SCellIndex, //CA_Tested_Type p_CA_Type, boolean p_UL_CA := false, // UL CA desired C_RNTI p_C_RNTI := tsc_C_RNTI_Def, template (omit) MeasGapConfig p_MeasGapConfig := omit, //@sic R5s130503 sic@ template (omit) DL_DCCH_Message p_RRCConnectionReconfiguration := omit, template (omit) DciUlInfo_Type p_DciUlInfo_SCell := cs_DciInfo_CcchDcchDtchUL_Def, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SCell := cs_UL_GrantConfig_Stop, //@sic R5s160443 sic@ //@sic new CA-Enh test development sic@ template (omit) STAG_ToReleaseList_r11 p_STAG_ToReleaseList := omit, template (omit) STAG_ToAddModList_r11 p_STAG_ToAddModList := omit, template (omit) MAC_MainConfigSCell_r11 p_MAC_MainConfigSCell_r11 := omit, template (omit) RACH_ConfigCommonSCell_r11 p_RACH_ConfigCommonSCell_r11 := omit, template (omit) PRACH_Config p_PRACH_Config_SCell := omit, TimeAlignmentTimer p_TimeAlignmentTimer := sf750, MAC_MainConfig_ScellDeactivationTimer_Type p_ScellDeactivationTimer := rf32, //@sic R5s150285 sic@ template (omit) MBSFN_SubframeConfigList p_MBSFN_SubframeConfigList := omit, //@sic R5s160162 sic@ template (omit) PHR_Config_Type p_PHR_Config := cs_508_PHR_Config_Setup, //@sic R5s160322 sic@ boolean p_UL_FreqInfo := false, boolean p_SetUL_SRS := false, integer p_EIMTA_UL_DL_ConfigIndex_r12 := 1, boolean p_EIMTA := false ) runs on EUTRA_PTC { var CellCarrierFreqEUTRA_Type v_Arfcn_SCell := f_EUTRA_CellInfo_GetEARFCN(p_SCellId); //@sic R5-165892 sic@ var PhysCellId v_PhysicalCellIdentity_Scell := f_EUTRA_CellInfo_GetPhyCellId(p_SCellId); var SCell_DL_Bandwidth_Type v_SCell_DL_Bandwidth := f_EUTRA_CellInfo_GetSCell_DL_ChBandwidth(p_SCellId); var template (value) RachProcedureConfig_Type v_RachProcedureConfig; var template (value) DL_DCCH_Message v_RRCConnectionReconfiguration; var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_SCellId); var EUTRA_FDD_TDD_CellInfo_Type v_PCell_FDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_PCellId); //@sic R5-169501 sic@ var template (omit) TDD_Config v_TDD_Config := omit; // omitted in FDD var boolean v_UL_CA := p_UL_CA; var template (omit) CommonSCell_UL_Configuration_Type v_CommonSCell_UL_Configuration := omit; var template (omit) DedicatedSCell_UL_Configuration_Type v_DedicatedSCell_UL_Configuration := omit; var template (value) SCellToAddMod_r10 v_SCellToAddMod; var Ul_Bandwidth_Type v_UL_Bandwidth := f_EUTRA_CellInfo_GetUL_ChBandwidth(p_SCellId); var Dl_Bandwidth_Type v_DL_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(p_SCellId); var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(v_DL_Bandwidth, v_UL_Bandwidth); var EUTRA_CellSysInfo_Type v_EUTRA_CellSysInfo_Type := f_EUTRA_CellInfo_GetSYSINFO(p_SCellId); var RadioResourceConfigCommonSIB v_RadioResourceConfigCommonSIB := v_EUTRA_CellSysInfo_Type.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.radioResourceConfigCommon; var template (value) Scell_Capability_Type v_Scell_Capability := DlOnly; var template (value) PUCCH_ConfigDedicated_v1020 v_PUCCH_ConfigDedicated; var template (omit) SCell_UL_Bandwidth_Type v_SCell_UL_Bandwidth := omit; var template (value) SoundingRS_UL_ConfigDedicated v_SoundingRS_UL_ConfigDedicated := cs_508_SoundingRsUl_ConfigDedicated_Default_FDD; var template (value) SoundingRS_UL_ConfigDedicated_v1020 v_SoundingRS_UL_ConfigDedicated_v1020 := {srs_AntennaPort_r10 := an1}; var template (omit) LAA_SCellConfiguration_r13 v_LAA_SCellConfiguration := omit; var boolean v_LAA_Scell := f_EUTRA_CellInfo_IsLAA (p_SCellId); if (v_LAA_Scell) { v_LAA_SCellConfiguration := cs_508_LAA_SCellConfiguration; } if (v_FDD_TDDInfo.FDD_TDD == TDD) { // initialise tdd-Configuration to be used in // Value omitted in FDD and LAA v_SoundingRS_UL_ConfigDedicated := cds_508_SoundingRsUl_ConfigDedicated_Default_TDD; } else if (p_UL_FreqInfo) { //FDD and include Frequency info v_SCell_UL_Bandwidth := f_EUTRA_CellInfo_GetSCell_UL_ChBandwidth(p_SCellId); //Band extension for UL frequency @sic R5-171150 sic@ removed as per @sic R5-172065 sic@ } //@sic R5s170221 sic@ if (isvalue(p_RRCConnectionReconfiguration)) { v_RRCConnectionReconfiguration := valueof(p_RRCConnectionReconfiguration); } else { if ((v_FDD_TDDInfo.FDD_TDD == TDD) and not v_LAA_Scell) { //@sic R5-169501 R5-171732 sic@ v_TDD_Config := cs_TDD_ConfigDef(v_FDD_TDDInfo.TDD_SubframeAssignment); // Value omitted in FDD } //@sic R5-169501 R5s170064 sic@ if ( (v_PCell_FDD_TDDInfo.FDD_TDD == TDD) and (v_FDD_TDDInfo.FDD_TDD == FDD) ) { //TDD FDD v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_TDD_FDD_CA; } else if (v_FDD_TDDInfo.FDD_TDD == TDD) { // initialise tdd-Configuration to be used in v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_2TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX, cs_N1PUCCH_AN_CS_r10_FDD_2TX); //@sic R5-133275 sic@ } else { //FDD v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_1TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX); // Not addressed for 2TX } //@sic R5s170064 sic@ f_EUTRA_SS_CommonCellConfig(p_PCellId, cas_CellConfig_PUCCH_Dedicated(p_PCellId, cs_TimingInfo_Now, v_PCell_FDD_TDDInfo.pucch_Config, v_PUCCH_ConfigDedicated)); if (v_UL_CA) { //@sic R5-160646 sic@ v_CommonSCell_UL_Configuration := cs_RadioResourceConfigCommonSCell_UL_Configuration_DEFAULT(v_ChannelBandwidthDependency.Pusch_HoppingOffset, v_RadioResourceConfigCommonSIB.soundingRS_UL_ConfigCommon, omit, v_SCell_UL_Bandwidth); //@sic R5s160322 sic@ if (p_SetUL_SRS) { v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT(-, cs_AntennaInfoUL_r10, v_SoundingRS_UL_ConfigDedicated, v_SoundingRS_UL_ConfigDedicated_v1020); f_EUTRA_SS_CommonCellConfig(p_SCellId, cas_CellConfig_ScellModSRS_AntennaInfo_REQ(p_SCellId, cs_TimingInfo_Now, f_EUTRA_CellInfo_GetSounding_ConfigCommon(p_SCellId), v_SoundingRS_UL_ConfigDedicated, v_SoundingRS_UL_ConfigDedicated_v1020)); } else { v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT();//@sic R5s150070 sic@ } } //Band extension @sic R5-171150 sic@ if (p_EIMTA==false){ v_SCellToAddMod := cs_SCellToAddMod_r10(p_SCellIndex, v_PhysicalCellIdentity_Scell, v_Arfcn_SCell.dl_CarrierFreq_r9, cs_508_RadioResourceConfigCommonSCell_r10_DEFAULT(v_SCell_DL_Bandwidth, tsc_P_b_1Tx, v_CommonSCell_UL_Configuration, v_TDD_Config, -, p_RACH_ConfigCommonSCell_r11, p_PRACH_Config_SCell, p_MBSFN_SubframeConfigList, //@sic R5s150285 sic@ omit //Band extension @sic R5-171150 sic@ ), cs_RadioResourceConfigDedicatedSCell_r10(cs_PhysicalConfigDedicatedSCell_r10_DEFAULT(v_DedicatedSCell_UL_Configuration,-,v_LAA_SCellConfiguration), p_MAC_MainConfigSCell_r11 ));//@sic new CA-Enh test development sic@ } else{ v_SCellToAddMod := cs_SCellToAddMod_r10(p_SCellIndex, v_PhysicalCellIdentity_Scell, v_Arfcn_SCell.dl_CarrierFreq_r9, cs_508_RadioResourceConfigCommonSCell_r10_DEFAULT(v_SCell_DL_Bandwidth, tsc_P_b_1Tx, v_CommonSCell_UL_Configuration, v_TDD_Config, -, p_RACH_ConfigCommonSCell_r11, p_PRACH_Config_SCell, p_MBSFN_SubframeConfigList, omit), cs_RadioResourceConfigDedicatedSCell_r10(cds_508_PhysicalConfigDedicated_Default_SCell_AddMod_eIMTA(v_DedicatedSCell_UL_Configuration,-,v_LAA_SCellConfiguration,p_EIMTA_UL_DL_ConfigIndex_r12), p_MAC_MainConfigSCell_r11 )); } v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_SCell_AddModRelease(tsc_RRC_TI_Def, v_SCellToAddMod, v_PUCCH_ConfigDedicated,-,p_TimeAlignmentTimer, -, -, p_STAG_ToReleaseList, p_STAG_ToAddModList, p_ScellDeactivationTimer, -, -, -, p_PHR_Config);//@sic new CA-Enh test development sic@ } if (v_UL_CA) { v_Scell_Capability := UL_DL; } v_RachProcedureConfig := cs_RachProcedureConfig_CRNTI(f_EUTRA_CellInfo_GetRAR_TA(p_SCellId), f_EUTRA_CellInfo_GetDL_ChBandwidth(p_SCellId)); f_EUTRA_508RRC_AddModRel_Scell_Common(p_PCellId, {p_SCellId}, -, v_RRCConnectionReconfiguration, v_Scell_Capability, v_RachProcedureConfig, p_C_RNTI, p_MeasGapConfig, p_DciUlInfo_SCell, p_UL_GrantConfig_SCell); }

After Change:

| function f_EUTRA_508RRC_AddMod_1Scell(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_SCellId, SCellIndex_r10 p_SCellIndex, //CA_Tested_Type p_CA_Type, boolean p_UL_CA := false, // UL CA desired C_RNTI p_C_RNTI := tsc_C_RNTI_Def, template (omit) MeasGapConfig p_MeasGapConfig := omit, //@sic R5s130503 sic@ template (omit) DL_DCCH_Message p_RRCConnectionReconfiguration := omit, template (omit) DciUlInfo_Type p_DciUlInfo_SCell := cs_DciInfo_CcchDcchDtchUL_Def, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SCell := cs_UL_GrantConfig_Stop, //@sic R5s160443 sic@ //@sic new CA-Enh test development sic@ template (omit) STAG_ToReleaseList_r11 p_STAG_ToReleaseList := omit, template (omit) STAG_ToAddModList_r11 p_STAG_ToAddModList := omit, template (omit) MAC_MainConfigSCell_r11 p_MAC_MainConfigSCell_r11 := omit, template (omit) RACH_ConfigCommonSCell_r11 p_RACH_ConfigCommonSCell_r11 := omit, template (omit) PRACH_Config p_PRACH_Config_SCell := omit, TimeAlignmentTimer p_TimeAlignmentTimer := sf750, MAC_MainConfig_ScellDeactivationTimer_Type p_ScellDeactivationTimer := rf32, //@sic R5s150285 sic@ template (omit) MBSFN_SubframeConfigList p_MBSFN_SubframeConfigList := omit, //@sic R5s160162 sic@ template (omit) PHR_Config_Type p_PHR_Config := cs_508_PHR_Config_Setup, //@sic R5s160322 sic@ boolean p_UL_FreqInfo := false, boolean p_SetUL_SRS := false, integer p_EIMTA_UL_DL_ConfigIndex_r12 := 1, boolean p_EIMTA := false, boolean p_updateDeactTimer :=false) //update scell deactivation timer runs on EUTRA_PTC { var CellCarrierFreqEUTRA_Type v_Arfcn_SCell := f_EUTRA_CellInfo_GetEARFCN(p_SCellId); //@sic R5-165892 sic@ var PhysCellId v_PhysicalCellIdentity_Scell := f_EUTRA_CellInfo_GetPhyCellId(p_SCellId); var SCell_DL_Bandwidth_Type v_SCell_DL_Bandwidth := f_EUTRA_CellInfo_GetSCell_DL_ChBandwidth(p_SCellId); var template (value) RachProcedureConfig_Type v_RachProcedureConfig; var template (value) DL_DCCH_Message v_RRCConnectionReconfiguration; var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_SCellId); var EUTRA_FDD_TDD_CellInfo_Type v_PCell_FDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_PCellId); //@sic R5-169501 sic@ var template (omit) TDD_Config v_TDD_Config := omit; // omitted in FDD var boolean v_UL_CA := p_UL_CA; var template (omit) CommonSCell_UL_Configuration_Type v_CommonSCell_UL_Configuration := omit; var template (omit) DedicatedSCell_UL_Configuration_Type v_DedicatedSCell_UL_Configuration := omit; var template (value) SCellToAddMod_r10 v_SCellToAddMod; var Ul_Bandwidth_Type v_UL_Bandwidth := f_EUTRA_CellInfo_GetUL_ChBandwidth(p_SCellId); var Dl_Bandwidth_Type v_DL_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(p_SCellId); var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(v_DL_Bandwidth, v_UL_Bandwidth); var EUTRA_CellSysInfo_Type v_EUTRA_CellSysInfo_Type := f_EUTRA_CellInfo_GetSYSINFO(p_SCellId); var RadioResourceConfigCommonSIB v_RadioResourceConfigCommonSIB := v_EUTRA_CellSysInfo_Type.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.radioResourceConfigCommon; var template (value) Scell_Capability_Type v_Scell_Capability := DlOnly; var template (value) PUCCH_ConfigDedicated_v1020 v_PUCCH_ConfigDedicated; var template (omit) SCell_UL_Bandwidth_Type v_SCell_UL_Bandwidth := omit; var template (value) SoundingRS_UL_ConfigDedicated v_SoundingRS_UL_ConfigDedicated := cs_508_SoundingRsUl_ConfigDedicated_Default_FDD; var template (value) SoundingRS_UL_ConfigDedicated_v1020 v_SoundingRS_UL_ConfigDedicated_v1020 := {srs_AntennaPort_r10 := an1}; var template (omit) LAA_SCellConfiguration_r13 v_LAA_SCellConfiguration := omit; var boolean v_LAA_Scell := f_EUTRA_CellInfo_IsLAA (p_SCellId); if (v_LAA_Scell) { v_LAA_SCellConfiguration := cs_508_LAA_SCellConfiguration; } if (v_FDD_TDDInfo.FDD_TDD == TDD) { // initialise tdd-Configuration to be used in // Value omitted in FDD and LAA v_SoundingRS_UL_ConfigDedicated := cds_508_SoundingRsUl_ConfigDedicated_Default_TDD; } else if (p_UL_FreqInfo) { //FDD and include Frequency info v_SCell_UL_Bandwidth := f_EUTRA_CellInfo_GetSCell_UL_ChBandwidth(p_SCellId); //Band extension for UL frequency @sic R5-171150 sic@ removed as per @sic R5-172065 sic@ } //@sic R5s170221 sic@ if (isvalue(p_RRCConnectionReconfiguration)) { v_RRCConnectionReconfiguration := valueof(p_RRCConnectionReconfiguration); } else { if ((v_FDD_TDDInfo.FDD_TDD == TDD) and not v_LAA_Scell) { //@sic R5-169501 R5-171732 sic@ v_TDD_Config := cs_TDD_ConfigDef(v_FDD_TDDInfo.TDD_SubframeAssignment); // Value omitted in FDD } //@sic R5-169501 R5s170064 sic@ if ( (v_PCell_FDD_TDDInfo.FDD_TDD == TDD) and (v_FDD_TDDInfo.FDD_TDD == FDD) ) { //TDD FDD v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_TDD_FDD_CA; } else if (v_FDD_TDDInfo.FDD_TDD == TDD) { // initialise tdd-Configuration to be used in v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_2TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX, cs_N1PUCCH_AN_CS_r10_FDD_2TX); //@sic R5-133275 sic@ } else { //FDD v_PUCCH_ConfigDedicated := cs_508_PUCCH_ConfigDedicated_v1020_1TX(cs_N1PUCCH_AN_CS_r10_FDD_1TX); // Not addressed for 2TX } //@sic R5s170064 sic@ f_EUTRA_SS_CommonCellConfig(p_PCellId, cas_CellConfig_PUCCH_Dedicated(p_PCellId, cs_TimingInfo_Now, v_PCell_FDD_TDDInfo.pucch_Config, v_PUCCH_ConfigDedicated)); if (v_UL_CA) { //@sic R5-160646 sic@ v_CommonSCell_UL_Configuration := cs_RadioResourceConfigCommonSCell_UL_Configuration_DEFAULT(v_ChannelBandwidthDependency.Pusch_HoppingOffset, v_RadioResourceConfigCommonSIB.soundingRS_UL_ConfigCommon, omit, v_SCell_UL_Bandwidth); //@sic R5s160322 sic@ if (p_SetUL_SRS) { v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT(-, cs_AntennaInfoUL_r10, v_SoundingRS_UL_ConfigDedicated, v_SoundingRS_UL_ConfigDedicated_v1020); f_EUTRA_SS_CommonCellConfig(p_SCellId, cas_CellConfig_ScellModSRS_AntennaInfo_REQ(p_SCellId, cs_TimingInfo_Now, f_EUTRA_CellInfo_GetSounding_ConfigCommon(p_SCellId), v_SoundingRS_UL_ConfigDedicated, v_SoundingRS_UL_ConfigDedicated_v1020)); } else { v_DedicatedSCell_UL_Configuration := cs_RadioResourceConfigDedicatedSCell_UL_Configuration_DEFAULT();//@sic R5s150070 sic@ } } //Band extension @sic R5-171150 sic@ if (p_EIMTA==false){ v_SCellToAddMod := cs_SCellToAddMod_r10(p_SCellIndex, v_PhysicalCellIdentity_Scell, v_Arfcn_SCell.dl_CarrierFreq_r9, cs_508_RadioResourceConfigCommonSCell_r10_DEFAULT(v_SCell_DL_Bandwidth, tsc_P_b_1Tx, v_CommonSCell_UL_Configuration, v_TDD_Config, -, p_RACH_ConfigCommonSCell_r11, p_PRACH_Config_SCell, p_MBSFN_SubframeConfigList, //@sic R5s150285 sic@ omit //Band extension @sic R5-171150 sic@ ), cs_RadioResourceConfigDedicatedSCell_r10(cs_PhysicalConfigDedicatedSCell_r10_DEFAULT(v_DedicatedSCell_UL_Configuration,-,v_LAA_SCellConfiguration), p_MAC_MainConfigSCell_r11 ));//@sic new CA-Enh test development sic@ } else{ v_SCellToAddMod := cs_SCellToAddMod_r10(p_SCellIndex, v_PhysicalCellIdentity_Scell, v_Arfcn_SCell.dl_CarrierFreq_r9, cs_508_RadioResourceConfigCommonSCell_r10_DEFAULT(v_SCell_DL_Bandwidth, tsc_P_b_1Tx, v_CommonSCell_UL_Configuration, v_TDD_Config, -, p_RACH_ConfigCommonSCell_r11, p_PRACH_Config_SCell, p_MBSFN_SubframeConfigList, omit), cs_RadioResourceConfigDedicatedSCell_r10(cds_508_PhysicalConfigDedicated_Default_SCell_AddMod_eIMTA(v_DedicatedSCell_UL_Configuration,-,v_LAA_SCellConfiguration,p_EIMTA_UL_DL_ConfigIndex_r12), p_MAC_MainConfigSCell_r11 )); } v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_SCell_AddModRelease(tsc_RRC_TI_Def, v_SCellToAddMod, v_PUCCH_ConfigDedicated,-,p_TimeAlignmentTimer, -, -, p_STAG_ToReleaseList, p_STAG_ToAddModList, p_ScellDeactivationTimer, -, -, -, p_PHR_Config);//@sic new CA-Enh test development sic@ } if (v_UL_CA) { v_Scell_Capability := UL_DL; } v_RachProcedureConfig := cs_RachProcedureConfig_CRNTI(f_EUTRA_CellInfo_GetRAR_TA(p_SCellId), f_EUTRA_CellInfo_GetDL_ChBandwidth(p_SCellId)); f_EUTRA_508RRC_AddModRel_Scell_Common(p_PCellId, {p_SCellId}, -, v_RRCConnectionReconfiguration, v_Scell_Capability, v_RachProcedureConfig, p_C_RNTI, p_MeasGapConfig, p_DciUlInfo_SCell, p_UL_GrantConfig_SCell,-,p_updateDeactTimer); //update scell deactivation timer } |  |
| --- | --- |

## f_EUTRA_508RRC_AddModRel_Scell_Common

| Function name | f_EUTRA_508RRC_AddModRel_Scell_Common |
| --- | --- |
| Reason for change | As part of the test procedure the test case configures sCellDeactivationTimer-r10 as rf32 and is tested at Step 13. The same value is also provided to the SS, although the SS mac is in transparent mode. In order to avoid Scell being deactived by the SS after timer expiry, it is proposed to provide a higher value to the SS than that is provided to the UE. |
| Summary of change | When providing sCellDeactivationTimer to local configuration it is proposed to use rf64. Modified the functions to add additional parameter p_updateDeactTimer and provided as an input to f_EUTRA_ConfigPcell_ConfigScells |
| TTCN module | MAC_CA |
| MCC160 Comment | The default configuration is SS and UE are configured with same. Requires Prose CR to 36.523-3 or 38.523-1. Postponed till Prose CR approval. |

Before Change:

function f_EUTRA_508RRC_AddModRel_Scell_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellIdList_Type p_SCellIdAddList := tsc_EUTRA_NullCellIdList, EUTRA_CellIdList_Type p_SCellIdDelList := tsc_EUTRA_NullCellIdList, template (value) DL_DCCH_Message p_RRC_ConnRecongMsg, template (value) Scell_Capability_Type p_Scell_Capability := UL_DL, template (value) RachProcedureConfig_Type p_RachProcedureConfig, C_RNTI p_C_RNTI := tsc_C_RNTI_Def, template (omit) MeasGapConfig p_MeasGapConfig := omit, //@sic R5s130503 sic@ template (omit) DciUlInfo_Type p_DciUlInfo_SCell := cs_DciInfo_CcchDcchDtchUL_Def, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SCell := cs_UL_GrantConfig_Stop, boolean p_PcelltoNormalCell := false) //Configure Pcell to Normal cell if p_SCellIdAddList is empty runs on EUTRA_PTC { var integer i; var SubFrameTiming_Type v_TimingInfo; var SubFrameTiming_Type v_TimingInfo2; var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDDInfo_PCell := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_PCellId); var boolean v_CnfFlag := tsc_NoCnfReq; //@sic R5s140376 r5s160707 sic@ v_TimingInfo := f_EUTRA_GetNextSendOccasion(p_PCellId); v_TimingInfo2 := f_EUTRA_TimingInfoAdd(v_TimingInfo, 5); // 5 ms later f_EUTRA_ConfigPcell_AddScells(p_PCellId, p_SCellIdAddList, //p_RRC_ConnRecongMsg, p_RachProcedureConfig, p_C_RNTI, p_MeasGapConfig, cs_TimingInfo_EUTRA_NB(v_TimingInfo2)); // Step 6: Transmit RRCConnectionReconfiguration/HO command to UE SRB.send(cas_SRB1_RrcPdu_REQ(p_PCellId, cs_TimingInfo_EUTRA_NB(v_TimingInfo), p_RRC_ConnRecongMsg)); //@sic R5s150070 sic@ f_EUTRA_ConfigPcell_ConfigScells(p_PCellId, p_SCellIdAddList, p_RRC_ConnRecongMsg, p_Scell_Capability, p_DciUlInfo_SCell, p_UL_GrantConfig_SCell, cs_TimingInfo_EUTRA_NB(v_TimingInfo2)); if (lengthof(p_SCellIdDelList) > 0) { // one or more SCell Removal if (p_PcelltoNormalCell and (lengthof(p_SCellIdAddList)== 0)) { // Pcell is no more a Pcell //@sic R5-153722 sic@ f_EUTRA_ConfigPcellAsNormalServingCell (p_PCellId, cs_TimingInfo_EUTRA_NB(v_TimingInfo2), v_FDD_TDDInfo_PCell.pucch_Config, v_CnfFlag) } for (i:=0; i< lengthof(p_SCellIdDelList); i:=i+1) { if (fl_Check_CellID_InList(p_SCellIdDelList[i], p_SCellIdAddList)) { // SCell Being simultaneously released and Added, hence nothing to release continue; } // Step 9: Configure SCell as Scell f_EUTRA_SS_CommonCellConfig(p_SCellIdDelList[i], cas_ServingCellRelease_REQ(p_SCellIdDelList[i], cs_TimingInfo_EUTRA_NB(v_TimingInfo2), v_CnfFlag)); } } // Step 7: Receive RRCConnectionReconfiguration Complete SRB.receive(car_SRB1_RrcPdu_IND(p_PCellId, cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def))); // No RACH configuration as Already configured for C-RNTI based contention //No update of C-RNTI if (lengthof(p_SCellIdDelList) > 0) { for (i:=0; i< lengthof(p_SCellIdDelList); i:=i+1) { // Step 8: Release DRX and MeasGap at the S cell (if necessary) // Step 10: configure Default grant configuration on S cell f_EUTRA_ConfigScellAsNormalCell(p_SCellIdDelList[i],v_CnfFlag); } } }

After Change:

| function f_EUTRA_508RRC_AddModRel_Scell_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellIdList_Type p_SCellIdAddList := tsc_EUTRA_NullCellIdList, EUTRA_CellIdList_Type p_SCellIdDelList := tsc_EUTRA_NullCellIdList, template (value) DL_DCCH_Message p_RRC_ConnRecongMsg, template (value) Scell_Capability_Type p_Scell_Capability := UL_DL, template (value) RachProcedureConfig_Type p_RachProcedureConfig, C_RNTI p_C_RNTI := tsc_C_RNTI_Def, template (omit) MeasGapConfig p_MeasGapConfig := omit, //@sic R5s130503 sic@ template (omit) DciUlInfo_Type p_DciUlInfo_SCell := cs_DciInfo_CcchDcchDtchUL_Def, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SCell := cs_UL_GrantConfig_Stop, boolean p_PcelltoNormalCell := false, //Configure Pcell to Normal cell if p_SCellIdAddList is empty boolean p_updateDeactTimer :=false) //WA#WI=1314303 update scell deactivation timer runs on EUTRA_PTC { var integer i; var SubFrameTiming_Type v_TimingInfo; var SubFrameTiming_Type v_TimingInfo2; var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDDInfo_PCell := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_PCellId); var boolean v_CnfFlag := tsc_NoCnfReq; //@sic R5s140376 r5s160707 sic@ v_TimingInfo := f_EUTRA_GetNextSendOccasion(p_PCellId); v_TimingInfo2 := f_EUTRA_TimingInfoAdd(v_TimingInfo, 5); // 5 ms later f_EUTRA_ConfigPcell_AddScells(p_PCellId, p_SCellIdAddList, //p_RRC_ConnRecongMsg, p_RachProcedureConfig, p_C_RNTI, p_MeasGapConfig, cs_TimingInfo_EUTRA_NB(v_TimingInfo2)); // Step 6: Transmit RRCConnectionReconfiguration/HO command to UE SRB.send(cas_SRB1_RrcPdu_REQ(p_PCellId, cs_TimingInfo_EUTRA_NB(v_TimingInfo), p_RRC_ConnRecongMsg)); //@sic R5s150070 sic@ f_EUTRA_ConfigPcell_ConfigScells(p_PCellId, p_SCellIdAddList, p_RRC_ConnRecongMsg, p_Scell_Capability, p_DciUlInfo_SCell, p_UL_GrantConfig_SCell, cs_TimingInfo_EUTRA_NB(v_TimingInfo2), -,p_updateDeactTimer); //WA#WI=1314303 update scell deactivation timer if (lengthof(p_SCellIdDelList) > 0) { // one or more SCell Removal if (p_PcelltoNormalCell and (lengthof(p_SCellIdAddList)== 0)) { // Pcell is no more a Pcell //@sic R5-153722 sic@ f_EUTRA_ConfigPcellAsNormalServingCell (p_PCellId, cs_TimingInfo_EUTRA_NB(v_TimingInfo2), v_FDD_TDDInfo_PCell.pucch_Config, v_CnfFlag) } for (i:=0; i< lengthof(p_SCellIdDelList); i:=i+1) { if (fl_Check_CellID_InList(p_SCellIdDelList[i], p_SCellIdAddList)) { // SCell Being simultaneously released and Added, hence nothing to release continue; } // Step 9: Configure SCell as Scell f_EUTRA_SS_CommonCellConfig(p_SCellIdDelList[i], cas_ServingCellRelease_REQ(p_SCellIdDelList[i], cs_TimingInfo_EUTRA_NB(v_TimingInfo2), v_CnfFlag)); } } // Step 7: Receive RRCConnectionReconfiguration Complete SRB.receive(car_SRB1_RrcPdu_IND(p_PCellId, cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def))); // No RACH configuration as Already configured for C-RNTI based contention //No update of C-RNTI if (lengthof(p_SCellIdDelList) > 0) { for (i:=0; i< lengthof(p_SCellIdDelList); i:=i+1) { // Step 8: Release DRX and MeasGap at the S cell (if necessary) // Step 10: configure Default grant configuration on S cell f_EUTRA_ConfigScellAsNormalCell(p_SCellIdDelList[i],v_CnfFlag); } } } |  |
| --- | --- |

## f_EUTRA_ConfigPcell_ConfigScells

| Function name | f_EUTRA_ConfigPcell_ConfigScells |
| --- | --- |
| Reason for change | As part of the test procedure the test case configures sCellDeactivationTimer-r10 as rf32 and is tested at Step 13. The same value is also provided to the SS, although the SS mac is in transparent mode. In order to avoid Scell being deactived by the SS after timer expiry, it is proposed to provide a higher value to the SS than that is provided to the UE. |
| Summary of change | When providing sCellDeactivationTimer to local configuration it is proposed to use rf64. Modified the functions to add additional parameter p_updateDeactTimer and set the value to rf64 when set to true |
| TTCN module | MAC_CA |
| MCC160 Comment | The default configuration is SS and UE are configured with same. Requires Prose CR to 36.523-3 or 38.523-1. Postponed till Prose CR approval. |

Before Change:

function f_EUTRA_ConfigPcell_ConfigScells(EUTRA_CellId_Type p_PCellId, EUTRA_CellIdList_Type p_SCellIdAddList := tsc_EUTRA_NullCellIdList, template (value) DL_DCCH_Message p_RRC_ConnRecongMsg, template (value) Scell_Capability_Type p_Scell_Capability := UL_DL, template (omit) DciUlInfo_Type p_DciUlInfo_SCell := cs_DciInfo_CcchDcchDtchUL_Def, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SCell := cs_UL_GrantConfig_Stop, template (value) TimingInfo_Type p_TimingInfo, boolean p_CnfFlag := tsc_NoCnfReq) runs on EUTRA_PTC { var template (value) RRCConnectionReconfiguration_r8_IEs v_RRCConnectionReconfigurationIEs := p_RRC_ConnRecongMsg.message_.c1.rrcConnectionReconfiguration.criticalExtensions.c1.rrcConnectionReconfiguration_r8; var MAC_MainConfig_ScellDeactivationTimer_Type v_DeactTimer := rf32; var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDDInfo_PCell := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_PCellId); var template (omit) STAG_Id_r11 v_STAG_Id := omit; //@sic new CA-Enh test development sic@ var boolean v_NoUL_SCell; //@sic R5s140083 sic@ var SCellIndex_r10 v_SCellIndex; var integer i; //@sic R5w150205 sic@ var template (omit)Pcell_Mode_Type v_Pcell_Mode := omit; var EUTRA_FDD_TDD_CellInfo_Type v_SCellFDD_TDDInfo ; var EUTRA_FDD_TDD_CellInfo_Type v_PcellFDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_PCellId); for (i:=0; i < lengthof(p_SCellIdAddList); i:=i+1) { v_SCellFDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_SCellIdAddList[i]); if (v_PcellFDD_TDDInfo.FDD_TDD != v_SCellFDD_TDDInfo.FDD_TDD) //Scell mode is different than Pcell { if (v_PcellFDD_TDDInfo.FDD_TDD == FDD) //Pcell is FDD { v_Pcell_Mode := cs_Pcell_Mode_FDD } else //Pcell is TDD { v_Pcell_Mode := cs_Pcell_Mode_TDD (v_PcellFDD_TDDInfo.TDD_SubframeAssignment); } break; // One scell mode mismatch is sufficient.. }; }; if (isvalue(v_RRCConnectionReconfigurationIEs.radioResourceConfigDedicated.mac_MainConfig.explicitValue.mac_MainConfig_v1020.sCellDeactivationTimer_r10)) { v_DeactTimer := valueof(v_RRCConnectionReconfigurationIEs.radioResourceConfigDedicated.mac_MainConfig.explicitValue.mac_MainConfig_v1020.sCellDeactivationTimer_r10); } if (ispresent(v_RRCConnectionReconfigurationIEs.radioResourceConfigDedicated.physicalConfigDedicated.pucch_ConfigDedicated_v1020)) { f_EUTRA_SS_CommonCellConfig(p_PCellId, cas_CellConfig_PUCCH_Dedicated(p_PCellId, p_TimingInfo, v_FDD_TDDInfo_PCell.pucch_Config, v_RRCConnectionReconfigurationIEs.radioResourceConfigDedicated.physicalConfigDedicated.pucch_ConfigDedicated_v1020)); } f_EUTRA_SS_CommonCellConfig(p_PCellId, cas_CellConfig_CQI_ReportConfig_r10(p_PCellId, p_TimingInfo, cs_CQI_ReportConfig_r10_DEFAULT)); // one or more SCell Addition for (i:=0; i < lengthof(p_SCellIdAddList); i:=i+1) { //Executed 0 or more times v_NoUL_SCell := f_EUTRA_CellInfo_GetNoUL_Frequency(p_SCellIdAddList[i]); v_SCellIndex := valueof(v_RRCConnectionReconfigurationIEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.sCellToAddModList_r10[i].sCellIndex_r10); if (ispresent(v_RRCConnectionReconfigurationIEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.sCellToAddModList_r10[i].radioResourceConfigDedicatedSCell_r10.mac_MainConfigSCell_r11)) { v_STAG_Id := v_RRCConnectionReconfigurationIEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.sCellToAddModList_r10[i].radioResourceConfigDedicatedSCell_r10.mac_MainConfigSCell_r11.stag_Id_r11; } // Step 4: Configure SCell as Scell f_EUTRA_SS_CommonCellConfig(p_SCellIdAddList[i], cas_SCellConfig_REQ(p_SCellIdAddList[i], p_TimingInfo, cs_SCellConfig(p_PCellId, v_SCellIndex, p_Scell_Capability, v_DeactTimer, - , v_STAG_Id, v_Pcell_Mode), //@sic R5w150205 sic@ p_CnfFlag)); if (not v_NoUL_SCell) { //@sic R5s140083 sic@ f_EUTRA_SS_CommonCellConfig(p_SCellIdAddList[i], cas_CellConfig_CQI_ReportConfig_r10(p_SCellIdAddList[i], p_TimingInfo, cs_CQI_ReportConfig_r10_DEFAULT)); // Step 5: configure Stop UL grant configuration on S cell f_EUTRA_ULGrantConfiguration_Common(p_SCellIdAddList[i], p_TimingInfo, p_DciUlInfo_SCell, cs_UplinkTimeAlignment_Synch_None, p_UL_GrantConfig_SCell, p_CnfFlag); } } }

After Change:

| function f_EUTRA_ConfigPcell_ConfigScells(EUTRA_CellId_Type p_PCellId, EUTRA_CellIdList_Type p_SCellIdAddList := tsc_EUTRA_NullCellIdList, template (value) DL_DCCH_Message p_RRC_ConnRecongMsg, template (value) Scell_Capability_Type p_Scell_Capability := UL_DL, template (omit) DciUlInfo_Type p_DciUlInfo_SCell := cs_DciInfo_CcchDcchDtchUL_Def, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SCell := cs_UL_GrantConfig_Stop, template (value) TimingInfo_Type p_TimingInfo, boolean p_CnfFlag := tsc_NoCnfReq, boolean p_updateDeactTimer :=false) //WA#WI=1314303 update scell deactivation timer runs on EUTRA_PTC { var template (value) RRCConnectionReconfiguration_r8_IEs v_RRCConnectionReconfigurationIEs := p_RRC_ConnRecongMsg.message_.c1.rrcConnectionReconfiguration.criticalExtensions.c1.rrcConnectionReconfiguration_r8; var MAC_MainConfig_ScellDeactivationTimer_Type v_DeactTimer := rf32; var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDDInfo_PCell := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_PCellId); var template (omit) STAG_Id_r11 v_STAG_Id := omit; //@sic new CA-Enh test development sic@ var boolean v_NoUL_SCell; //@sic R5s140083 sic@ var SCellIndex_r10 v_SCellIndex; var integer i; //@sic R5w150205 sic@ var template (omit)Pcell_Mode_Type v_Pcell_Mode := omit; var EUTRA_FDD_TDD_CellInfo_Type v_SCellFDD_TDDInfo ; var EUTRA_FDD_TDD_CellInfo_Type v_PcellFDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info(p_PCellId); for (i:=0; i < lengthof(p_SCellIdAddList); i:=i+1) { v_SCellFDD_TDDInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_SCellIdAddList[i]); if (v_PcellFDD_TDDInfo.FDD_TDD != v_SCellFDD_TDDInfo.FDD_TDD) //Scell mode is different than Pcell { if (v_PcellFDD_TDDInfo.FDD_TDD == FDD) //Pcell is FDD { v_Pcell_Mode := cs_Pcell_Mode_FDD } else //Pcell is TDD { v_Pcell_Mode := cs_Pcell_Mode_TDD (v_PcellFDD_TDDInfo.TDD_SubframeAssignment); } break; // One scell mode mismatch is sufficient.. }; }; if (isvalue(v_RRCConnectionReconfigurationIEs.radioResourceConfigDedicated.mac_MainConfig.explicitValue.mac_MainConfig_v1020.sCellDeactivationTimer_r10)) { v_DeactTimer := valueof(v_RRCConnectionReconfigurationIEs.radioResourceConfigDedicated.mac_MainConfig.explicitValue.mac_MainConfig_v1020.sCellDeactivationTimer_r10); } if(p_updateDeactTimer) { v_DeactTimer :=rf64;} // update scell deactivation timer if (ispresent(v_RRCConnectionReconfigurationIEs.radioResourceConfigDedicated.physicalConfigDedicated.pucch_ConfigDedicated_v1020)) { f_EUTRA_SS_CommonCellConfig(p_PCellId, cas_CellConfig_PUCCH_Dedicated(p_PCellId, p_TimingInfo, v_FDD_TDDInfo_PCell.pucch_Config, v_RRCConnectionReconfigurationIEs.radioResourceConfigDedicated.physicalConfigDedicated.pucch_ConfigDedicated_v1020)); } f_EUTRA_SS_CommonCellConfig(p_PCellId, cas_CellConfig_CQI_ReportConfig_r10(p_PCellId, p_TimingInfo, cs_CQI_ReportConfig_r10_DEFAULT)); // one or more SCell Addition for (i:=0; i < lengthof(p_SCellIdAddList); i:=i+1) { //Executed 0 or more times v_NoUL_SCell := f_EUTRA_CellInfo_GetNoUL_Frequency(p_SCellIdAddList[i]); v_SCellIndex := valueof(v_RRCConnectionReconfigurationIEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.sCellToAddModList_r10[i].sCellIndex_r10); if (ispresent(v_RRCConnectionReconfigurationIEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.sCellToAddModList_r10[i].radioResourceConfigDedicatedSCell_r10.mac_MainConfigSCell_r11)) { v_STAG_Id := v_RRCConnectionReconfigurationIEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.sCellToAddModList_r10[i].radioResourceConfigDedicatedSCell_r10.mac_MainConfigSCell_r11.stag_Id_r11; } // Step 4: Configure SCell as Scell f_EUTRA_SS_CommonCellConfig(p_SCellIdAddList[i], cas_SCellConfig_REQ(p_SCellIdAddList[i], p_TimingInfo, cs_SCellConfig(p_PCellId, v_SCellIndex, p_Scell_Capability, v_DeactTimer, - , v_STAG_Id, v_Pcell_Mode), //@sic R5w150205 sic@ p_CnfFlag)); if (not v_NoUL_SCell) { //@sic R5s140083 sic@ f_EUTRA_SS_CommonCellConfig(p_SCellIdAddList[i], cas_CellConfig_CQI_ReportConfig_r10(p_SCellIdAddList[i], p_TimingInfo, cs_CQI_ReportConfig_r10_DEFAULT)); // Step 5: configure Stop UL grant configuration on S cell f_EUTRA_ULGrantConfiguration_Common(p_SCellIdAddList[i], p_TimingInfo, p_DciUlInfo_SCell, cs_UplinkTimeAlignment_Synch_None, p_UL_GrantConfig_SCell, p_CnfFlag); } } } |  |
| --- | --- |