3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240631

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3562 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC EPS Fallback test case 11.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case fails in multi-PDN scenario as the test control seems to be moving too quickly to f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech() before the default handler could be activated to handle additional PDN connections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a delay of 0.5sec to allow the activation of default handler for additional PDN connections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | fl_TC_11_1_2_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | The test case fails in multi-PDN scenario as the test control seems to be moving too quickly to f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech() before the default handler could be activated to handle additional PDN connections. |
| Summary of change | Added a delay of 0.5sec to allow the activation of default handler for additional PDN connections. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | The dedicated bearer can only be activated once the IMS PDN has been configured – which may not be the first PDN in a multi-PDN scenario. Alternative solution implemented: moved the configuration of additional PDNs before activation of the dedicated bearer. As this function is now being called immediately after activating the default behaviour, removed the if statement before it as there’s no time to receive anything. |

Before Change:

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var integer v_EPSBearerId; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var default v_MyDefaultVar := null; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5AA" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); v_PduSessionInfoList := f_TAUReject_AttachWithoutN26 (eutra_Cell1); // @sic R5s230015 sic@ // First clear store of previously received messages f_EUTRA38_MsgInDefault_ResetStoreOfPDNConnectivityReq(); // @sic R5-231581 sic@ // Then activate store for any new msgs received @sic R5-231581 sic@ v_MyDefaultVar := activate(a_EUTRA38_AdditionalPDNConnectivityReq(eutra_Cell1, cr_PDNConnReqAdditionalPDN(f_GetPdnType(), omit, '010'B))); // @siclog "Step 24A - 24C" siclog@ // @sic R5-223430 sic@ v_EPSBearerId := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s220448 sic@ // @siclog "Parallel Behaviour" siclog@ if (f_EUTRA38_MsgInDefault_GetNumOfPDNConnectivityReq() > 0) { // @sic R5-231581 sic@ f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated (eutra_Cell1, lengthof(v_PduSessionInfoList), // @sic R5-231581 sic@ false, tsc_SRB2, omit, '010'B, -, -, v_PduSessionInfoList); // @sic R5-216188, R5s230015 sic@ } /*****************Skipping code ***************************************************/ }

After Change:

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var integer v_EPSBearerId; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var default v_MyDefaultVar := null; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5AA" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); v_PduSessionInfoList := f_TAUReject_AttachWithoutN26 (eutra_Cell1); // @sic R5s230015 sic@ // First clear store of previously received messages f_EUTRA38_MsgInDefault_ResetStoreOfPDNConnectivityReq(); // @sic R5-231581 sic@ // Then activate store for any new msgs received @sic R5-231581 sic@ v_MyDefaultVar := activate(a_EUTRA38_AdditionalPDNConnectivityReq(eutra_Cell1, cr_PDNConnReqAdditionalPDN(f_GetPdnType(), omit, '010'B))); f_Delay(0.5);//KS@24xxxx // @siclog "Step 24A - 24C" siclog@ // @sic R5-223430 sic@ v_EPSBearerId := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s220448 sic@ // @siclog "Parallel Behaviour" siclog@ if (f_EUTRA38_MsgInDefault_GetNumOfPDNConnectivityReq() > 0) { // @sic R5-231581 sic@ f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated (eutra_Cell1, lengthof(v_PduSessionInfoList), // @sic R5-231581 sic@ false, tsc_SRB2, omit, '010'B, -, -, v_PduSessionInfoList); // @sic R5-216188, R5s230015 sic@ } /***************Skipping code*********************/

#### MCC160 Implementation

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@

{

var integer v_EPSBearerId;

var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList;

var default v_MyDefaultVar := null;

// Send EUTRA frequency to NR and wait for NR info

f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1}));

f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level");  // @sic R5-216190 sic@

// @siclog "Step 5AA" siclog@

f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE);

v_PduSessionInfoList := f_TAUReject_AttachWithoutN26 (eutra_Cell1); // @sic R5s230015 sic@

// First clear store of previously received messages

f_EUTRA38_MsgInDefault_ResetStoreOfPDNConnectivityReq(); // @sic R5-231581 sic@

// Then activate store for any new msgs received  @sic R5-231581 sic@

v_MyDefaultVar := activate(a_EUTRA38_AdditionalPDNConnectivityReq(eutra_Cell1, cr_PDNConnReqAdditionalPDN(f_GetPdnType(), omit, '010'B)));

// @siclog "Parallel Behaviour" siclog@

f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated (eutra_Cell1,

lengthof(v_PduSessionInfoList),  // @sic R5-231581 sic@

false,

tsc_SRB2,

omit,

'010'B,

-, -, v_PduSessionInfoList); // @sic R5-216188, R5s230015 sic@

// all expected msgs have been processed,  so deactivate the extra default

deactivate(v_MyDefaultVar);

// @siclog "Step 24A - 24C" siclog@

// @sic R5-223430 sic@

v_EPSBearerId := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s220448 sic@

// Test body finishes on IMS PTC

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(v_EPSBearerId, 1)); // @sic R5s220448 sic@

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble"));

}