3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240635

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4862 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_8_3_1_11_EUTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At step15a1, whenever the UE supports pc_Featrgrp_25, if the UE also supports intraFreq-CE-NeedForGaps, the value of measGapConfig should be MeasGapConfig-CE. Otherwise, the value of measGapConfig should be set to cs_508_MeasGapConfig_GP1. In the current implementation, only MeasGapConfig-CE is set. The value for measGapConfig should consider both LTE and Cat-M. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the measGapConfig value in step 15a1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_8_3_1_11_EUTRA() |
| --- | --- |
| Reason for change | At step15a1, whenever the UE supports pc_Featrgrp_25, if the UE also supports intraFreq-CE-NeedForGaps, the value of measGapConfig should be MeasGapConfig-CE. Otherwise, the value of measGapConfig should be set to cs_508_MeasGapConfig_GP1. In the current implementation, only MeasGapConfig-CE is set. The value for measGapConfig should consider both LTE and Cat-M. |
| Summary of change | Modify the measGapConfig value in step 15a1 |
| TTCN module | \LTE\8_3\RRC_Measurement.ttcn |
| MCC160 Comments | Accepted |

Before Change:

| function f_TC_8_3_1_11_EUTRA() runs on EUTRA_PTC { /* Measurement configuration control and reporting / intra E-UTRAN measurements: continuation of the measurements after RRC Connection Re-establishment */ var integer v_T0_RS_EPRE_Cell1 := -85; var integer v_T0_RS_EPRE_Cell2 := tsc_NonSuitableOffCellRS_EPRE; //@sic R5-103498 sic@ var integer v_T0_RS_EPRE_Cell3 := tsc_NonSuitableOffCellRS_EPRE; //@sic R5-103498 sic@ var integer v_T1_RS_EPRE_Cell3 := -73; var integer v_T2_RS_EPRE_Cell2 := -79; var integer v_T2_RS_EPRE_Cell3 := tsc_NonSuitableOffCellRS_EPRE; //@sic R5-103498 sic@ var integer v_T3_RS_EPRE_Cell1 := tsc_NonSuitableOffCellRS_EPRE; //@sic R5-103498 sic@ var integer v_T4_RS_EPRE_Cell1 := -79; var integer v_T4_RS_EPRE_Cell2 := -85; <<CODE SKIPPED>> //EXCEPTION: Steps 15a1 to 15a4 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. //@sic R5-150557 sic@ if (v_FeatrGrp_25) { //@sic R5-187127 sic@ //@siclog "Step 15a1-15a2" siclog@ //IF (pc_FeatrGrp_25) THEN the SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 2 and //receives an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 2 //@sic R5s110743 sic@ f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell2, cs_RRCConnectionReconfiguration_ResumeInterFreqMeas(v_RRC_TI, cs_508_MeasGapConfig_CE)); //@sic R5s190526 sic@ <<CODE SKIPPED>> |  |
| --- | --- |

After Change:

| function f_TC_8_3_1_11_EUTRA() runs on EUTRA_PTC { /* Measurement configuration control and reporting / intra E-UTRAN measurements: continuation of the measurements after RRC Connection Re-establishment */ var integer v_T0_RS_EPRE_Cell1 := -85; var integer v_T0_RS_EPRE_Cell2 := tsc_NonSuitableOffCellRS_EPRE; //@sic R5-103498 sic@ var integer v_T0_RS_EPRE_Cell3 := tsc_NonSuitableOffCellRS_EPRE; //@sic R5-103498 sic@ var integer v_T1_RS_EPRE_Cell3 := -73; var integer v_T2_RS_EPRE_Cell2 := -79; var integer v_T2_RS_EPRE_Cell3 := tsc_NonSuitableOffCellRS_EPRE; //@sic R5-103498 sic@ var integer v_T3_RS_EPRE_Cell1 := tsc_NonSuitableOffCellRS_EPRE; //@sic R5-103498 sic@ var integer v_T4_RS_EPRE_Cell1 := -79; var integer v_T4_RS_EPRE_Cell2 := -85; <<CODE SKIPPED>> //EXCEPTION: Steps 15a1 to 15a4 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. //@sic R5-150557 sic@ if (v_FeatrGrp_25) { //@sic R5-187127 sic@ //@siclog "Step 15a1-15a2" siclog@ //IF (pc_FeatrGrp_25) THEN the SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 2 and //receives an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 2 //@sic R5s110743 sic@ f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell2, cs_RRCConnectionReconfiguration_ResumeInterFreqMeas(v_RRC_TI, f_EUTRA_SS_MeasGapConfigIntraFreqOverrideCE())); //@sic R5s190526 sic@ <<CODE SKIPPED>> |  |
| --- | --- |