3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240637

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3565 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of function fl_TC_6_3_1_1_TestBody at Step 17 the template cr_NG_SOR_Header is used. This template expects the IE msSSNPNSILS to be present. However, this IE has been introduced for a Rel-18 device (24.501v18.8.0 cl 9.11.3.51.6). A REl-17 compliant device need not include this IE (24.501 v17.12.0 cl 9.11.3.51.6). The current implementation will fail a conformant Rel-17 UE at Step 17, and later at Step 20 – 21. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Declared a variable v_AccessStratumRelease, and checked the accessStratumReleaseIndicator before Step 17. Based on the accessStratumReleaseindicator, performed checks for the IEs of template cr_NG_SOR_Header. Also updated the implementation of function f_NR5GC_SorRegistration_Step5_20 for similar reasons. Please see below for detailed changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	8

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_6_3_1_1_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of function fl_TC_6_3_1_1_TestBody at Step 17 the template cr_NG_SOR_Header is used. This template expects the IE msSSNPNSILS to be present. However, this IE has been introduced for a Rel-18 device (24.501v18.8.0 cl 9.11.3.51.6). A REl-17 compliant device need not include this IE (24.501 v17.12.0 cl 9.11.3.51.6). The current implementation will fail a conformant Rel-17 UE at Step 17, and later at Step 20 – 21. This needs to be addressed. |
| Summary of change | Declared a variable v_AccessStratumRelease, and checked the accessStratumReleaseIndicator before Step 17. Based on the accessStratumReleaseindicator, performed checks for the IEs of template cr_NG_SOR_Header. Please see below |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment | Alternative solution implemented. The prose does not specify that this bit should be checked so set the non-checked bits to ? |

Before Change:

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202596 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202596 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202596 sic@ var O16_Type v_Mac_UE; var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O;//@sic R5s200663 sic@ var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := cs_NG_SOR_HeaderType0('1'B,'1'B,'1'B,'0'B); // Jun24 @sic R5s240531 Baseline Moving sic@ timer t_Tmax; timer t_Wait:= 10.0; //@siclog "Step 1" siclog@ //Power on the UE f_UT_SwitchOnUE(UT); //@siclog "Steps 2-14" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing //steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception v_Mac_UE:=f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,-,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));//@sic R5s210658 R5s211298 sic@ //@siclog "Step 16" siclog@ //SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); //@siclog "Step 17" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(cr_NG_SOR_Header('0'B,'0'B,?,'1'B),v_Mac_UE)))) {//@sic R5s230347 R5s230547 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } //EXCEPTION: Step 18a1 is performed in pc_noOf_PDUsSameConnection > 0.//@sic R5-211379 sic@ //@siclog "Step 18a1" siclog@//@sic R5-211379 sic@ // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (nr_Cell13,-,tsc_NR_RbId_SRB1);//@sic R5s210576 sic@ } <<SKIPPED CODE>>

After Change:

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202596 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202596 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202596 sic@ var O16_Type v_Mac_UE; var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O;//@sic R5s200663 sic@ var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var AccessStratumRelease v_AccessStratumRelease; //WA#WI1342492 WA#6_3_1_1 var template (value) SOR_Header v_Header := cs_NG_SOR_HeaderType0('1'B,'1'B,'1'B,'0'B); // Jun24 @sic R5s240531 Baseline Moving sic@ timer t_Tmax; timer t_Wait:= 10.0; //@siclog "Step 1" siclog@ //Power on the UE f_UT_SwitchOnUE(UT); //@siclog "Steps 2-14" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing //steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception v_Mac_UE:=f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,-,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));//@sic R5s210658 R5s211298 sic@ v_AccessStratumRelease:= f_NR_MobileInfo_NRCap_Get_AccessStratumRelease(); //WA#WI1342492 WA#6_3_1_1 //@siclog "Step 16" siclog@ //SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); //@siclog "Step 17" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; /*WA#WI1342492 WA#6_3_1_1 COMMENTED OUT if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(cr_NG_SOR_Header('0'B,'0'B,?,'1'B),v_Mac_UE)))) {//@sic R5s230347 R5s230547 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } */ if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } if ( not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.mac , v_Mac_UE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } if (not ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.header.dataType1.msSI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed, msSI not present in sorTransparentContainer"); } if (not (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.header.dataType1.msSNPNSI == '0'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed, msSNPNSI in sorTransparentContainer not matching"); } if (not (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.header.dataType1.dataType == '1'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed, dataType in sorTransparentContainer not matching"); } if (v_AccessStratumRelease == rel18) { if (not ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.header.dataType1.msSSNPNSILS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed, msSSNPNSILS in sorTransparentContainer missing"); } } //EXCEPTION: Step 18a1 is performed in pc_noOf_PDUsSameConnection > 0.//@sic R5-211379 sic@ //@siclog "Step 18a1" siclog@//@sic R5-211379 sic@ // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (nr_Cell13,-,tsc_NR_RbId_SRB1);//@sic R5s210576 sic@ } <<SKIPPED CODE>>

#### MCC160 Implementation

...

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0));

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(cr_NG_SOR_Header(?, ?,?,'1'B),v_Mac_UE)))) {//@sic R5s230347 R5s230547 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed");

}

//EXCEPTION: Step 18a1 is performed in pc_noOf_PDUsSameConnection > 0.//@sic R5-211379 sic@

//@siclog "Step 18a1" siclog@//@sic R5-211379 sic@

// The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2,

//takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection.

if (pc_noOf_PDUsSameConnection > 0 ){

f_NR5GC_PDUSessionEstablishment (nr_Cell13,-,tsc_NR_RbId_SRB1);//@sic R5s210576 sic@

}

//@siclog "Steps 18a2-18a5a1" void siclog@//@sic R5-211379 sic@

//@siclog "Step 18A" siclog@

//The SS transmits an RRCRelease message.

f_NR_RRCRelease(nr_Cell13);

//EXCEPTION: Step 18Ba1 is performed if pc_noOf_PDUsNewConnection > 0 and the UE starts PDU session establishment automatically

//@siclog "Step 18Ba1" siclog@//@sic R5-211379 sic@

//The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place from step 2 to 9a1 performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection.

if (pc_noOf_PDUsNewConnection > 0 ){

t_Wait.start;

alt {

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Wait.stop;

f_NR5GC_ExIdlePDUsession(nr_Cell13);

}

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Wait.stop;

}

[] t_Wait.timeout{}

}

}

//@siclog "Step 19" siclog@

//Check: Does the UE transmits an RRCSetupRequest on NR Cell 11 before tmax expires?

alt { //@sic R5s201466 sic@

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Tmax.stop;

f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 19 PASS");

}

[] t_Tmax.timeout

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 19 Failed");

}

}

//@siclog "Step 20-21" siclog@

//Steps 3 to 4 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 11.

//@sic R5-202596 sic@

v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell11);

if (v_TypeOfRegistration == Initial_Secure) {

// @siclog "Steps 22a1 - 22a16a1" siclog@//@sic R5-202596 sic@

// IF 5GS registration type is set as Initial Registration in step 21, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 11.

//@sic R5-204407 R5s201043 sic@

f_NR5GC_SorRegistration_Step5_20(nr_Cell11,

v_Data,

v_Header,

cr_NG_SOR_Header(?, ?, ?,,'1'B),      //@sic R5-206314, R5s240531 sic@

-,

Initial_Secure,

v_GMM_MobilityInfo,

v_ReceivedMsg);

}

else if(v_TypeOfRegistration == Mobility){

// @siclog "Steps 22b1 - 22b2a1" siclog@//@sic R5-202596 sic@

//ELSE IF 5GS registration type is set as Mobility Registration in step 21, THEN steps 5 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11.

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11,v_GMM_MobilityInfo);

}

## Change 2

| Function name | f_NR5GC_SorRegistration_Step5_20 |
| --- | --- |
| Reason for change | In the current TTCN implementation of function fl_TC_6_3_1_1_TestBody f_NR5GC_SorRegistration_Step5_20, at Step 21 the template cr_NG_SOR_Header is used. This template expects the IE msSSNPNSILS to be present. However, this IE has been introduced for a Rel-18 device (24.501v18.8.0 cl 9.11.3.51.6). A REl-17 compliant device need not include this IE (24.501 v17.12.0 cl 9.11.3.51.6). The current implementation will fail a conformant Rel-17 UE at 21. This needs to be addressed. |
| Summary of change | Declared a variable v_AccessStratumRelease, and checked the accessStratumReleaseIndicator before Step 17. Based on the accessStratumReleaseindicator, performed checks for the IEs of template cr_NG_SOR_Header. Please see below |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment | Not Needed. The prose does not specify that this bit should be checked. |

Before Change:

function f_NR5GC_SorRegistration_Step5_20(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, template (present) SOR_Header p_Header_UE := ?, boolean p_MacError:= false, TypeOfRegistration_Type p_RegType := Initial_NoSecurity, GMM_MobilityInfo_Type p_GMM_MobilityInfo, NG_NAS_MSG_Indication_Type p_ReceivedMsg, boolean p_AckSor:= true) runs on NR5GC_PTC { //@sic R5s230902 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var O16_Type v_Mac_UE; v_Mac_UE:=f_NR5GC_SorRegistration_Step5_13 (p_NR_CellId, p_GMM_MobilityInfo, p_ReceivedMsg, p_Data, p_Header, p_RegType, p_MacError); //@sic R5s210236 sic@ if(p_AckSor) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(p_Header_UE,v_Mac_UE)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } f_NR5GC_RRC_Idle_Steps16_20(p_NR_CellId,-, TESTMode_OFF,rrcConnectionRelease); } else{ f_NR5GC_RRC_Idle_Steps15_20(p_NR_CellId, TESTMode_OFF, rrcConnectionRelease); } }

After Change:

function f_NR5GC_SorRegistration_Step5_20(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, template (present) SOR_Header p_Header_UE := ?, boolean p_MacError:= false, TypeOfRegistration_Type p_RegType := Initial_NoSecurity, GMM_MobilityInfo_Type p_GMM_MobilityInfo, NG_NAS_MSG_Indication_Type p_ReceivedMsg, boolean p_AckSor:= true) runs on NR5GC_PTC { //@sic R5s230902 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var AccessStratumRelease v_AccessStratumRelease; //WA#WI1342492 WA#6_3_1_1 var O16_Type v_Mac_UE; v_Mac_UE:=f_NR5GC_SorRegistration_Step5_13 (p_NR_CellId, p_GMM_MobilityInfo, p_ReceivedMsg, p_Data, p_Header, p_RegType, p_MacError); //@sic R5s210236 sic@ if(p_AckSor) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; /*WA#WI1342492 WA#6_3_1_1 COMMENTED OUT if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(p_Header_UE,v_Mac_UE)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } */ //WA#WI1342492 WA#6_3_1_1 : Changes start v_AccessStratumRelease:= f_NR_MobileInfo_NRCap_Get_AccessStratumRelease(); if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } if ( not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.mac , v_Mac_UE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.header.dataType1.msSI, p_Header.dataType1.msSI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed, msSI in sorTransparentContainer not matching"); } if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.header.dataType1.msSNPNSI, p_Header.dataType1.msSNPNSI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed, msSNPNSI in sorTransparentContainer not matching"); } if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.header.dataType1.dataType, p_Header.dataType1.dataType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed, dataType in sorTransparentContainer not matching"); } if (v_AccessStratumRelease == rel18) { if (not ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.registration_Complete.sorTransparentContainer.header.dataType1.msSSNPNSILS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed, msSSNPNSILS in sorTransparentContainer missing"); } } //WA#WI1342492 WA#6_3_1_1 : Changes stop f_NR5GC_RRC_Idle_Steps16_20(p_NR_CellId,-, TESTMode_OFF,rrcConnectionRelease); } else{ f_NR5GC_RRC_Idle_Steps15_20(p_NR_CellId, TESTMode_OFF, rrcConnectionRelease); } }